| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Sounding Power Control in IEEE 802.16m AWD |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | KeunChul Hwang, Sungho Lee, Yongseok Kim, In-Seok Hwang, Soon-Young Yoon Samsung Electronics | keunchul.hwang@samsung.com |
| Re: | 802.16m Amendment Working Document (AWD) |  |
| Abstract | The contribution proposes the text of UL sounding power control to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Sounding Power Control in IEEE 802.16m AWD

KeunChul Hwang, Sungho Lee, Yongseok Kim, In-Seok Hwang, Soon-Young Yoon

Samsung Electronics

1. Introduction

In current AWD, there is no specific power control scheme for sounding channel. This contribution proposes a text on the sounding channel power control scheme.

2. Proposed Text

[Insert the following proposed text in blue at line 34, page 310 of 802.16m-09_0010r2 (section 15.3.9.4.4 Sounding Channel Power Control]

15.3.9.4.4 Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. AMS’s transmit power for UL sounding channel is controlled separately according to its sounding channel target CINR value. The power per subcarrier shall be maintained for the UL sounding transmission as shown in Equation (249):

(249)

where:

Ptx(dBm) is the transmit power per subcarrier and per transmit antenna power level.

PL is the estimated average pathloss.

CINRtarget is the target sounding CINR required at ABS.

NI_sounding is the estimated average power level (dBm) of the noise and interference per subcarrier at ABS.

OffsetAMSperAMS is the correction term for AMS-specific power offset controlled by AMS.

OffsetABSperAMS is the correction term for AMS-specific power offset controlled by ABS.

In Equation (249), CINRtarget is the sounding channel target CINR, which is set according to the DL CINR of AMS. In order to maintain the UL sounding quality, the different target CINR values are assigned according to DL CINR of each AMS; the AMS with high DL CINR applies relatively high target CINR and the AMS with low DL CINR applies relatively low target CINR. DL CINR threshold values that can determine AMS’ sounding Channel Class appear in Table [YYY] and the normalized target C/N values for different sounding Channel Classes appear in Table [XXX].

Relationship DL CINR threshold values and sounding Channel Class is as follows:

An AMS has sounding Channel Class # 0, if                      DL CINR <= DL CINR Threshold # 0

An AMS has sounding Channel Class # 1, if DL CINR Threshold # 0 < DL CINR <= DL CINR Threshold # 1,

An AMS has sounding Channel Class # 2, if DL CINR Threshold # 1 < DL CINR <= DL CINR Threshold # 2,

An AMS has sounding Channel Class # 3, if DL CINR Threshold # 2 < DL CINR

Table [YYY]. DL CINR Threshold for Different Sounding Channel Classes

| DL CINR Threshold | DL CINR [dB] |
| --- | --- |
| DL CINR Threshold # 0 | 0 |
| DL CINR Threshold # 1 | 3 |
| DL CINR Threshold # 2 | 6 |

Table [XXX]. Normalized target C/N for Different Sounding Channel Classes

| Sounding Channel Class | Required C/N [dB] |
| --- | --- |
| Channel Class # 0 | 0 |
| Channel Class # 1 | 3 |
| Channel Class # 2 | 5 |
| Channel Class # 3 | 8 |