| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AMS synchronization during Network Entry(AWD-15.2.15) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Youngkyo Baek Hyeonjeong Kang Yeongmoon Son JaeJeong Shim Rakesh Taori Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” Area : “Section 15.2.15 Network Entry and Initialization” |  |
| Abstract | This contribution provides some clarification on AMS synchronization during Network Entry |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AMS synchronization during Network Entry(AWD-15.2.15)

Youngkyo Baek,Hyeonjeong Kang,Yeongmoon Son,JaeJeong Shim,Rakesh Taori Samsung Electronics

# Introduction

Current AWD describes AMS PHY/MAC synchronization in subclause 15.2.15.1 and 15.2.15.2. However some contents of the above two subclauses are overlapped and we modify or rearrange some texts for clarification. Additionally this contribution gives some modification according to the following points.

- At line 60 page 86, In order to obtain total system information containing UL and DL parameters, AMS has to receive extended system information, which is a broadcast message (we name it as AAI_SCD message; refer to the contribution C802.16m-09/1491 or its latest version), as well as SFH.

- At line 61 page 86, network information means NSP list which is transferred by S-SFH SP2 or AAI_SII-ADV message.

- At line 5 page 87, UL synchronization is done after successful initial ranging procedure.

# Proposed Text

Modify the section 15.2.15.1 and 15.2.15.2 in the AWD as follows.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2.15.1 AMS synchronization.

On initialization or after signal loss, the AMS shall acquire a A-PREAMBLE according to cell selection rule.

Once the PHY has achieved synchronization the MAC shall attempt to acquire the channel control parameters for the DL and then the UL.

On initialization or after signal loss, the AMS shall acquire the DL PHY synchronization by A-PREAMBLE. The detailed procedure for PHY synchronization is reported in section 15.3.6.1.

15.2.15.2 AMS obtaining DL/UL parameters

To establish the DL synchronization the AMS shall perform the following operations:

• Scan the air interface and synchronize to the ABS

• Acquire network parameters and select the network

• Acquire SFH information

On initialization or after signal loss, the AMS shall acquire the DL synchronization. The AMS shall have nonvolatile storage in which the last operational parameters are stored and may first try to reacquire the stored DL channel. If the aforementioned process fails, the AMS shall begin to scan the possible channels of the DL frequency band of operation until it finds a valid DL signal.

The AMS shall synchronize at PHY level through the A-PREAMBLE. The detailed procedure for PHY synchronization is reported in section 15.3.6.1.

For initial network entry, o Once the AMS has achieved PHY synchronization with a particular ABS, the AMS shall attempt to decode essential system information (i.e., P-SFH and the S-SFH ) and extended system information (i.e., AAI_SCD message) in order to obtain the total necessary system information containing the DL and UL parameters for the initial network entry. Based on the network information such as NSP list which can be obtained from S-SFH SP2 or AAI_SII-ADV, the AMS shall decide whether to continue the network entry process with this ABS or to scan for other another ABSs.

After these steps the DL synchronization with the ABS is established.

If the AMS succeeds to decode the system information, the AMS is DL synchronized with the ABS.

The procedure to maintain the DL synchronization is TBD.

The AMS shall wait for the remaining S-SFH SPs from the ABS in order to retrieve a set of transmission parameters for a possible UL channel as indicated in Section 15.3.5.1.

After DL synchronization, the AMS shall attempt to perform initial ranging with the ABS. If the ranging procedure is successfully completed, the AMS is UL synchronized with the ABS.

--------------------------------------------------- End of Proposed Text------------------------------------------------------