| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on message formats for AAI_RNG-REQ/RSP, AAI_SBC-REQ/RSP and AAI_REG-REQ/RSP(AWD-15.2.5) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Youngkyo Baek Hyunjeong Kang Yeongmoon Son Jaehyuk Jang JaeJeong Shim Mihyun Lee Rakesh Taori Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” Area : “15.2.5 AAI MAC Management Messages” and “Section 15.2.15 Network Entry and Initialization” |  |
| Abstract | This contribution provides message format for AAI_RNG-REQ/RSP, AAI_SBC-REQ/RSP and AAI_REG-REQ/RSP which are used during initial network entry. |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on message formats for AAI_RNG-REQ/RSP, AAI_SBC-REQ/RSP and AAI_REG-REQ/RSP(AWD-15.2.5)

Youngkyo Baek,Hyunjeong Kang,Yeongmoon Son,Jaehyuk Jang,JaeJeong Shim, Mihyun Lee, Rakesh Taori

Samsung Electronics

# Introduction

During initial network entry AAI_RNG-REQ/RSP messages are used for initial ranging procedure. Pre-authentication capability negotiation is done by AAI_SBC-REQ/RSP messages, and registration and AMS capability negotiation is done by AAI_REG-REQ/RSP messages. This contribution provides message formats for AAI_RNG-REQ/RSP, AAI_SBC-REQ/RSP and AAI_REG-REQ/RSP.

# Proposed Text

Add following subsections to the section 15.2.5 ‘AAI MAC Management Messages’ in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2.5.xxx AAI_RNG-REQ

An AAI_ RNG-REQ message is transmitted by AMS at network entry. An AMS shall generate AAI_ RNG-REQ message in the format shown in Table xxx

Table xxx—AAI_ RNG-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ RNG-REQ _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| Encoded Information | Variable | Message elements |
| } | — | — |

All parameters are coded as message element:

Parameters shall only be included in AAI_RNG-REQ messages of adequate UL bandwidth. If required parameters cannot be accommodated in the UL bandwidth of a current AAI_RNG-REQ message, after AMS obtains a STID from the ABS, the AMS shall make UL BR of sufficient size to conduct additional AAI_RNG-REQ including all required parameters at the first available opportunity.

The following parameters are included according to their scope:

Table xxx.a—parameters for AAI_ RNG-REQ

| Name | Value | Scope |
| --- | --- | --- |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC version | Version number of IEEE 802.16 supported by the AMS |  |
| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO. If Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. Bit#5-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| Serving BSID | The BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). The serving BSID shall not be included if the aging timer is timed-out (serving BSID AGINGTIMER). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform HO reentry |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform unprepared HO reentry |
| Assigned STID | The STID which is assigned through HO-CMD message in the previous serving ABS | It shall be included when the AMS is attempting to perform prepared HO reentry |
| Paging Controller ID | The Paging Controller ID which the AMS currently maintains in idle mode. | It shall be included when the AMS is attempting to perform reentry or location update |
| Temporary ID | The temporary ID which the AMS is assigned for idle mode and currently maintains. |  |
| Paging Cycle Change | PAGING_CYCLE requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update |
| Power Down Indicator | Indicates the AMS is currently attempting to perform location update due to power down. | It shall be included when the AMS is attempting to perform location update due to power down |
| CMAC_KEY_COUNT | The AMS’s current value of the CMAC_KEY_COUNT, which is used to generate the security keys in the target ABS. | It shall be included during re-entry, secure Location Update or HO |
| CMAC Tuple | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Re-Entry from idle mode, Secure Location Update, or HO, if the AMS has a CMAC tuple necessary to expedite security authentication. |

15.2.4.xxy AAI_RNG-RSP

A AAI_RNG-RSP shall be transmitted by the ABS in response to a received AAI_RNG-REQ. In addition, it may also be transmitted asynchronously to send corrections based on measurements that have been made on other received data or MAC messages. As a result, the AMS shall be prepared to receive an AAI_RNG-RSP at any time, not just following an AAI_ RNG-REQ transmission.

The AAI_RNG-RSP message shall be encrypted and not contain CMAC Tuple, when the ABS notifies the AMS through the HO Process Optimization parameter that the AAI_PKM-REQ/RSP sequence may be omitted for the current HO reentry attempt, or when the ABS wishes to respond to the acknowledged AAI_RNG-REQ message containing a valid CMAC.

The format of the AAI_RNG-REQ message is shown in Table xxy.

Table xxy—AAI_ RNG-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ RNG-RSP _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| Ranging Status | 2 | Used to indicate whether UL messages are received within acceptable limits by ABS. 1 = continue, 2 = abort, 3 = success |
| Encoded Information | Variable | message elements |
| } | — | — |

The following parameters are included according to their scope:

Table xxy.a— parameters for AAI_ RNG-RSP

| Name | Value | Scope |
| --- | --- | --- |
| STID | The STID, which we call temporary STID, is used for AMS identification until STID is assigned to the AMS during registration procedure. | It shall be included in the AAI_RNG-RSP message in response to the AAI_RNG-REQ message when the AMS is not assigned its STID yet |
| MS_Random | A required parameter when the AMS confirms if the AAI_RNG-RSP is a response to the AAI_RNG-REQ message which the AMS sent. |  |
| Ranging code attributes | Indicates the OFDMA time symbols reference, subchannel reference, and frame reference(i.e., superframe number and position of frame in the superframe) used to transmit the ranging code, and the ranging code index that was sent by the AMS. | It shall be included in the AAI_RNG-RSP message in response to a received initial or periodic ranging code in order to distinguish AMSs which sent the ranging code |
| Timing Adjust Information | Tx timing offset adjustment | if needed |
| Power Adjust Information | Tx power offset adjustment |  |
| Frequency Adjust Information | Tx frequency offset adjustment |  |
| Location Update Response | 0x00= Success of Location Update 0x01= Failure of Location Update 0x02 = Reserved 0x03=Success of location update and DL traffic pending 0x04~0xFF: Reserved | It shall be included when an ABS sends an AAI_RNG-RSP message in response to an AAI_ RNG-REQ message used to perform location update |
| Paging Information | New Paging Information assigned to AMS | It shall be included only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed |
| Paging Controller ID | The new Paging Controller ID which the AMS shall maintain in idle mode. | It shall be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and Paging Controller ID has changed |
| Temporary ID | The new temporary ID which the AMS shall maintain in idle mode. |  |
| HO Process Optimization | Identifies reentry process management messages that may be omitted during the current HO attempt due to the availability of MS service and operational context information obtained by means that are beyond the scope of this standard, and the MS service and operational status post-HO completion. The AMS shall not enter normal operation with target ABS until completing receiving all network reentry, MAC management message responses as indicated in HO process optimization. | It shall be included when the AMS is attempting to perform network reentry or HO and the target ABS wishes to identify reentry process management messages that may be omitted during the current HO attempt |

15.2.4.yyx AAI_SBC-REQ

An AAI_SBC-REQ message is transmitted by AMS to negotiate pre-authentication capability during network entry.

The AAI_SBC-REQ message shall be encrypted and not contain CMAC Tuple during HO reentry if authentication has been completed:

An AMS shall generate AAI_ SBC-REQ message in the format shown in Table yyx

Table yyx—AAI_ SBC-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ SBC-REQ _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| Encoded Information | Variable | Message elements |
| } | — | — |

The following parameters may be included:

Table yyx.a— parameters for AAI_ SBC-REQ

| Name |  | Value |
| --- | --- | --- |
| Security negotiation parameters | Authorization policy support | Bit #0: EAP-based authorization is supported; default =1(supported) Bit#1-7 : reserved |
|  | PN Window Size | Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks; default =128 |
|  | Auth type for EAP | Auth Type for EAP shall only be included when EAP-based authorization is supported. If supported, either bit #0 or bit#1 is set to 1. Default (bit#0,bit#1)=(0,1) bit #0: device authentication bit #1: user authentication bit #2 -7: reserved |
| Physical parameters supported | Capabilities for DL HARQ support | TBD |
|  | Capabilities for UL HARQ support | TBD |
| current TX power |  | Current transmit power |

15.2.4.yyy AAI_SBC-RSP

An AAI_SBC-RSP message is transmitted by ABS in response to a received AAI_SBC-REQ during initialization.

The AAI_SBC-RSP message shall be encrypted and not contain CMAC Tuple during HO reentry if authentication has been completed.

An AMS shall generate AAI_ SBC-RSP message in the format shown in Table yyy

Table yyy—AAI_ SBC-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ SBC-REQ _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| Encoded Information | Variable | Message elements |
| } | — | — |

The following parameters may be included:

Table yyy.a— parameters for AAI_ SBC-RSP

| Name |  | Value |
| --- | --- | --- |
| Security negotiation parameters | Authorization policy and MAC encryption support | Bit #0: EAP-based authorization is supported; default =1(supported) Bit#1-7 : reserved |
|  | PN Window Size | Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks; default =128 |
| Physical parameters supported | Capabilities for DL HARQ support | TBD |
|  | Capabilities for UL HARQ support | TBD |

15.2.4.zzx AAI_REG-REQ

An AAI_REG-REQ message is transmitted by AMS to negotiate general AMS capabilities and do registration during network entry.

The AAI_REG-REQ message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

An AMS shall generate AAI_ REG-REQ messages in the format shown in Table zzx

Table zzx—AAI_ REG-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ REG-REQ _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| AMS MAC address | 48 | This AMS MAC address is transferred from AMS to ABS, which is used to derive security keys. |
| Encoded Information | Variable | Message elements |
| } | — | — |

The following parameters may be included:

Table zzx.a— parameters for AAI_ REG-REQ

| Name |  |  | Value |
| --- | --- | --- | --- |
| AMS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | the number of unique SN values; default =2^10 |
|  |  | ARQ_WINDOW_SIZE | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |
|  |  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | the maximum time interval an ARQ block shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEOUT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size |
|  | Co-located coexistence capability supported |  | Indicates if co-located coexistence is supported. |
|  | Capabilities for MS measurement |  | TBD |
|  | Capabilities for MIMO support |  | TBD |
| Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  | Indicates which classification/PHS options and SDU encapsulation the AMS supports |
|  | Maximum number of classification rules |  | Maximum number of admitted classification rules that the AMS supports. |
|  | ROHC support |  | Indicates the level of ROHC support. |
|  | PHS support |  | indicates the level of PHS support. |

15.2.4.zzy AAI_REG-RSP

An AAI_REG-RSP message is transmitted by ABS in response to AAI_ REG-REQ message during initialization.

The AAI_REG-RSP message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

An ABS shall generate AAI_ REG-RSP messages in the format shown in Table zzy

Table zzy—AAI_ REG-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ REG-REQ _Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| STID | 12 | This STID is used for AMS identification in place of the temporary STID which is transferred by AAI-RNG-RSP message. |
| Encoded Information | Variable | Message elements |
| } | — | — |

The following parameters may be included:

Table zzy.a— parameters for AAI_ REG-RSP message

| Name |  |  | Value |
| --- | --- | --- | --- |
| MS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | the number of unique SN values; default =2^10 |
|  |  | ARQ_WINDOW_SIZE | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |
|  |  | ARQ_SUB_BLOCK_SIZE | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME | the maximum time interval an ARQ block shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEOUT | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE | The maximum ARQ buffer size |
|  | Co-located coexistence capability supported |  | Indicates if co-located coexistence is supported. |
|  | Capabilities for MS measurement |  | TBD |
|  | Capabilities for MIMO support |  | TBD |
| Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  | indicates which classification/PHS options and SDU encapsulation the AMS supports |
|  | Maximum number of classification rules |  | Maximum number of admitted classification rules that the AMS supports. |
|  | ROHC support |  | Indicates the level of ROHC support. |
|  | PHS support |  | Indicates the level of PHS support. |

--------------------------------------------------- End of Proposed Text------------------------------------------------------