| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_SCN-REQ/RSP/REP message format |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0028r1: Call for Comments and Contributions on the AWD Section 15.2.5 AAI MAC Management Messages |  |
| Abstract | This contribution provides detailed message format of the AAI_SCN-REQ/RSP/REP |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_SCN-REQ/RSP/REP message format

Jaehyuk Jang, Jung Je Son, Rakesh Taori

Samsung Electronics

# Introduction

This contribution provides detailed message format of the AAI_SCN-REQ/RSP/REP.

# Proposed Text #1

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.4 AAI MAC Management Messages

15.2.4.XX AAI_SCN-REQ

An AAI_SCN-REQ message is transmitted by an AMS to request a scanning interval for the purpose of seeking available ABSs and/or YBSs and determining their suitability as targets for HO. An AMS may request a scanning interval during the scan interleaving interval.

The following parameters shall be included in the AAI_SCN-REQ message:

■ Scan duration

Duration (in units of subframes) of the requested scanning period.

■ Interleaving Interval

The period of AMS’s normal operation which is interleaved between Scan Durations.

■ Scan Iteration

The requested number of iterating scanning interval by an AMS.

■ Recommended_Start_Frame_indication

Indicates whether Recommended start super frame number field and Recommended start frame field are included.

■ N_Recommended_BS_Index_indication

Indicates whether N_Recommended_BS_Index field is included in this message

■ N_Recommended_BS_Full_indication

Indicates whether N_Recommended_BS_Full field is included in this message.

According to Indication values that MS indicates, the following parameters may be included in the AAI_SCN-REQ message:

■ Recommended start super frame number

Represents the 6 least significant bits of the absolute super frame number for which the AMS recommends the first Scanning Interval to start. This field is set to 0 if an AMS has no preferred value.

■ Recommended start frame number

Represents recommended start frame number within the super frame number which is indicated by Recommended start super frame number field. The value indicates:

0b00: The 1st frame in the super frame
0b01: The 2nd frame in the super frame
0b10: The 3rd frame in the super frame
0b11: The 4th frame in the super frame

■ N_Recommended_BS_Index

This is the number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message. When an AMS receives AAI_SCN-RSP message from BS in response to AAI_SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI_SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI_SCN-REQ message to the ABS.

■ Configuration Change Count for AAI_NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID

■ Neighbor_BS_Index

Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included.

■ N_Recommended_BS_Full

Number of neighboring BS to be scanned, which are using full 48-bit BSID.

■ Recommended BS ID

Based on the value of N_Recommended_BS_Full, identifiers of the BSs the MS plans to scan is included.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------

# Proposed Text #2

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.4 AAI MAC Management Messages

15.2.4.YY AAI_SCN-RSP

An AAI_SCN-RSP message shall be transmitted by the ABS either unsolicited or in response to an AAI_SCN-REQ message sent by an AMS. An ABS may transmit AAI_SCN-RSP to start AMS scan reporting with or without scanning allocation. If Scan Duration field contains a non-zero value, the system parameters which are carried through SFH shall not be changed during whole scanning duration specified in the AAI_SCN-RSP message. If Scan Duration field contains a non-zero value, scanning interval pattern which includes the AAI_SCN-RSP message replaces previous existing scanning interval pattern.

The following parameters shall be included in the AAI_SCN-RSP message:

■ Scan duration

Duration (in units of subframes) of the requested scanning period.

■ Report mode

0b00:No report
0b01:Periodic report
0b10:Event-triggered report
0b11:One-time scan report

■ Report metric

Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric:

Bit 0: BS CINR mean
Bit 1: BS RSSI mean
Bit 2: Relative delay
Bit 3: BS RTD
Bits 4–5: Reserved; shall be set to zero

■ Report period

The period of MS’s report of scanning result when the MS is required to report the value periodically or one-time. The period is calculated from the start of the first scan duration. If the BS sends an unsolicited AAI_SCN-RSP message without assignment of a scanning interval and the scan duration is set to zero, the period is calculated from the frame the MS receives the AAI_SCN-RSP message. For MS scanning request denied by AAI_SCN-RSP with Scan Duration set to zero, Report period is the number of frames that BS suggests to MS before transmitting next AAI_SCN-REQ

■ N_Recommended_BS_Index_indication

Indicates whether N_Recommended_BS_Index field is included in this message. This is valid only if Scan duration is not equal to zero.

■ N_Recommended_BS_Full_indication

Indicates whether N_Recommended_BS_Full field is included in this message. This is valid only if Scan duration is not equal to zero.

According to Indication values that BS indicates, the following parameters may be included in the AAI_SCN-RSP message:

■ Start super frame number

Represents the 6 least significant bits of the absolute frame number in which the first Scanning interval starts.

■ Start frame number

Represents start frame number in the Start super frame number.

0b00: The 1st frame in the super frame
0b01: The 2nd frame in the super frame
0b10: The 3rd frame in the super frame
0b11: The 4th frame in the super frame

■ Interleaving Interval

The period of AMS’s normal operation which is interleaved between Scan Durations.

■ Scan Iteration

The number of iterating scanning interval.

■ N_Recommended_BS_Index

Number of neighboring BS to be scanned, which are included in AAI_NBR-ADV message.

■ Configuration Change Count for AAI_NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID

■ Neighbor_BS_Index

Based on the value of N_Recommended_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included.

■ N_Recommended_BS_Full

Number of neighboring BS to be scanned, which are using full 48-bit BS ID.

■ Recommended BS ID

Based on the value of N_Recommended_BS_Full, Recommended BS ID list for scan is included

------------------------------------------------------ End of Proposed Text ------------------------------------------------------

# Proposed Text #3

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.4 AAI MAC Management Messages

15.2.4.ZZ AAI_SCN-REP

When the report mode is 0b10 (i.e., event triggered) in the most recently received AAI_SCN-RSP, the MS shall transmit a AAI_SCN-REP message to report the scanning results to its serving ABS after each scanning period if the trigger condition is met. For a periodic report (i.e., Report Mode is 0b01) and for One-time Scan Report (Report Mode is 0b11), the AMS reports the scanning results to its serving ABS at the time indicated in the AAI_SCN-RSP message except when it is in the scanning interval. For a periodic report (i.e., Report Mode is 0b01), the AMS stops reporting after all scanning intervals in the AAI-SCN-RSP message. The AMS shall include all available scanning results for the requested ABSs specified in the said AAI_SCN-RSP message. The MS may transmit an AAI_SCN-REP message to report the scanning results to its serving ABS at anytime.

The following parameters shall be included in the AAI_SCN-REP message:

■ Report mode

Action code for an AMS’s scan report of its measurement

0: Event-triggered report
1: Periodic report according to Scan report period of AAI_SCN-RSP

■ Report metric

Bitmap indicator of trigger metrics that the serving ABS requests the AMS to report. The serving ABS shall indicate only the trigger metrics agreed during AAI_SBC-REQ/RSP negotiation. For each bit location, a value of 0 indicates the trigger metric is not included, while a value of '1' indicates the trigger metric is included in the message. The bitmap interpretation for the metrics shall be:

Bit 0: BS CINR mean
Bit 1: BS RSSI mean
Bit 2: Relative delay
Bit 3: BS RTD
Bits 4–5: Reserved; shall be set to zero

■ N_Neighbor_BS_Index_indication

Indicates whether N_Neighbor_BS_Index field is included in this message

■ N_Neighbor_BS_Full_indication

Indicates whether N_Neighbor_BS_Full field is included in this message.

According to Report metric that MS indicates, the AAI_SCN-REP message includes the following parameters:

■ BS CINR mean

The BS CINR Mean parameter indicates the CINR measured by the AMS from the serving ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS’s segment and averaged over the measurement period.

■ BS RSSI mean

The BS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the serving ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as –103.75 dBm. An MS shall be able to report values in the range –103.75 dBm to –40 dBm. The measurement shall be performed on the frame preamble and averaged over the measurement period.

■ Relative delay

This parameter indicates the delay of neighbor DL signals relative to the serving BS, as measured by the MS for the particular BS. The value shall be interpreted as a signed integer in units of samples.

■ BS RTD

The BS RTD parameter indicates the round trip delay (RTD) measured by the AMS from the serving ABS. RTD can be given by the latest time advance taken by MS. The value shall be interpreted as an unsigned byte with units of 1/Fs.

According to Indication values that MS indicates, the following parameters may be included in the AAI_SCN-RSP message:

■ N_Neighbor_BS_Index

Number of neighboring BS reported in this message and which are included in AAI_NBR-ADV message.

■ Configuration Change Count for AAI_NBR-ADV

The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID

■ Neighbor_BS_Index

Based on the value of N_Neighbor_BS_Index, BS index corresponds to position of BS in AAI_NBR-ADV message is included.

■ N_Neighbor_BS_Full

Number of neighboring BS reported in this message that are using full 48 bits BS ID.

■ Neighbor_BSID

Based on the value of N_Neighbor_BS_Full, full BSID (48-bit) is included.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------