| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD Comments on Handover Related MAC Management Messages (AWD-15.2.5) |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Comment on the 802.16m amendment working document |  |
| Abstract | AWD text proposal on handover related MAC management messages |  |
| Purpose | To be adopted by TGm for the 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Comments on Handover Related MAC Management Messages (AWD-15.2.5)

Jaehyuk Jang, Jung Je Son, Rakesh Taori

Samsung Electronics

Introduction

This contribution proposes AWD text to complete the handover related MAC management messages.

Proposed AWD Text Changes

On the 16m AWD 80216m-09_0010r2, page 18, replace current text in 15.2.5.1, 15.2.5.2 and 15.2.5.3 with the proposed text as follows:
--------------------------------------------------  Text Start  -----------------------------------------------------------------

15.2.5 AAI MAC Management Messages

15.2.5.1 AAI HO-IND

The AMS may send the AAI_HO-IND MAC management message in a variety of scenarios, including HO preparation, HO execution and HO cancellation. To distinguish between the different scenarios, the AAI_HO-IND message shall contain an event code which takes the following values:

The specific format of the AAI_HO-IND message is TBD

• Code 1: Target ABS selection for multiple candidate T-ABS case.

• Code 2: All target ABSs in AAI_HO-CMD are unreachable. In this case, the AMS shall include a new target ABS that was not included in AAI_HO-CMD.

• Code 3: AMS unable to stay connected to serving ABS until expiration of disconnect time

• Code 4: HO cancel.

The AMS may send the AAI_HO-IND MAC management message in HO preparation, HO execution and HO cancellation. The AAI_HO-IND message includes the following information:

- HO Event Code: This is used to distinguish AAI_HO-IND among different scenarios:

  - 0b00: Target ABS selection in case of multiple candidate T-ABSs.

  - 0b01: All target ABSs in AAI_HO-CMD are unreachable. In this case, the AMS shall include a new target ABS that was not included in AAI_HO-CMD.

  - 0b10: AMS unable to stay connected to serving ABS until expiration of disconnect time

  - 0b11: HO cancel.

For HO Event Code = 0b00, the following parameters shall be included:

- Target ABS ID

15.2.5.2 AAI_HO-REQ

In MS-initiated HO, the AMS shall send the AAI_HO-REQ to S-ABS to initiate the HO procedure. The specific format of the AAI_HO-REQ message is TBD. The following parameters shall be included in the AAI_HO-REQ message:

- HO type:

    - Bit #0: BBE HO

    - Bit #1: EBB HO

    - Bit #2: Legacy HO

    - Bit #3: Zone switch

- Report metric: Bitmap indicator of trigger metrics that the MS reports in this message. The bitmap interpretation for the metrics shall be:

    - Bit #0: BS CINR mean

    - Bit #1: BS RSSI mean

    - Bit #2: Relative delay

    - Bits #3-6: Reserved; shall be set to zero.

- N_New_BS_Index: Number of neighboring ABSs to be considered for HO, which are included in AAI_NBR-ADV message.

- For each recommended neighbor BS, the following parameters shall be included:

    - Neighbor_ABS_ID

  - According to Report metric that MS indicates, the AAI_HO-REQ message includes the following parameters:

    - ABS CINR mean

    - ABS RSSI mean

    - Relative delay

15.2.5.3 AAI_HO-CMD

The S-ABS shall send AAI_HO-CMD to initiate the HO procedure, or to acknowledge the AAI_HO-REQ sent by the AMS. The AAI_HO-CMD message serves one of the following purposes.

• HO command

• HO rejection in response of AAI_HO-REQ from AMS

In case of HO command function, AAI_HO-CMD message should include one or more target ABS. For each target ABS, the following parameters are included:

1)Target ABS preamble index

2)Optional T-ABS MAC identity if preamble index has ambiguity (placeholder for dense overlay deployment or femtocell deployment)

3)HO_Reentry_Mode

4)Action Time

5)Disconnect Time (if HO_Reentry_Mode=1, disconnect time > action time; if HO_Reentry_Mode=0, Disconnect Time <= Action Time)

6)Optional EBB configuration if HO_Reentry_Mode = 1

i)HO_Reentry_interval

7)Resource_Retain_Time

8)CDMA_RNG_FLAG

9)Optional dedicated CDMA ranging code/opportunity if CDMA_RNG_FLAG=1

10)HO optimization related context, e.g., Indication of which components of the AMS' static and dynamic context are available at the target ABS.

11)Ranging initiation deadline.

12)Service level prediction, which indicates the level of service the MS can expect from this BS. The following encodings apply:

i)0 = No service possible for this MS

ii)1 = Some service is available for one or several service flows authorized for the MS or No service level prediction available.

iii)2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet.

iv)3 = No service level prediction available.

In case of HO rejection function, AAI_HO-CMD message shall not include any target ABS.

The S-ABS shall send AAI_HO-CMD to initiate the HO procedure, or to acknowledge the AAI_HO-REQ sent by the AMS. The AAI_HO-CMD message shall include the following parameters:

- Mode:

  - 0b00: HO command

  - 0b01: Zone switch command from MZone to LZone

  - 0b10: Perform legacy HO to a YBS

  - 0b11: AMS HO request rejected (ABS in list unavailable).

In case of Mode = 0b00, AAI_HO-CMD message should include one or more target ABS and the following parameters:

- Disconnect time offset: Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by subtracting this value from the value of Action time specified for this T-ABS.

- Resource_Retain_Time

- HO_Reentry_Mode: If set to 1, the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS; otherwise the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS. The following parameters shall be included when HO_Reentry_Mode is set to 1:

  - HO_Reentry_Interleaving Interval: If HO_Reentry_Interleaving Interval is greater than zero, the AMS continues data transmission with the Serving ABS within the HO_Reentry_Interleaving Interval and performs network reentry to the target ABS in the remaining time. Otherwise if HO_Reentry_Interleaving Interval is set to 0, the AMS maintains communication with Serving ABS on one carrier while performing network reentry with Target ABS on another carrier.

    - For HO_Reentry_Interleaving_Interval >= 0, the following parameter is further included:

      - HO_Reentry Iteration: The requested number of iterating HO Reentry Intervals by an AMS.

For Mode = 0b00 and for each target ABS, the following parameters shall be included:

- Target ABS ID

- Action Time: . Action Time included in this message is measured in frames and is calculated from the beginning of the frame where this message was received.

- CDMA_RNG_FLAG: The following parameters may be included when CDMA_RNG_FLAG=1:

  - Dedicated CDMA ranging code/opportunity

- HO process optimization: For each Bit location, a value of 0 indicates the associated reentry management messages shall be required, a value of 1 indicates the reentry management message may be omitted.

  - Bit #0: Omit AAI_SBC-REQ/RSP management messages during reentry processing

  - Bit #1: Omit PKM Authentication phase except TEK phase during current reentry processing

  - Bit #2: Seamless HO

  - Bit #3: TBD.

- Ranging initiation deadline.

- Service level prediction, which indicates the level of service the AMS can expect from this ABS. The following encodings apply:

  - 0 = No service possible for this AMS

  - 1 = Some service is available for one or several service flows authorized for the AMS

  - 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet.

  - 3 = No service level prediction available.

For Mode = 0b00 for each target ABS which is multi-carrier ABS, the following parameters should be included:

- Physical_Carrier_Index: Physical carrier index of the Target ABS where AMS will perform network reentry procedure.

For Mode = 0b00 for an AMS with multi-carrier capability, the following parameters should be included:

- MCHO_Indication: If MCHO_Indication is 0, the AMS shall disconnect all its active secondary carriers and perform single-carrier handover procedure per 15.2.6.3; otherwise it performs multi-carrier handover procedure per 15.2.9.2.9.2.

In case of Mode = 0b01, the AAI_HO-CMD message is used as a zone switch instruction from MZone to LZone, and the following parameters shall be included:

- HO_Reentry_Mode: If set to 1, the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier; otherwise the AMS disconnects from the MZone before performing network reentry with the LZone.

- Action Time: This value is defined as the frame number that MS starts zone switch. Action Time included in this message is indicated by frame number.

- Resource_Retain_Time

- CDMA_RNG_FLAG: The following parameters may be included when CDMA_RNG_FLAG=1:

  - Dedicated CDMA ranging code/opportunity

- LZone information: LZone information for the AMS to switch from MZone to LZone.

In case of Mode = 0b11, AAI_HO-CMD message shall not include any target ABS.

15.2.5.X AAI_ZS-REQ (Zone Switch Request)

An AMS shall send the AAI_ZS-REQ message to initiate the zone switching procedure from MZone to LZone.

15.2.5.X AAI_ZS-CMD (Zone Switch Command)

An ABS shall send AAI_ZS-CMD to initiate the zone switching procedure from MZone to LZone, or to acknowledge the AAI_ZS-REQ sent by the AMS. The AAI_ZS-CMD message shall include the following parameters.

- HO_Reentry_Mode: If set to 1, the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier; otherwise the AMS disconnects from the MZone before performing network reentry with the LZone.

- Action Time: This value is defined as the frame number that MS starts zone switch. Action Time included in this message is indicated by frame number.

- Resource_Retain_Time

- CDMA_RNG_FLAG: The following parameters may be included when CDMA_RNG_FLAG=1:

  - Dedicated CDMA ranging code/opportunit

- LZone information: LZone information for the AMS to switch from MZone to LZone.

15.2.6.4.1.4 Zone switch from MZone to Lzone

The ABS indicates zone switch of AMSs that currently operate in the MZone for several reasons, such as load balancing purposes. The HO-Command AAI_ZS-CMD message is used to trigger the zone switch from MZone to LZone which is specified in TBD section 15.2.5.X. Zone switch from LZone to MZone follows the procedure per section 15.2.5.3.1.2. During the request or command of the zone switch to LZone, the AMS is provided with LZone information in prior, such as CID, FID, security parameters or capability information in the MZone via the AAI-HO-CMD AAI-ZS-CMD message.

------------------------------------------------------  Text End  ---------------------------------------------------------------