| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD Comments on Zone Swirching Related MAC Management Messages (AWD-15.2.5) |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Comment on the 802.16m amendment working document |  |
| Abstract | AWD text proposal on handover related MAC management messages |  |
| Purpose | To be adopted by TGm for the 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Comments on Zone Switching Related MAC Management Messages (AWD-15.2.5)

Jaehyuk Jang, Jung Je Son, Rakesh Taori

Samsung Electronics

Introduction

This contribution proposes AWD text to complete the handover related MAC management messages.

Proposed AWD Text Changes

On the 16m AWD 80216m-09_0010r2, page 18, replace current text in 15.2.5.1, 15.2.5.2 and 15.2.5.3 with the proposed text as follows:
--------------------------------------------------  Text Start  -----------------------------------------------------------------

15.2.5 AAI MAC Management Messages

15.2.5.X AAI_ZS-REQ (Zone Switch Request)

An AMS shall send the AAI_ZS-REQ message to initiate the zone switching procedure from MZone to LZone.

15.2.5.X AAI_ZS-CMD (Zone Switch Command)

An ABS shall send AAI_ZS-CMD to initiate the zone switching procedure from MZone to LZone, or to acknowledge the AAI_ZS-REQ sent by the AMS. The AAI_ZS-CMD message shall include the following parameters.

- HO_Reentry_Mode: If set to 1, the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier; otherwise the AMS disconnects from the MZone before performing network reentry with the LZone.

- Action Time: This value is defined as the frame number that MS starts zone switch. Action Time included in this message is indicated by frame number.

- Resource_Retain_Time

- CDMA_RNG_FLAG: The following parameters may be included when CDMA_RNG_FLAG=1:

  - Dedicated CDMA ranging code/opportunit

- LZone information: LZone information for the AMS to switch from MZone to LZone.

15.2.6.4.1.4 Zone switch from MZone to Lzone

The ABS indicates zone switch of AMSs that currently operate in the MZone for several reasons, such as load balancing purposes. The HO-Command AAI_ZS-CMD message is used to trigger the zone switch from MZone to LZone which is specified in TBD section 15.2.5.X. Zone switch from LZone to MZone follows the procedure per section 15.2.5.3.1.2. During the request or command of the zone switch to LZone, the AMS is provided with LZone information in prior, such as CID, FID, security parameters or capability information in the MZone via the AAI-HO-CMD AAI-ZS-CMD message.

------------------------------------------------------  Text End  ---------------------------------------------------------------