| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_ORAT-MSG message format |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori, Jungshin Park, Jicheol Lee Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0028r1: Call for Comments and Contributions on the AWD Section 15.2.5 AAI MAC Management Messages |  |
| Abstract | This contribution provides detailed message format of the AAI_ORAT-MSG |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_ORAT-MSG message format

Jaehyuk Jang, Jung Je Son, Rakesh Taori, Jungshin Park, Jicheol Lee

Samsung Electronics

# Introduction

This contribution provides detailed message format of the AAI_ORAT-MSG.

Detailed contents of AAI_ORAT-MSG will be defined in other group, not in 802.16 (e.g., 802.21, 3GPP, 3GPP or NWG specification). To carry other RAT’s message, we need to define the detailed format of the AAI_ORAT-MSG message.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.5 AAI MAC Management Messages

15.2.5.XX AAI_ORAT-MSG

An AAI_ORAT-MSG message is transmitted either by an AMS or an ABS to convey inter-RAT HO related messages (e.g., MIHF). This message is transmitted in a unicast or broadcast manner. An AMS which does not support inter-RAT HO feature shall ignore the AAI_ORAT-MSG message.

An AMS shall generate AAI_ORAT-MSG messages in the format shown in Table XXX.

Table XXX—AAI_ORAT-MSG message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ORAT-MSG_Message_format() { | — | — |
| Management Message Type = XX | 8 | — |
| TLV Encoded Information | Variable | TLV specific |
| } | — | — |

The following parameters may be included in the AAI_ORAT-MSG message; at least one TLV shall be included in an AAI_ORAT-MSG message:

MIHF frame

The encapsulated MIH response

RAT specific information TLV

RAT specific information TLV may include following parameters according to RAT Type.

• Pre-registration supported
• Network boundary indication
• RAN Information
• RAP Information

15.2.YY TLV encodings

15.2.YY.1 AAI_ORAT-MSG management message encodings

15.2.YY.1.1 MIHF frame

This TLV is used to carry an MIHF frame. MIHF frames are specified in IEEE Std 802.21.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 1 | variable | An MIHF frame. MIHF frames are specified in IEEE Std 802.21. | AAI-ORAT-MSG |

15.2.YY.1.2 RAT specific information TLV

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [10-63] | variable | RAT specific information (TBD) | AAI-ORAT-MSG |

Type field of RAT specific information TLV specifies air interface technology type.

| Type value | RAT Type |
| --- | --- |
| 10 | IEEE 802.11 |
| 11 | GERAN(GSM/GPRS/EGPRS) |
| 12 | UTRAN |
| 13 | E-UTRAN |
| 14 | CDMA 2000 |
| 15-31 | Reserved |

RAT specific information may include following parameters:

• Pre-registration supported: This field indicates whether pre-registration is supported or not for Inter-RAT HO.
• Network boundary indication: This field indicates the whether the ABS which is sending this message is located in the boundary area of the 802.16m network or not.
• RAN Information: The RAN information specifies information for different radio access networks with various RATs defined by different standard bodies
• RAP Information: The RAP information specifies information for different radio access points (such as carrier frequency, BSID, preamble).

------------------------------------------------------ End of Proposed Text ------------------------------------------------------