| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Usages of P-FBCH for bandwidth request |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics Jie Hui, Xiangying Yang, Muthaiah Venkatachalam Intel Corporation | E-mail: jack.jang@samsung.com E-mail: jie.hui@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0028r1: Call for Comments and Contributions on the AWD Section 15.2.11.1.4 Bandwidth Request using FFB |  |
| Abstract | This contribution provides the usages of P-FBCH for bandwidth request. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Usages of P-FBCH for bandwidth request

Jaehyuk Jang, Rakesh Taori, Jung Je Son

Samsung Electronics

Jie Hui, Xiangying Yang, Muthaiah Venkatachalam

Intel Corporation

# Introduction

The latest AWD does not include the detailed usages of primary feedback channel (P-FBCH). This contribution provides two usages of P-FBCH for bandwidth request.

We propose to use P-FBCH with two purposes: one is to send signaling header (including BR header), and the other is to resume existing ertPS service flow. Two purposes are already defined in the current 802.16e specification.

# Proposed Text

Add following text in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2.11.1.4 Bandwidth Request using FFB

15.2.11.1.4.1 Indication flag feedback

An AMS can send an indication flag on the P-FBCH. The indication flag is used by the AMS to indicate to the ABS its intention to transmit a feedback header or a BR header without the need to perform BR ranging. The codeword 0bXXXXXX (TBD) is used for that purpose. After receiving the indication flag from the AMS, the ABS may allocate the required UL resource to the AMS.

In order to maintain operation of link adaptation mechanisms at the ABS and adequate CINR reporting, the AMS shall not transmit the indication flag codeword on two consecutive P-FBCH allocations that are allocated to it.

15.2.11.1.4.2 ertPS/aGPS BR

If an AMS has ertPS connections, the AMS may inform the serving ABS of the existence of pending ertPS data. If an AMS has aGPS connections, the AMS may inform the serving ABS of the existence of pending aGPS data related to primary QoS parameters. The codeword 0bYYYYYY (TBD) is used for those purposes.

If the ABS receives the codeword (i.e. 0bYYYYYY (TBD)) on P-FBCH from the AMS, the ABS should make UL allocation as follows:

1. If an AMS has ertPS connections only, the ABS should allocate for the AMS an UL burst corresponding to the largest Maximum Sustained Traffic Rate of the AMS's stopped ertPS UL service flows.

2. If an AMS has aGPS connections only, the ABS should change allocation for the AMS UL bursts corresponding to the Primary service flow QoS parameter set of the AMS's aGPS UL service flow, which has the largest Grant_Size_primary among all aGPS service flows.

3. If an AMS has both ertPS and aGPS connections, the ABS should allocate for the AMS UL bursts corresponding to the largest Maximum Sustained Traffic Rate of the AMS's ertPS and aGPS UL service flows.

If an AMS has ertPS connections, this codeword also implicitly indicates the stopped ertPS UL service flow to resume.

In order to maintain operation of link adaptation mechanisms at the ABS and adequate CINR reporting, the AMS shall not transmit the ertPS/aGPS BR codeword on two consecutive P-FBCH allocations that are allocated to it.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------