| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for Sounding Antenna Selection |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Wookbong Lee, Jinyoung Chun, Bin-Chul Ihm LG Electronics. | wbong@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 DL control channel and 15.3.9 Uplink control channel “Comments on AWD 15.3.6 DL CTRL and 15.3.9 UL CTRL” |  |
| Abstract | This contribution proposes the antenna selection method for sounding. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for Sounding Antenna Selection

Wookbong Lee, Jinyoung Chun, and Bin-Chul Ihm

LG Electronics

- Introduction

In current AWD [1], Sounding command IE for multiple uplink transmit antennas are defined. However, there is some issues related to multiple uplink transmit antenna sounding.

First of all, it is not clear that where to transmit each transmit antennas’ sounding channel. Currently only one unique offset for FDM or one unique cyclic shift for CDM is indicated. However, if there are multiple antennas at MS, we need to indicate multiple sounding channels. So, we propose to assign N contiguous sounding channel index starting from the offset or cyclic shift value, when the number of transmit antennas for sounding of the MS is N.

Second of all, currently MS shall transmit same number of sounding channel as the actual transmit antenna at MS. However, due to the antenna power imbalance, sometimes it is better to turn off some of transmit antennas to reduce uplink power consumption.

Antenna power imbalance is caused by e.g. user hand-gripping or direction of polarized antenna and so on.

So, the power imbalance pattern among the transmit antennas does not change very shortly but slowly.

The following figure shows the impact of antenna power imbalance.

As you can see in the figure, when antenna power imbalance is higher than a certain threshold, single antenna transmission is better than two antenna transmission (SFBC).

So, we propose to adopt antenna selection in uplink transmission. Since uplink sounding is required for determine uplink MCS level or precoder, we further propose to use uplink sounding commend IE to indicate which antenna/antenna group to be selected for transmitting sounding signal or data.

To enable BS to get the other transmit antenna’s channel response, BS may allocate another sounding channel with longer period.

- Reference

[1] 80216m-09/0010r2, “IEEE 802.16m Amendment Working Document”

Remedy #1. Line 1 to 25, page 201. Modify UL basic assignment A-MAP IE as follows;

-------------------------------  Text Start  ---------------------------------------------------

- 15.3.6.5.2.3 UL basic assignment A-MAP IE

Table 670 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 670— UL basic assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| … |  |  |
| if (PMI Indicator == 0b1){ |  |  |
| if(NEFt Nt == 2){ |  |  |
| PMI | 3 | - Precoding Matrix Index for NEFt Nt = 2 |
| } else if(NEFt Nt == 4){ |  |  |
| PMI | 6 | - Precoding Matrix Index for NEFt Nt = 4 |
| } |  |  |
| } |  |  |
| … |  |  |

TNS: Total number of streams in the LRU for CSM

SI: First pilot index for CSM.

PF: Precoding flag to indicate adaptive or non-adsptive precoding.

PMI: Precoding matrix index .

NEFt: Number of effective transmit antenna derived from UL Sounding Command A-MAP IE. If there is no UL Sounding Command A-MAP IE, NEFt is set to number of transmit antenna (Nt).

Remedy #2. Line 50, page 228 to line 47, page 229. Modify SOUNDING Command A-MAP IE as follows;

-------------------------------  Text Start  ---------------------------------------------------

- 15.3.6.5.2.11 UL Sounding Command A-MAP IE

Table 684—UL Sounding Command A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| UL Sounding Command IE format () { |  |  |
| A-MAP IE Type | 4 |  |
| Sounding subframe | 3 | Indicates the sounding subframe [TBD] |
| Sounding subband bitmap | Variable [max. 12] | - [TBD] FFT size dependant TBD |
| If (Multiplexing type == 0) { |  |  |
| Decimation offset d (SCI) | [4] or [5] | Unique decimation offset |
| } else { |  |  |
| Cyclic time shift n (SCI) | [4] or [5] | Unique cyclic shift [TBD] |
| } |  |  |
| Periodicity (n) | 3 | 0b000 = Single command, not periodic, or terminate the periodicy. Otherwise, repeat sounding once per 2(n-1) frames, where n is decimal value of the periodicity field |
| Multi-antenna sounding | 1 | 0b0: sounding with antenna switching 0b1: sounding from multiple antennas |
| Multi-Antenna Index (MAI) | 2 | If the number of transmit antennas, Nt, is one, then this field shall be ignored, and the MS transmits sounding signals via sounding channel indicated by SCI. If the number of transmit antennas, Nt, is two, then this field indicates which transmit antennas to transmit sounding channel and the number of antennas in group (NAG) equals to 1. If i-th MSB bit of this field is ‘1’, then the MS transmits sounding signals of i-th transmit antennas. If i-th MSB bit of this field is ‘0’, then the MS does not transmit sounding singals of i-th transmit antennas. If the number of transmit antennas, Nt, is four, then this fields indicates which transmit antenna groups to transmit sounding channel and the number of antennas in group (NAG) equals to 2. If i-th MSB bit of this field is ‘1’, then the MS transmits sounding signals of i-th group of transmit antennas which are NAG·i and NAG·i +1. If i-th MSB bit of this field is ‘0’, then the MS does not transmit sounding singals of i-th group of transmit antennas. |
| Antenna Switching | 1 | 0b0: Antenna switching AMS transmits sounding signals via sounding channels indicated by SCI. 0b1: No antenna switching AMS transmits sounding signals via sounding channels indicated by SCI to SCI+ NSA-1. |
| Padding | [TBD] | Padding to reach byte boundary |
| MCRC | 16 | 16 bit CRC masked by Station ID |
| } |  |  |

SCI: Sounding channel index indicates unique decimation offset or cyclic time shift.

NAG: The number of antennas in group. If Nt equals to 1 or 2, then NAG is 1. If Nt equals to 4, then NAG is 2.

NSA: The number of sounding antenna equals to NAG multiplied by M (number of ‘1’s in Multi-Antenna Index)

Table 684 specifies the fields of UL Sounding Command A-MAP IE used by the ABS to request sounding transmission by the AMS. Decimal equivalent of the sounding subframe indicates the uplink subframe with sounding symbol. The sounding subband bitmap field is used to indicate the sounding subbands used in the sounding allocation. For that purpose, the Nused contiguous subcarriers are further subdivided into sounding subbands, where each sounding subband compromises N1 * Nsc adjacent subcarriers with Nsc = 18. For multiplexing type equal to 0 the first subcarrier index of kth transmit antenna within sounding allocation is determined from g = F(d, k, Frame Index), where function F() is TBD. The three periodicy bits are used to indicate the MS to periodically repeat the sounding transmission. Setting periodicy bits to 0b000 indicates a single sounding command or terminates the sounding if periodic sounding command is being performed. Multi-Antenna Index (MAI) indicates which antenna(s)/group of antenna(s) to be selected to sound. For multi-antenna Antenna Switching flag equals to 0 the MS sounds with antenna switching, while for multi-antenna Antenna Switching flag equals to 1 the MS sounds all transmit antennas.

The effective number of transmit antenna (NEFt) is set to NSA. When there are multiple UL Sounding Commend A-MAP IEs are transmitted to the AMS, NEFt is set to NSA of shorter periodic report. If multiple UL Sounding Commend A-MAP IEs’ Periodicity (n) are same, then NEFt is set to Nt of the AMS.

-------------------------------  Text End  ---------------------------------------------------

Remedy #3. Line 6-23, page 294. Modify texts as follows;

-------------------------------  Text Start  ---------------------------------------------------

15.3.9.2.3.2 Multiplexing for multi-antenna and multi-AMS

The uplink sounding channels of multiple AMS and multiple antennas per AMS can be multiplexed through decimation separation or cyclic shift separation in each sounding allocation. Also, in case of multiple UL subframes for sounding, time division separation can be applied by assigning different AMS to different UL subframe. For cyclic shift separation each AMS occupies all subcarriers within sounding allocation and uses the different sounding waveform. For frequency decimation separation each AMS uses decimated subcarrier subset from the sounding allocation set with different frequency offset.

For antenna selection capable AMS, ABS can command the AMS to transmit sounding waveform for selected physical transmit antenna(s).

For antenna switching capable AMS and multi-antenna AMS, ABS can command the AMS to switch the physical transmit antenna(s) for sounding transmission. For sounding with antenna switching, the AMS shall transmit sounding symbol with the i-th antenna (0, 1, …, NSANt-1) on frames nt = j • 2(n-1)T + i, where nt = 0 corresponds to the frame where UL sounding command A-MAP IE is received, Tn is periodicity in UL sounding command A-MAP IE, and j is a running index ( j = 0, 1, 2, … for Tn ≠ 0 and j = 0 for Tn = 0). For sounding with antenna switching and periodical sounding allocation (Tn ≠ 0), the assigned periodicity T2(n-1) shall be larger or equal to the number of sounding AMS transmit antennas NSANt.

-------------------------------  Text End  ---------------------------------------------------

Remedy #4. From line 36, page 310 to line 13, page 319.

Change Nt to NEFt

Change “Number of transmit antennas” to “The effective number of transmit antennas”