| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Broadcast Control Information Elements (SFH-IE) for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Sassan Ahmadi, Shantidev Mohanty Intel | Emails: Sassan.ahmadi@intel.com Shantidev.mohanty@intel.com |
| Re: | TGm AWD |  |
| Abstract | The contribution proposes the modifications to the DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Broadcast Control Information Elements (SFH-IE) for IEEE 802.16m Amendment

Sassan Ahmadi, Shantidev Mohanty

Intel

- 1 Text Proposal for Modification of P/S-SFH IE information fields

This contribution proposes modification to the information elements of P-SFH and S-SFH IEs.

Proposed Text #1

-------------------------------  Proposed Text #1 Start  ---------------------------------------------------

S-SFH SP1 IE contains information for network re-entry, see Table 656

Table 656 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSB of superframe number | [8] | Remaining bit of SFN except LSB of SFN in P-SFH |
| LSB of 48 bit BS MAC ID | 24 12 | Specifies the 24 least bit of BS ID |
| Periodicity of A-MAP | 1 | 0b0: every subframe 0b1: every 2 subframes |
| DL Ack/Nack channel resource size | 2 |  |
| Power control channel resource size indicator | 2 |  |
| Non-user specific AMAP location | 1 | Reuse 1 or reuse 3 |
| DL permutation configuration (CRU, DRU partitioning and signaling related to that) | [22] 13 | DL_CAS_SB0(4), DL_CAS_MB0(6), DL CAS_SBi (4x3) (Up to 22 bits, Need the decision from DL physical structure section) |
| UL permutation configuration (CRU, DRU partitioning and signaling related to that) | [22] 13 | UL_CAS_SB0(4), UL_CAS_MB0(6), UL_CAS_SBi (4x3) (Up to 22 bits, Need the decision from UL physical structure section) |
| Intial Unsync ranging allocation interval channel information (initial ranging region location periodicity) | TBD 5 | (Need the decision from UL Ctrl section) |
| Unsync ranging location in the frame (time and frequency) | 4 | Assuming max 4 UL subframes per frame. Bitmap 1: indicating an allocation. The first bit corresponds to the 1st UL sub-frame. |
| RNG codes information | 12 3 | Parameters for determining the root sequences and their cyclic shifts in the preamble set for the cell (Up to 12 bits, Need the decision from UL Ctrl section) 000: 001: 010: |
| HO ranging codes | 6 4 | 64 RNG codes (Need the decision from UL Ctrl section) |
| HO Ranging backoff start | 4 | Initial backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| HO Ranging backoff end | 4 | Final backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| BS EIRP | 16 8 | Signed in units of 1 dBm |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for any new initial entry |
| Reserved | TBD |  |
| } |  |  |

S-SFH SP2 IE contains information for initial network entry and network discovery, see Table 657

Table 657 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Duplexing mode | 1 |  |
| Sub-frame configuration (DL/UL ratio, duplexing mode) | 6 |  |
| If (Duplexing mode == FDD) { |  |  |
| UL carrier frequency | [12] | (Need more discussion) |
| UL bandwidth | [3] | (Need more discussion) |
| } |  |  |
| MSB bytes of BSID | 24 | Specifies the Operator ID |
| MAC protocol revision | 4 | Version number of IEEE 802.16m supported on this channel |
| FFR partitioning info for DL region | [11] | DL_SAC(4), DL_FPSC(3), DL_FPC(4) (Up to 11 bits, Need the decision from DL physical structure section) |
| FFR partitioning info for UL region | [11] | UL_SAC(4), UL_FPSC(3), UL_FPC(4) (up to 11 bits, Need the decision from UL physical structure section) |
| Initial ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| Initial ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| NSP ID | 24 | Network service provider ID |
| Additional broadcast information indicator (ABI) | TBD | This field provides the frame offset where a generic MAC management message containing information about EMBS, LBS, inter-RAT etc is transmitted. The MAC management has a bit map for each type of messages when they are present. |
| MS Transmit Power Limitation Level | 8 | Unsigned 8-bit integer. Specifies the maximum allowed MS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| Minimum level of power offset adjustment | TBD |  |
| Maximum level of power offset adjustment | TBD |  |
| NI information for UL power control | 10 |  |
| SA-preamble soft partitioning information | TBD | Preamble indication of cell types. |
| Pointer to EMBS zone | TBD | This field provides information for an AMS to find the location of EMBS zone inside a super-frame |
| reserved |  |  |
| } |  |  |

S-SFH SP3 IE contains remaining essential system information, see Table 658.

Table 658 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Periodic ranging channel information (periodic ranging region location) | TBD | Need the decision from UL Ctrl section |
| Periodic ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| Periodic ranging backoff start | 4 | Initial backoff window size for periodic ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Periodic ranging backoff end | 4 | Final backoff window size for periodic ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| UL FB Size | 4 | Specifies the size of UL feedback channel per a UL subframe (Need the decision from UL Ctrl section) |
| # Tx antenna | 2 | 0b00: 2 antennas 0b01: 4 antennas 0b10: 8 antennas 0b11: reserved |
| Default RSSI and CINR averaging parameter | 8 | Bits 0-3: Default averaging parameter αavgfor physical CINR measurements, in multiple of 1/16 Bits 4-7: Default averaging parameter αavgfor RSSI measurements, in multiple of 1/16 |
| Tx Power report | TBD |  |
| SP scheduling periodicity information | TBD |  |
| Maximum rate of change of SP information | TBD |  |
| MIMO rank 1 OL region | 8 |  |
| MIMO rank 1 OL region signaling | 3 |  |
| FFR Partition resource Metrics | 16 |  |
| HO Ranging backoff start | 4 | Initial backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| HO Ranging backoff end | 4 | Final backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| Frame offset where new S-SFH information is used | TBD |  |
| Reserved | TBD |  |
| } |  |  |

-------------------------------  Proposed Text #1 End  ---------------------------------------------------

- 1 Text Proposal To Remove Inconsistencies between P/S-SFH IE information field definition and their usage

There are several inconsistencies between the definition of information fields of P/S-SFH IEs and their usage for different operations. This comments identifies such inconsistencies and resolves those so that the definition and usage of P/S-SFH IE information fields are consistent.

Proposed Text #2

-------------------------------  Proposed Text #2 Start  ---------------------------------------------------

[Modify line 58-61 of page 139 as follows]

The frequency partition configuration is transmitted in the S-SFH SP1 in a 4 or 3-bit called the Downlink Frequency Partition Configuration (DFPC) depending on system bandwidth. Frequency Partition Count (FPCT) defines the number of frequency partitions.

-------------------------------  Proposed Text #2 End ---------------------------------------------------