| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Broadcast Control Information Elements (SFH-IE) for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Yih-Shen Chen, Kelvin Chou, I-Kang FuMediaTek Inc. Ming-Hung Tao, Ying-Chuan Hsiao ITRI Kiran Kuchi, J. Klutto Milleth CeWit ZEXIAN LI Nokia Sassan Ahmadi, Shantidev Mohanty Intel | Emails: Sassan.ahmadi@intel.com Shantidev.mohanty@intel.com |
| Re: | TGm AWD |  |
| Abstract | The contribution proposes the modifications to the DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Broadcast Control Information Elements (SFH-IE) for IEEE 802.16m Amendment

Yih-Shen Chen, Kelvin Chou, I-Kang Fu

MediaTek Inc.

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Kiran Kuchi, J. Klutto Milleth

CeWit

Sassan Ahmadi, Shantidev Mohanty

Intel

- 1 Text Proposal for Modification of P/S-SFH IE information fields

This contribution proposes modification to the information elements of P-SFH and S-SFH IEs.

Proposed Text #1

-------------------------------  Proposed Text #1 Start  ---------------------------------------------------

[Note to Editor: Add the following text to Table 656]

Table 656 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| Start superframe offset where new SP1 information is used | 2 |  |
| Reserved | TBD |  |

[Note to Editor: Add the following text to Table 657]

Table 657 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Start superframe offset where new SP2 information is used | 2 |  |
| } |  |  |

[Note to Editor: Add the following text to Table 658]

Table 658 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Start super-frame offset where new SP3 information is used | 2 |  |
| Rate of change of SP (1-3) info | 4 |  |
| SA-sequence soft partitioning information | 4 |  |
| FFR Partition resource Metrics | TBD |  |
| MIMO rank 1 OL region signaling | TBD |  |
| MIMO rank 1 OL region | TBD |  |
| NI information for UL power control | TBD |  |
| } |  |  |

-------------------------------  Proposed Text #1 End  ---------------------------------------------------

- 1 Text Proposal To Remove Inconsistencies between P/S-SFH IE information field definition and their usage

There are several inconsistencies between the definition of information fields of P/S-SFH IEs and their usage for different operations. This comments identifies such inconsistencies and resolves those so that the definition and usage of P/S-SFH IE information fields are consistent.

Proposed Text #2

-------------------------------  Proposed Text #2 Start  ---------------------------------------------------

[Modify line 58-61 of page 139 as follows]

The frequency partition configuration is transmitted in the S-SFH SP1 in a 4 or 3-bit called the Downlink Frequency Partition Configuration (DFPC) depending on system bandwidth. Frequency Partition Count (FPCT) defines the number of frequency partitions.

-------------------------------  Proposed Text #2 End ---------------------------------------------------