| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for UL feedback allocation IE and feedback contents |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jinyoung Chun, Wookbong Lee, Bin-Chul Ihm LG Electronics. | jychun03@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (Downlink control structure) and Chapter 15.3.9 (Uplink control channel) “Comments on AWD 15.3.6 DL-CTRL and 15.3.9 UL-CTRL” |  |
| Abstract | This contribution proposes the clarification of MIMO feedback mechanism. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for UL Feedback Allocation IE and feedback contents

Jinyoung Chun, Wookbong Lee, and Bin-Chul Ihm

LG Electronics

- Introduction

In Amendment Working Document, MIMO feedback mechanism is not completed. We propose to define Feedback format and contents based on MIMO feedback modes and to clarify UL feedback allocation IE.

- Feedback contents in AWD

There is a table defined MIMO feedback modes. It is baseline to make feedback mechanism.

Table 691—MIMO feedback modes

| Feedback Mode | Description | Feedback contents | Type of RU | Supported MIMO transmission mode |
| --- | --- | --- | --- | --- |
| Mode 0 | OL SU MIMO SFBC/SM (Diversity) | 1. STC Rate 2. Wideband CQI | Diversity(DRU, Mini-band based CRU) | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI STC Rate 2: SM CQI In DRU: Mt=2 for SM.In Miniband based CRU: Mt >=2 for SM |
| Mode 1 | OL SUMIMO SM (Diversity) | 1. STC Rate 2. Wideband CQI | Diversity(Mini-band based CRU) | MIMO mode 1 |
| Mode 2 | OL SUMIMO SM (localized) | 1. STC Rate 2. Subband CQI 3. Subband Selection | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 1 |
| Mode 3 | CL SU MIMO (localized) | 1. STC Rate 2. Subband CQI 3. Subband PMI 4.Subband selection [5. Wideband PMI] 6. Wideband correlation Matrix | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 2 |
| Mode 4 | CL SU MIMO (Diversity) | 1. Wideband CQI [2. Wideband PMI] 3. Wideband correlation Matrix | Diversity(Mini-band based CRU) | MIMO mode 2 (Mt=1) |
| Mode 5 | OL MU MIMO (localized) | 1. Subband CQI 2. Subband Selection 3. Stream indicator | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 3 |
| Mode 6 | CL MU MIMO (localized) | 1. Subband CQI 2. Subband PMI 3. Subband Selection [4. Wideband PMI] 5. Wideband correlation Matrix | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 4 |
| Mode 7 | CL MU MIMO (Diversity) | 1. Wideband CQI [2. Wideband PMI] 3. Wideband correlation Matrix | Diversity (Mini-band based CRU) | MIMO mode 4 |

- Text proposal for inclusion in the 802.16m amendment

[Add the texts and figures marked in blue and remove the texts and figures marked in red in the IEEE 802.16m-09/0010r2]

[ Remedy 1. Adopt the below changes from page 224.]

---------------------------------------------  Text Start  --------------------------------------------------------

15.3.6.5.2.9 Feedback Allocation A-MAP IE

Table 682 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating UL fast feedback control channels (including both PFBCH and SFBCH) to an AMS.

Definitions of the fields in the Feedback Allocation A-MAP IE are listed below in Table 682.Note: 1. Need to differentiate it is in OL region or not. In OL region, MaxMt is not required.

Table 682—Feedback Allocation A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Feedback Allocation MAP IE() { |  |  |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE = 0b0010 |
| Channel Index | Variable 6 | Feedback channel index within the UL fast feedback control resource region (Dependent on LFB,FPi The maximum value is LFB defined in 15.3.8.3.3.2) |
| Allocation Period (p) | [3] | A feedback is transmitted on the FBCH every 2p frames |
| Long-term Allocation Period (LP) (TBD) | 2 | TBD A feedback with long-term information defined 15.3.9.3.1.4 Feedback Format is transmitted on the FBCH every 2p+LP frames. |
| Frame offset(TBD) |  | Depends on the decision of A-MAP allocation IE |
| Subframe offset(s) | 3 | MS transmits fast feedback channel in s-th subframe. |
| Allocation Duration(d) | [3] | A fast feedback is transmitted on the fast feedback channels for 2d superframes. If d == 0b000, the channel is deallocated. If d == 0b111, the MS should report until the BS command for the MS to stop. |
| MaxMt | Variable [1-2] | Variable number of bits - depends on number of transmit antenna Nt. If Nt=2: 0b0: 1, 0b1: 2 If Nt=4: 0b00: 1, 0b01: 2, 0b10: 3, 0b11: 4 If Nt=8 (SU-MIMO) 0b000: 1, 0b001: 2, 0b011: 4, 0b111: 8 If Nt=8: (MU-MIMO) 0b00: 1, 0b01: 2, 0b10: 3, 0b11: 4 |
| MFM | [3] | MIMO Feedback Mode as defined in Table 671691 |
| Feedback Format | Variable [2-3] 1 | Variable number of bits – It depends on MIMO feedback mode. See 15.3.9.3.1.4 Feedback format. |
| If (MFM=0,1,4,7) { |  | (TBD depending on IM group decision on FFR) |
| Number of Reports (n) |  | 0: Same Feedback reported on each FFB opportunity 1: Different Feedback reported in a time-interleaved fashion over the PFBC opportunities |
| If (Number of Reports>0) { |  | (TBD depending on IM group decision on FFR) |
| Long Allocation Period_multiple_reports (q) |  | Long Period Fast Feedback reports are transmitted on the FBCH channel every 2p+q+1 frames |
| } |  |  |
| For (i=0 to n) { |  | For specified Number of Reports (n) (TBD depending on IM group decision on FFR) |
| FPS |  | Frequency Partition and Reuse Factor as follows: (Refer to section 20.1 for the frequency partition indexing definition.) 0b00: Frequency Partition 0 (Reuse 1) 0b01: Deboosted reuse 3 0b10: Boosted Reuse 3 0b11: Reserved. |
| } |  |  |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | Variable [0,2] | Codebook Feedback Mode and Codebook Coordination Enable 0b00: standard with CCE disabled 0b01: adaptive with CCE disabled 0b10: differential with CCE disabled 0b11: standard with CCE enabled If MFM = 2,5 CM is empty (0 bit) |
| CS indication | [1] | Codebook subset |
| } |  |  |
| IF (MFM == 2,3,5,6) (TBD) { |  | Localized feedback mode |
| If (feedback_Format == 0) { |  | Specified CQI (used to report specified DLRU/subband CQI) |
| DLRU/subband index | Variable [3-4] | Variable number of bits – depends on system bandwidth 0b000-0b110 (3bits): to index 6 subband when system bandwidth is 5MHx 0b0000-0b1100 (4bits): to index 12 subband when system bandwidth is 10MHz |
| } |  |  |
| } |  |  |
| Padding |  |  |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } |  |  |

Channel Index: Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between Channel Index and the AMS. In case of multiple SFBCH are allocated for an AMS, it is assumed that multiple SFBCH are allocated contiguously and this field specifies the index of the first one.

MFM: MIMO feedback mode, defined in Table 691.

Feedback Format: This field specifies the feedback format index when reporting fast feedback information in FBCH. Feedback format definitions for different MIMO feedback modes are described in 15.3.9.3.1.4 Error! Reference source not found.

Long-term Allocation Period: Long term period report punctures the normal report.

Number of Reports (n): Indicates whether the FFB channel allocated to the AMS carries 1 or 2 PFBCH Reports.

Allocation Period (p):

Long Allocation Period multiple reports (q): Indicates the allocation period for Long-Period report and these puncture the Short- Allocation Period reports when MFM=0, 1, 4, 7.

FPS: Frequency Partition Selection & Reuse Factor Information. (Refer to <<<section 20.1>>> for the frequency partition indexing definition)

MaxMt: This field specifies the maximum rank to be fed back by the AMS if MFM=0,1,2,3,4 (which indicates a SU MIMO feedback mode for SM), or it specifies the maximum number of users scheduled on each RU at the BS if MFM=5,6,7 (which indicates a MU MIMO feedback mode)

CCE: Codebook Coordination Enable. When CCE is enabled, MS finds PMI within whole broadcasted codebook type entry; when this fields is set to 0b1, it means MS finds rate-1 PMI within broadcasted codebook entries indicated by BC_SI and codebook subset indication (CS in DL A-MAP IE).

DLRU: This field specifies the index of a certain RU which needs AMS to estimate and report CQI/PMI feedback. When a certain RU is specified, AMS only report one CQI (may plus RI)/PMI for it.

CM: this field specifies codebook feedback mode, which is needed only for MFM 3 and 6.

---------------------------------------------  Text End  --------------------------------------------------------

[Remedy 2. Adopt the below changes from page 300.]

---------------------------------------------  Text Start  --------------------------------------------------------

15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback and MIMO feedback.

Table 725—PFBCH Feedback Content

| PFBCH Feedback Content | Related MIMO feedback mode | Description/Notes |
| --- | --- | --- |
| CQI |  | 1) Wideband CQI 2) Subband CQI for Best -1 subband (TBD) 3) Average CQI over resource units indicated by BS (TBD) |
| STC Rate Indicator |  | Number of streams |
| BSI (Band Selection Info for best-1 subband) (TBD) |  |  |
| PMI (TBD) |  | 1) wideband PMI (TBD) 2) subband PMI for best-1 |
| Event-driven Indicator (EDI) for CQICH de-allocation (TBD) | N/A | MS to indicate channel variation event |
| Event-driven for confirmation of CQICH de-allocation. (TBD) |  |  |
| Event-driven Indicator (EDI) for Preferred MIMO operation (TBD) |  | Sequences: Option 1: 1) OL distributed 2) OL localized 3) CL distributed 4) CL localized Option 2: 1) OL 2) CL 3) SU 4) MU MaxMt =2 5) MU MaxMt = 3 6) MU MaxMt = 4 Option 3: 1) two steps method |
| Event-driven Indicator (EDI) for Multi-cell MIMO Initiation (TBD) |  |  |
| Event-driven Indicator (EDI) for Bandwidth Request Indicator | N/A | This is used to request UL bandwidth. 2 sequences (two services) |
| Event-driven Indicator (EDI) for Frequency partition selection (FPS) (TBD) | N/A | Inform BS the frequency partition of the reported CQI (DL diversity mode only) |
| Event-driven Indicator (EDI) for Buffer management (TBD) | N/A | Indicates occupancy status of HARQ soft buffer |

15.3.9.3.1.2 Secondary fast feedback control channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive.

Table 726—SFBCH Feedback Content

| PFBCH Feedback Content | Related MIMO feedback mode | Description/Notes |
| --- | --- | --- |
| Subband CQI |  | Reporting of average and differential CQI of selected sub-bands. Reporting of transformed coefficients is TBD |
| Band selection information |  | Selected band position. Index to indicate sub-band selection If transformed coefficients method is used, band selection is not necessary. |
| Subband PMI |  | Precoding Matrix Indicator of one sub-band for CL MIMO |
| Wideband PMI (TBD) |  | Wideband PMI with subband CQI |
| Stream Indicator |  | It is needed for OL MU MIMO only and used to indicate which spatial stream to estimate CQI |
| STC Rate Indicator |  | Number of streams |
| Event-Driven (TBD) |  |  |

---------------------------------------------  Text End  --------------------------------------------------------

[Remedy 3. Adopt the below changes from page 303.]

---------------------------------------------  Text Start  --------------------------------------------------------

15.3.9.3.1.4 Feedback format

Feedback format for MFM 0,1,4,7

| Feedback format | FBCH | Number of reports | Feedback Fields | Size in bits | Description / Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 1 | Rank, WB CQI |  | MFM 0, 1, 7(if no PMI) |
| 1 | PFBCH | 1 | Rank, WB CQI, WB PMI |  | MFM 4, 7 (if PMI) |

Feedback format for MFM 2

The detailed format is listed in Table 727. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 727—Feedback formats for MIMO feedback mode 2

| Feedback format | FBCH | Number of reports | Feedback Fields | Size in bits | Description / Notes |
| --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |
| 1 (M=1) |  |  |  |  |  |
| 2 (M=3) |  |  |  |  |  |

Feedback format for MFM 3 and 6

The detailed format is listed in Table 728. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 728—Feedback formats for MIMO feedback mode 3 and 6

| Feedback format | FBCH | Number of reports | Feedback Fields | Size in bits | Description / Notes |
| --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |
| 1 (M=1) |  |  |  |  |  |
| 2 (M=3) |  |  |  |  |  |

Feedback format for MFM 5

The detailed format is listed in Table 729. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 729—Feedback formats for MIMO feedback mode 5

| Feedback format | FBCH | Number of reports | Feedback Fields | Size in bits | Description / Notes |
| --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |
| 1 (M=1) |  |  |  |  |  |
| 2 (M=3) |  |  |  |  |  |

Feedback format defines the detail feedback fields depending MIMO feedback mode and the measured frequency bands.

Table 727- Feedback format for MIMO feedback mode 0, 1

| Feedback format | Types of FFBCH | Number of reports | Feedback Fields (Allocation period) | Feedback Fields (Long-term allocation period) |
| --- | --- | --- | --- | --- |
| 0 | PFBCH | 1 | STC rate + Wideband CQI | - |

Table 728 - Feedback format for MIMO feedback mode 2

| Feedback format | Types of FFBCH | Number of reports | Feedback Fields (Allocation period) | Feedback Fields (Long-term allocation period) |
| --- | --- | --- | --- | --- |
| 0 (M=1) | PFBCH | 1 | STC rate + 1 subband CQI | Subband selection information |
| 1 (M=3) | SFBCH | 1 | STC rate, Average CQI + 2bit differential CQIs per subband | Subband selection information |

Table 729 - Feedback format for MIM feedback mode 3, 6

| Feedback format | Types of FFBCH | Number of reports | Feedback Fields (Allocation period) | Feedback Fields (Long-term allocation period) |
| --- | --- | --- | --- | --- |
| 0 (M=1) | PFBCH | 2 | STC rate + 1 subband CQI, | Subband selection information |
|  |  |  | 1 subband PMI | - |
| 1 (M=3) | SFBCH | 2 | STC rate, Average CQI + 2bit differential CQIs per subband | Subband selection information |
|  |  |  | Subband PMIs | - |

Table 730 - Feedback format for MIMO feedback mode 4, 7

| Feedback format | Types of FFBCH | Number of reports | Feedback Fields (Allocation period) | Feedback Fields (Long-term allocation period) |
| --- | --- | --- | --- | --- |
| 0 | PFBCH | 1 | Wideband CQI | - |

Table 731 - Feedback format for MIMO feedback mode 5

| Feedback format | Types of FFBCH | Number of reports | Feedback Fields (Allocation period) | Feedback Fields (Long-term allocation period) |
| --- | --- | --- | --- | --- |
| 0 (M=1) | PFBCH | 1 | Stream indicator, 1 subband CQI | Subband selection information |
| 1 (M=3) | SFBCH | 1 | Stream indicator, Average CQI + 2bit differential CQIs per subband | Subband selection information |

---------------------------------------------  Text End  --------------------------------------------------------

- Reference

[1] IEEE 802.16m System Requirements Document, IEEE 802.16m-07/002r8, January 2009.

[2] IEEE 802.16m Evaluation Methodology Document, IEEE 802.16m-08/004r5, January 2009.

[3] IEEE 802.16m Amendment Working Document, IEEE 802.16m-09/0010r2, March 2009.