| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD Comment on zone switch from MZone to LZone (AWD-15.2.6.4.1.4) |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Yang Liu，Hongyun Qu, Mary Chion, Yuqin Chen ZTE Corporation Kelvin Chou, Yih-Shen Chen MediaTek Inc. Inuk Jung LGE | liu.yang8@zte.com.cn qu.hongyun@zte.com.cn mchion@zteusa.com kelvin.chou@mediatek.com yihshen.chen@mediatek.com cooper@lge.com |
| Re: | 802.16m amendment working document. |  |
| Abstract | This contribution provides more details for zone switch from MZone to LZone . |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- AWD Comments on Zone Switch from MZone to LZone (AWD-15.2.6.4.1.4)

Yang Liu, Hongyun Qu, Mary Chion, Yuqin Chen

ZTE Corporation

Kelvin Chou and Yih-Shen Chen

MediaTek Inc.

Inuk Jung

LGE

- Introduction

- In current AWD (C802.16m-09/0010r2) text, the zone switch from MZone to LZone is indicated by AAI-HO_CMD message.  Since AMS obtains some LZone information other than normal HO from 16m ABS to 16m ABS in this case, there should be an indication which identify the AAI-HO-CMD is used for zone switch.

- This contribution proposed two revisions. The difference lies how we reuse the current AAI-HO_CMD.

- For instance, since the MZone and LZone are time divided, the HO_Renntry_interval in AAI-HO_CMD may be set zero even if the AMS is working in a single carrier mode. In the case of HO_Renntry_interval is found, the HO_Reentry_Mode is set one according to current AWD text.

The revised Text 1

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

15.2.6.4.1.4 Zone switch from MZone to LZone

The ABS indicates zone switch of AMSs that currently operate in the MZone for several reasons, such as load balancing purposes. The HO-CommandAAI_HO-CMD message is used to trigger the zone switch from MZone to LZone TBD when HO Mode =0b01.. Zone switch from LZone to MZone follows the procedure per section 15.2.5.3.1.2. During the request or command of the zone switch to LZone , The AMS is provided with LZone information in prior, such as CID, FID, security parameters or capability information in the MZone via the AAI-HO-CMD message. After receiving the AAI_HO-CMD with HO_Mode = 0b01, the AMS maintains communication with MZone until the network reentry is finished in the LZone.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

Revised text 2

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

15.2.6.4.1.4 Zone switch from MZone to LZone

The ABS indicates zone switch of AMSs that currently operate in the MZone for several reasons, such as load balancing purposes. The HO-CommandAAI_HO-CMD message is used to trigger the zone switch from MZone to LZone TBD when HO Mode =0b01.. Zone switch from LZone to MZone follows the procedure per section 15.2.5.3.1.2. During the request or command of the zone switch to LZone , When the AMS is instructed by the ABS to perform zone switch from MZone to LZone, the AMS is provided with LZone information in prior, such as CID, FID, security parameters or capability information in the MZone via the AAI-HO-CMD message. In this case, when HO_Reentry_Mode is set to one, the AMS maintains communication with MZone until the network reentry is finished in the LZone.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

- We prefer text 2 since it has more flexibility.

- AWD Text Proposal

Black text: existing text in AWD (C802.16m-09/0010r2)

Red Strikethrough text: deleted text

Blue text: new text

15.2.6.4.1.4 Zone switch from MZone to LZone

The ABS indicates zone switch of AMSs that currently operate in the MZone for several reasons, such as load balancing purposes. The HO-CommandAAI_HO-CMD message is used to trigger the zone switch from MZone to LZone TBD when HO Mode =0b01.. Zone switch from LZone to MZone follows the procedure per section 15.2.5.3.1.2. During the request or command of the zone switch to LZone , When the AMS is instructed by the ABS to perform zone switch from MZone to LZone, the AMS is provided with LZone information in prior, such as CID, FID, security parameters or capability information in the MZone via the AAI-HO-CMD message. In this case, when HO_Reentry_Mode is set to one, the AMS maintains communication with MZone until the network reentry is finished in the LZone.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++