| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of hierarchically advertising the available intervals for femtocell's low-duty operation |  |
| Date Submitted | 2008-07-06 |  |
| Source(s) | Namgi Kim Kyonggi University Hyun Lee, Jaesun Cha, Kwangjae Lim, Won-Ik Kim, Soojung Jung, Juhee Kim, Eunkyung Kim, Chulsik Yoon ETRI | Voice: +82-31-249-9662 E-mail: ngkim@kyonggi.ac.kr Voice: +82-42-860-5456 E-mail: {hyunlee, csyoon}@etri.re.kr |
| Re: | Category: AWD-New contribution /Area: Femtocell BS |  |
| Abstract | This contribution provides a text for hierarchically advertising the available intervals for femtocell's low-duty operation in IEEE 802.16m. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of hierarchically advertising the available intervals for femtocell's low-duty operation

Namgi Kim

Kyonggi University

Hyun Lee, Jaesun Cha, Kwangjae Lim, Won-Ik Kim, Soojung Jung, Juhee Kim, Eunkyung Kim, Chulsik Yoon

ETRI

# Introduction

According to the SDD [1], femtocell BSs may support low-duty operation mode in order to reduce interference to neighbor cells. The low-duty operation mode consists of available and unavailable internals. During an unavailable interval, the femtocell BS does not transmit anything on the air interface. By the way, an MS which wants to enter a femtocell in low-duty operation mode has to previously know available intervals of the femtocell BS to scan it. However, the complete pattern of available intervals may be different at every femtocell BS due to diverse sleep and idle patterns of MSs. Informing an MS of complete available intervals of all femtocell BSs at a time may be large overhead to the system. Therefore, we propose a way to efficiently inform an MS of minimum available intervals for scanning and complete available intervals for full low-duty operation.

# Proposal

In order to efficiently notify available intervals to MSs, we hierarchically inform MSs of available intervals of femtocell BSs. Before entering the femtocell in low-duty operation mode, an MS needs to just know minimum available intervals for scanning the femtocell BS. The minimum available intervals may have the same pattern in many femtocell BSs and can be easily notified to MSs by transmitting on the air interface through macrocell BSs, previously writing in the white list of MSs, or many other ways. After entering the femtocell, an MS gets complete pattern of available intervals to fully cooperate with the femtocell BS in the low-duty operation mode. Informing an MS of complete pattern of available intervals may be performed by the entered femtocell BS or other ways. Figure 1 shows the example of hierarchically advertising available intervals. In the example, there are two different complete patterns of available intervals from two femtocells. However, if an MS only knows the minimum available intervals, that are the common parts of available intervals of two femtocells, the MS can scan both of them before entering femtocells.

Figure 1. Example of hierarchically advertising available intervals

# References

[1] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, May 2009.

# Proposed Text

-------- Start of the text proposal ---------------------------------------------------------------------------------------

### 15.2.x Femtocell Support

### 15.2.x.x Femtocell low-duty operation mode

In order to efficiently notify available intervals to MSs, the pattern of available intervals may be hierarchically notified to MSs. Before entering the femtocell in low-duty operation mode, an MS may get minimum available intervals for scanning the femtocell BS. After entering the femtocell, an MS may get complete pattern of available intervals to fully cooperate with the femtocell BS in the low-duty operation mode. Figure xxx shows the example of hierarchically advertising available intervals. In the example, there are two different complete patterns of available intervals from two femtocells. However, if an MS only knows the minimum available intervals, that are the common parts of available intervals of two femtocells, the MS can scan both of them before entering femtocells.

Figure xxx. Example of hierarchically advertising available intervals

-------- End of the text proposal ---------------------------------------------------------------------------------------