| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Primary Advanced Preamble for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-7-6 |  |
| Source(s) | Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho ITRI | zhengyanxiu@itri.org.tw |
| Re: | IEEE 802.16m-09/0028r1, Call for contributions on 16m AWD content “Comments on AWD 15.3.6.1.1 Primary Advanced Preamble” |  |
| Abstract | The contribution proposes the text of synchronization channel of DL control structure section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Primary Advanced Preamble 
for the IEEE 802.16m Amendment

Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho

ITRI

- Introduction

The contribution proposes the PA-Preamble for A-Preamble. The proposed PA-Preamble improves the performance A-Preamble disclosed in IEEE 802.16m-09/0010r2[1]. The proposed PA-Preamble features partially overlapped MFSK. This proposal also introduces cluster-based phase rotation MIMO scheme to enhance the reception performance of PA-Preamble for the scenario that whole network sends identical PA-Preamble.

- References

[1] IEEE 802.16m-09/0010r2, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems Advanced Air Interface (working document),” May 2009.

.

- Text proposal for inclusion in the 802.16m amendment

---------------------------------------------  Text Start  ---------------------------------------------------

Delete the following text at at section 15.3.6.1.1:

The length of sequence for PA-Preamble is 216 regardless of the FFT size. PA-Preamble carries the information of ABS type, system bandwidth, and carrier configuration. When the subcarrier index 256 is reserved for DC, the allocation of subcarriers is accomplished by following equation

PAPreambleCarrierSet= 2∙k +41

where

PAPreambleCarrierSet specifies all subcarriers allocated to the PA-Preamble, and

k 	is a running index 0 to 215.

Figure xxx depicts the symbol structure of PA-Preamble in the frequency domain.

Figure xxx─ PA-Preamble symbol structure

In Table xxx the sequence of PA-Preamble is defined in a hexadecimal format. The defined series is mapped onto subcarriers in ascending order. The value of the series is obtained by converting the series to a binary series and starting the series from the MSB up to 216 bits (0 mapped to +1 and 1 mapped to -1).

Table xxx─ PA-Preamble series

| Index | Carrier | BW | Series to modulate |
| --- | --- | --- | --- |
| 0 | Fully configured | 5 MHz | 6DB4F3B16BCE59166C9CEF7C3C8CA5EDFC16A9D1DC01F2AE6AA08F |
| 1 |  | 10 MHz | 1799628F3B9F8F3B22C1BA19EAF94FEC4D37DEE97E027750D298AC |
| 2 |  | 20 MHz | 92161C7C19BB2FC0ADE5CEF3543AC1B6CE6BE1C8DCABDDD319EAF7 |
| 3 |  | reserved | 6DE116E665C395ADC70A89716908620868A60340BF35ED547F8281 |
| 4 |  | reserved | BCFDF60DFAD6B027E4C39DB20D783C9F467155179CBA31115E2D04 |
| 5 |  | reserved | 7EF1379553F9641EE6ECDBF5F144287E329606C616292A3C77F928 |
| 6 |  | reserved | 8A9CA262B8B3D37E3158A3B17BFA4C9FCFF4D396D2A93DE65A0E7C |
| 7 |  | reserved | DA8CE648727E4282780384AB53CEEBD1CBF79E0C5DA7BA85DD3749 |
| 8 |  | reserved | 3A65D1E6042E8B8AADC701E210B5B4B650B6AB31F7A918893FB04A |
| 9 |  | reserved | D46CF86FE51B56B2CAA84F26F6F204428C1BD23F3D888737A0851C |
| 10 | Partially configured | N/A | 640267A0C0DF11E475066F1610954B5AE55E189EA7E72EFD57240F |

The magnitude boosting levels for different FFT size are as follows:

| 512 | 1k | 2k |
| --- | --- | --- |
| 2.3999 | 3.4143 | 5.1320 |

For 512-FFT, the boosted PA-Preamble at k-th subcarrier can be written as

ck=2.3999⋅bk

where bkrepresents PA-Preamble before the boosting (+1 or - 1).

Insert the following texts at section 15.3.6.1.1:

The length of sequence for Preamble is 212 regardless of the FFT size. PA-Preamble carries the information of ABS type, system bandwidth and carrier configuration. When the subcarrier index 256 is reserved for DC, the allocation of subcarriers is accomplished by following equation

PreambleCarrierSet= 2∙k +45

where PAPreambleCarrierSet specifies all subcarriers allocated to the PA-Preamble, and

k 	is a running index 0 to 211.

Figure yyy depicts the symbol structure of PA-Preamble in the frequency domain.

Figure yyy─ PA-Preamble symbol structure

Table xxx provides preamble code. C(i) stands for the codes on the ith subcluster. The nth preamble sets C(1) on subcarrier indexes {45+6·n, 47+6·n, 49+6·n }, C(2) on subcarrier indexes {151+6·(3·n mod 17), 153+6·(3·n mod 17), 155+6·(3·n mod 17)}, C(3) on subcarrier indexes {253, 255, 257}, C(4) on subcarrier indexes {261+6·(5·n mod 17), 263+6·(5·n mod 17), 265+6·(5·n mod 17)}, C(5) on subcarrier indexes {367+6·(7·n mod 17), 369+6·(7·n mod 17), 371+6·(7·n mod 17)} and {0} on the other subcarriers indexes. All preambles has the same C(3) on the same location to indicate the center frequency.

There are 7 phase rotation vectors for each sequence. j means−1. These five numbers are phase rotation for C(1) to C(5). Different antennas or base stations can apply different phase rotation to exploit spatial domain diversity gain.

When there is a transmitter with two transmission antennas, the transmitter will sends the preamble with two different phase rotation vector shown in the table. We can restrict the select of phase rotation vectors to the first two.

The magnitude boosting levels for different FFT size are shown in Table ABC.

Table ABC—PA-Preamble boosting levels.

| 512 | 1k | 2k |
| --- | --- | --- |
| 7.8819 | 11.2135 | 16.8548 |

For 512-FFT, the boosted PA-Preamble at kth subcarrier can be written as

ck=7.8819⋅bk

where bkrepresents PA-Preamble before the boosting (+1, - 1, +j or - j).

When we apply all phase rotation vectors, the magnitude boosting levels for different FFT size are shown in Table DEF.

Table DEF—PA-Preamble boosting levels.

| 512 | 1k | 2k |
| --- | --- | --- |
| 7.3526 | 10.4604 | 15.7230 |

For 512-FFT, the boosted PA-Preamble at kth subcarrier can be written as

ck=7.3526⋅bk

where bkrepresents PA-Preamble before the boosting (+1, - 1, + j or - j).

Table xxx: An exemplary preamble.

| Sequence Number | PAPR | Phase rotation vector |  |  |  |  |  |  |  | Minimum PAPR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | C(1) | C(2) |  | C(3) |  |  | C(4) | C(5) |  |
| 0 | C(1)=[-1 -1 -1], C(2)=[-1 1 1], C(3)=[1 1 -1], C(4)=[1 -1 1], C(5)=[-1 1 -1] |  |  |  |  |  |  |  |  |  |
|  | 4.003088183822409 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 4.014190434828029 | 1 |  | j |  | -1 | -1 |  | -j |  |
|  | 4.014190434828031 | 1 |  | -j |  | -1 | -1 |  | j |  |
|  | 4.888527720143059 | 1 |  | -j |  | -1 | j |  | 1 |  |
|  | 4.888527720143064 | 1 |  | j |  | -1 | -j |  | 1 |  |
|  | 4.909385506258269 | 1 |  | -1 |  | 1 | j |  | -j |  |
|  | 4.909385506258273 | 1 |  | -1 |  | 1 | -j |  | j |  |
| 1 | C(1)=[1 -1 1], C(2)=[-1 1 1], C(3)=[1 1 -1], C(4)=[1 -1 -1], C(5)=[1 1 1] |  |  |  |  |  |  |  |  |  |
|  | 4.413854374140299 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 4.448410795286004 | 1 |  | -j |  | J | -1 |  | j |  |
|  | 4.448410795286008 | 1 |  | J |  | -j | -1 |  | -j |  |
|  | 4.628116196814346 | 1 |  | -j |  | 1 | -1 |  | j |  |
|  | 4.628116196814350 | 1 |  | J |  | 1 | -1 |  | -j |  |
|  | 4.662204480684627 | 1 |  | J |  | -1 | -1 |  | -j |  |
|  | 4.662204480684630 | 1 |  | -j |  | -1 | -1 |  | j |  |
| 2 | C(1)=[-1 1 -1], C(2)=[1 -1 -1], C(3)=[1 1 -1], C(4)=[-1 -1 1], C(5)=[1 1 1] |  |  |  |  |  |  |  |  |  |
|  | 4.260525260818429 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 5.193081892861322 | 1 |  | -1 |  | J | 1 |  | -j |  |
|  | 5.193081892861330 | 1 |  | -1 |  | -j | 1 |  | j |  |
|  | 5.300501965823672 | 1 |  | j |  | -j | -1 |  | -j |  |
|  | 5.300501965823674 | 1 |  | -j |  | J | -1 |  | j |  |
|  | 5.313434094457139 | 1 |  | j |  | 1 | -1 |  | -j |  |
|  | 5.313434094457139 | 1 |  | -j |  | 1 | -1 |  | j |  |
| 3 | C(1)=[-1 1 1], C(2)=[1 1 1], C(3)=[1 1 -1], C(4)=[1 -1 1], C(5)=[-1 -1 -1] |  |  |  |  |  |  |  |  |  |
|  | 4.916815730009944 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 5.086490264936585 | 1 |  | -1 |  | J | 1 |  | -j |  |
|  | 5.086490264936586 | 1 |  | -1 |  | -j | 1 |  | j |  |
|  | 5.108205091527410 | 1 |  | -j |  | -1 | -1 |  | j |  |
|  | 5.108205091527417 | 1 |  | j |  | -1 | -1 |  | -j |  |
|  | 5.116539256214900 | 1 |  | j |  | -j | -1 |  | 1 |  |
|  | 5.116539256214903 | 1 |  | -j |  | J | -1 |  | 1 |  |
| 4 | C(1)=[1 1 1], C(2)=[-1 1 -1], C(3)=[1 1 -1], C(4)=[1 -1 -1], C(5)=[-1 -1 -1] |  |  |  |  |  |  |  |  |  |
|  | 4.870544149752826 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 4.970817925965569 | 1 |  | -j |  | J | -j |  | -1 |  |
|  | 4.970817925965577 | 1 |  | j |  | -j | j |  | -1 |  |
|  | 5.081899883347408 | 1 |  | j |  | -1 | -j |  | 1 |  |
|  | 5.081899883347408 | 1 |  | -j |  | -1 | j |  | 1 |  |
|  | 5.981032971234257 | 1 |  | j |  | J | -1 |  | -j |  |
|  | 5.981032971234259 | 1 |  | -j |  | -j | -1 |  | j |  |
| 5 | C(1)=[1 1 1], C(2)=[-1 1 -1], C(3)=[1 1 -1], C(4)=[1 -1 1], C(5)=[-1 -1 1] |  |  |  |  |  |  |  |  |  |
|  | 5.052668329233451 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 5.307024863984728 | 1 |  | 1 |  | -j | j |  | -1 |  |
|  | 5.307024863984731 | 1 |  | 1 |  | J | -j |  | -1 |  |
|  | 5.343596570163389 | 1 |  | j |  | 1 | -1 |  | -j |  |
|  | 5.343596570163399 | 1 |  | -j |  | 1 | -1 |  | j |  |
|  | 5.434252023375128 | 1 |  | j |  | -1 | -1 |  | -j |  |
|  | 5.434252023375135 | 1 |  | -j |  | -1 | -1 |  | j |  |
| 6 | C(1)=[-1 -1 -1], C(2)=[-1 1 -1], C(3)=[1 1 -1], C(4)=[-1 -1 -1], C(5)=[1 -1 1] |  |  |  |  |  |  |  |  |  |
|  | 5.142072883424945 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 5.209624497948164 | 1 |  | j |  | -1 | -j |  | 1 |  |
|  | 5.209624497948171 | 1 |  | -j |  | -1 | j |  | 1 |  |
|  | 5.333605470178338 | 1 |  | j |  | 1 | -1 |  | -j |  |
|  | 5.333605470178338 | 1 |  | -j |  | 1 | -1 |  | j |  |
|  | 5.380644166268082 | 1 |  | -1 |  | -1 | -j |  | j |  |
|  | 5.380644166268085 | 1 |  | -1 |  | -1 | j |  | -j |  |
| 7 | C(1)=[1 1 -1], C(2)=[1 1 1], C(3)=[1 1 -1], C(4)=[-1 -1 -1], C(5)=[-1 1 -1] |  |  |  |  |  |  |  |  |  |
|  | 5.377233540694850 | 1 |  | 1 |  | 1 | 1 |  | 1 |  |
|  | 5.305132940678785 | 1 |  | j |  | -j | -1 |  | -1 | Min |
|  | 5.305132940678790 | 1 |  | -j |  | J | -1 |  | -1 |  |
|  | 5.448732975090332 | 1 |  | -1 |  | J | 1 |  | -j |  |
|  | 5.448732975090341 | 1 |  | -1 |  | -j | 1 |  | j |  |
|  | 5.477789605680488 | 1 |  | j |  | 1 | -1 |  | -j |  |
|  | 5.477789605680488 | 1 |  | -j |  | 1 | -1 |  | j |  |
| 8 | C(1)=[1 -1 1], C(2)=[1 -1 -1], C(3)=[1 1 -1], C(4)=[1 -1 -1], C(5)=[-1 -1 -1] |  |  |  |  |  |  |  |  |  |
|  | 4.441469220819505 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 4.504804368675837 | 1 |  | -j |  | 1 | -1 |  | j |  |
|  | 4.504804368675841 | 1 |  | j |  | 1 | -1 |  | -j |  |
|  | 4.642417406841394 | 1 |  | -j |  | -j | -1 |  | j |  |
|  | 4.642417406841401 | 1 |  | j |  | J | -1 |  | -j |  |
|  | 4.663297227386376 | 1 |  | -j |  | J | -1 |  | j |  |
|  | 4.663297227386384 | 1 |  | j |  | -j | -1 |  | -j |  |
| 9 | C(1)=[1 1 1], C(2)=[-1 -1 1], C(3)=[1 1 -1], C(4)=[1 1 -1], C(5)=[1 -1 1] |  |  |  |  |  |  |  |  |  |
|  | 4.572511422830747 | 1 |  | 1 |  | 1 | 1 |  | 1 |  |
|  | 4.553085035023059 | 1 |  | j |  | -j | -1 |  | -j | Min |
|  | 4.553085035023059 | 1 |  | -j |  | J | -1 |  | j |  |
|  | 4.671788847148256 | 1 |  | -j |  | 1 | -1 |  | j |  |
|  | 4.671788847148260 | 1 |  | j |  | 1 | -1 |  | -j |  |
|  | 4.687636223708376 | 1 |  | j |  | J | -1 |  | -j |  |
|  | 4.687636223708378 | 1 |  | -j |  | -j | -1 |  | j |  |
| 10 | C(1)=[-1 1 -1], C(2)=[-1 1 1], C(3)=[1 1 -1], C(4)=[-1 1 1], C(5)=[-1 -1 -1] |  |  |  |  |  |  |  |  |  |
|  | 4.281044748561849 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 4.300948265176404 | 1 |  | -j |  | -1 | -1 |  | j |  |
|  | 4.300948265176406 | 1 |  | j |  | -1 | -1 |  | -j |  |
|  | 4.420202412753502 | 1 |  | j |  | -j | -1 |  | 1 |  |
|  | 4.420202412753503 | 1 |  | -j |  | J | -1 |  | 1 |  |
|  | 4.476771718813565 | 1 |  | -1 |  | J | 1 |  | -j |  |
|  | 4.476771718813569 | 1 |  | -1 |  | -j | 1 |  | j |  |
| 11 | C(1)=[1 -1 -1], C(2)=[1 1 1], C(3)=[1 1 -1], C(4)=[1 1 -1], C(5)=[1 -1 1] |  |  |  |  |  |  |  |  |  |
|  | 4.647705658221341 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 4.824305240263763 | 1 |  | j |  | J | -1 |  | -j |  |
|  | 4.824305240263769 | 1 |  | -j |  | -j | -1 |  | j |  |
|  | 4.986825236318058 | 1 |  | -1 |  | -1 | j |  | -j |  |
|  | 4.986825236318067 | 1 |  | -1 |  | -1 | -j |  | j |  |
|  | 5.832592595029739 | 1 |  | -1 |  | J | j |  | -j |  |
|  | 5.832592595029745 | 1 |  | -1 |  | -j | -j |  | j |  |
| 12 | C(1)=[-1 1 -1], C(2)=[-1 -1 -1], C(3)=[1 1 -1], C(4)=[1 1 1], C(5)=[-1 1 1] |  |  |  |  |  |  |  |  |  |
|  | 5.126299241805716 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 5.323629269162233 | 1 |  | j |  | -1 | -j |  | j |  |
|  | 5.323629269162236 | 1 |  | -j |  | -1 | j |  | -j |  |
|  | 5.363543368275654 | 1 |  | j |  | 1 | -1 |  | -j |  |
|  | 5.363543368275659 | 1 |  | -j |  | 1 | -1 |  | j |  |
|  | 5.468248625302861 | 1 |  | -1 |  | J | -j |  | 1 |  |
|  | 5.468248625302862 | 1 |  | -1 |  | -j | j |  | 1 |  |
| 13 | C(1)=[1 -1 1], C(2)=[1 -1 1], C(3)=[1 1 -1], C(4)=[1 1 1], C(5)=[-1 -1 -1] |  |  |  |  |  |  |  |  |  |
|  | 5.114928592097466 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 5.209908394646481 | 1 |  | -j |  | -1 | j |  | 1 |  |
|  | 5.209908394646482 | 1 |  | j |  | -1 | -j |  | 1 |  |
|  | 5.234679739557707 | 1 |  | -1 |  | 1 | -j |  | j |  |
|  | 5.234679739557713 | 1 |  | -1 |  | 1 | J |  | -j |  |
|  | 5.315475127838651 | 1 |  | -j |  | J | -j |  | -1 |  |
|  | 5.315475127838654 | 1 |  | j |  | -j | J |  | -1 |  |
| 14 | C(1)=[-1 1 -1], C(2)=[-1 1 -1], C(3)=[1 1 -1], C(4)=[1 -1 -1], C(5)=[-1 -1 -1] |  |  |  |  |  |  |  |  |  |
|  | 4.856054015866970 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 5.216648593581611 | 1 |  | j |  | -j | -1 |  | -j |  |
|  | 5.216648593581612 | 1 |  | -j |  | J | -1 |  | j |  |
|  | 5.507879035556247 | 1 |  | j |  | J | -1 |  | -j |  |
|  | 5.507879035556247 | 1 |  | -j |  | -j | -1 |  | j |  |
|  | 5.559345404211627 | 1 |  | j |  | -1 | -1 |  | -j |  |
|  | 5.559345404211631 | 1 |  | -j |  | -1 | -1 |  | j |  |
| 15 | C(1)=[1 -1 1], C(2)=[1 -1 -1], C(3)=[1 1 -1], C(4)=[1 -1 -1], C(5)=[-1 -1 -1] |  |  |  |  |  |  |  |  |  |
|  | 4.428907398534808 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 4.586159282955573 | 1 |  | j |  | -j | -1 |  | -j |  |
|  | 4.586159282955576 | 1 |  | -j |  | J | -1 |  | j |  |
|  | 4.646011906059639 | 1 |  | -j |  | -j | -1 |  | j |  |
|  | 4.646011906059639 | 1 |  | j |  | J | -1 |  | -j |  |
|  | 4.680275704726993 | 1 |  | -j |  | 1 | -1 |  | j |  |
|  | 4.680275704726999 | 1 |  | j |  | 1 | -1 |  | -j |  |
| 16 | C(1)=[-1 1 -1], C(2)=[-1 1 1], C(3)=[1 1 -1], C(4)=[-1 1 1], C(5)=[1 1 1] |  |  |  |  |  |  |  |  |  |
|  | 4.363268518430906 | 1 |  | 1 |  | 1 | 1 |  | 1 | Min |
|  | 4.517234522148936 | 1 |  | j |  | J | -1 |  | -j |  |
|  | 4.517234522148942 | 1 |  | -j |  | -j | -1 |  | j |  |
|  | 4.607301847771376 | 1 |  | j |  | -1 | -1 |  | -j |  |
|  | 4.607301847771379 | 1 |  | -j |  | -1 | -1 |  | j |  |
|  | 4.637573505602798 | 1 |  | -j |  | 1 | -1 |  | j |  |
|  | 4.637573505602806 | 1 |  | j |  | 1 | -1 |  | -j |  |

Table zzz shows the carried information of PA-Preamble. Bandwidth, ABS type and carrier type are informed by PA-Preamble.

Table zzz─ Carried Information by the associated PA-Preamble

| Index | Carrier | ABS type | BW |
| --- | --- | --- | --- |
| 0 | Fully configured | ABS included in NBR_ADV | 5 MHz |
| 1 |  |  | 10 MHz |
| 2 |  |  | 20 MHz |
| 3 |  | ABS not included in NBR_ADV | 5 MHz |
| 4 |  |  | 10 MHz |
| 5 |  |  | 20 MHz |
| 6 |  | N/A | N/A |
| 7 |  | N/A | N/A |
| 8 |  | N/A | N/A |
| 9 |  | N/A | N/A |
| 10 |  | N/A | N/A |
| 11 |  | N/A | N/A |
| 12 |  | N/A | N/A |
| 13 |  | N/A | N/A |
| 14 |  | N/A | N/A |
| 15 |  | CSG femto BS | N/A |
| 16 | Partially configured | N/A | N/A |

Macro BS and Relay BS are included in NBR_ADV.

----------------------------------------------------Text End------------------------------------------