| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Broadcast Paging MAC management message format (AWD-15.2.17.2.1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. Anshuman Nigam Samsung India Software Operation Baowei Ji Samsung Telecommunication America | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | 802.16m AWD: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | The contribution proposes AAI_PAG-ADV MAC management message format for idle mode operation. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Broadcast Paging MAC Management Message Format

Hyunjeong Kang, Jungje Son, Rakesh Taori, Anshuman Nigam, Baowei Ji

Samsung Electronics Co., Ltd.

- Introduction

An AMS in idle mode shall monitor a broadcast paging message that is transmitted in the paging listening interval. Subclause 15.2.17.2.1 of the current AWD describes the information of the broadcast paging message but the message format should be defined in section 15.2.5 AAI MAC management messages.

This contribution proposes a format of AAI_PAG-ADV message as broadcast paging message that is transmitted during paging listening interval.

-------------------------------  Text Start  ---------------------------------------------------

[Insert a new subclause 15.2.5.x at line 65 of page 24 after subclause 15.2.5.7 CLC_Info:]

15.2.5.x AAI_PAG-ADV Management Message

Table x – AAI_PAG-ADV message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_PAG-ADV_Message_Format(){ |  |  |
| Management Message Type | TBD | //AAI_PAG-ADV message |
| Num_AMS | TBD | Number of AMS Identifier |
| For(i=0 ; i++ ; i<Num_AMS){ |  |  |
| AMS temporary identifier | TBD | Temporary Identifier of the AMS |
| Action code | 2 | Paging action instruction to AMS 0b00: Perform network re-entry 0b01: Perform ranging to establish location 0b10: Perform LBS measurement 0b11: reserved |
| } |  |  |
| } |  |  |

An ABS shall generate AAI_PAG-ADV message including the following parameters.

AMS temporary identifier

An identifier that is assigned during idle mode entry or location update

Action code

Paging action instruction to the AMS to perform the following action:

0b00: Enter network

0b01: Perform ranging to establish location and acknowledge message

0b10: Perform LBS measurement

0b11: Reserved

The following parameter may be included in the AAI_PAG-ADV message when the ABS cannot transmit the entire paging message in a predetermined paging transmission frame.

Extension Flag

Indicate that the remaining part of the AAI_PAG-ADV message is transmitted in the subsequent subframe.

-------------------------------  Text End  ---------------------------------------------------