| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of broadcast paging message transaction during paging listening interval (AWD-15.2.17.2.3) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. Anshuman Nigam Samsung India Software Operation Baowei Ji Samsung Telecommunication America | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | 802.16m AWD: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | The contribution proposes the text to clarify the broadcast paging message transaction during paging listening interval of idle mode operation section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of Broadcast paging message transaction during Paging listening interval

Hyunjeong Kang, Yeongmoon Son, Jungje Son, Rakesh Taori, Anshuman Nigam, Baowei Ji

Samsung Electronics Co., Ltd.

- Introduction

Current AWD says that an idle mode AMS shall monitor broadcast paging message at a predetermined frame during paging listening interval but it has not defined how to the paging message.

Therefore we propose a scheme to decode the paging message at a predetermined frame by using Broadcast A-MAP IE [1] to indicate the encoding rule of the paging message.

During paging listening interval, the AMS shall monitor the Broadcast A-MAP IE to find whether any broadcast A-MAP IE indicates the existence of a paging message. The existence of broadcast A-MAP IE for paging message is identified by Type of the IE.

In Broadcast A-MAP IE, one value of Type field is used to indicate the paging message as below.

Table 1—Broadcast A-MAP IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast A-MAP IE_format() { | — | — |
| Type | 4 | This indicates the type of broadcast management message 0b0000 : AAI_TRF-IND message 0b0001 : AAI_PAG-ADV message 0b0010 : PGID Info 0b0011 ~ 0b1111 : Reserved |
| Length | 12 | This indicates the length of broadcast management message |
| } | — | — |

If there is a broadcast A-MAP IE of paging message, the AMS shall further decode the paging message that follows the A-MAP region. The paging message is transmitted on the same MCS level (i.e., robust MCS) as non-user specific A-MAP. If there are more than one broadcast A-MAP IEs, then the order of broadcast message follows the order of corresponding broadcast A-MAP IE.

Reference

[1] C80216m-09_1277 Proposal of the Broadcast A-MAP IE for Concurrent transmission of the Broadcast management messages

-------------------------------  Text Start  ---------------------------------------------------

15.2.17.2.3 Operation during paging listening interval

[Add the following texts after the 1st sentence ‘The AMS shall monitor pre-determined frame for paging message.’ in line 43 of page 101 in subclause 15.2.17.2.3:]

From the first subframe in the pre-determined frame, the AMS decodes non-user specific A-MAP to find whether any broadcast A-MAP IE is transmitted in the subframe. If any broadcast A-MAP IE exists in the subframe, the AMS shall decode a broadcast A-MAP IE with a type corresponding to AAI_PAG-ADV message. The broadcast A-MAP IE of the AAI_PAG-ADV message indicates the size of AAI_PAG-ADV message. After decoding the broadcast A-MAP IE of the AAI_PAG-ADV message, the AMS shall receive the AAI_PAG-ADV message that follows the A-MAP.

-------------------------------  Text End  ---------------------------------------------------