| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A method to transmit PGID information in idle mode (AWD-15.2.17.2.3) |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. Anshuman Nigam Samsung India Software Operation Baowei Ji Samsung Telecommunication America | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | 802.16m AWD: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | The contribution proposes the text for the location of paging group ID to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A method to transmit PGID information in idle mode

Hyunjeong Kang, Yeongmoon Son, Jungje Son, Rakesh Taori, Anshuman Nigam, Baowei Ji

Samsung Electronics Co., Ltd.

- Introduction

In current AWD, the location of PGID is not clearly defined because some parts of AWD describes that the location of PGID is TBD but some parts of AWD have defined that the PGID is transmitted in SFH SP5 IE.

Assuming that SFH will be used as a container for the PGID, then the PGID information may not be carried in the SFH based on the capacity of SFH. Therefore, it is required to define a scheme to transmit the PGID information.

Out of SFH, we can consider some container applied with predetermined encoding rule such as non-user specific A-MAP. But the non-user specific A-MAP does not have enough room to carry the PGID information, where the length of PGID is 16bits. In addition, the size of PGID information is not fixed among the ABSs because the number of paging groups assigned to an ABS can be different.

Therefore, we propose to apply the unified Broadcast A-MAP IE [1] to transmit the PGID information.

In Broadcast A-MAP IE, one value of Type field is used to indicate the PGID Information as below.

Table 1—Broadcast A-MAP IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Broadcast A-MAP IE_format() { | — | — |
| Type | 4 | This indicates the type of broadcast management message 0b0000 : AAI_TRF-IND message 0b0001 : AAI_PAG-ADV message 0b0010 : PGID Info 0b0011 ~ 0b1111 : Reserved |
| Length | 12 | This indicates the length of broadcast management message |
| } | — | — |

When the Broadcast A-MAP IE indicates a transmission of PGID information, then the PGID information is sent right after the A-MAP region. The PGID information shall exists before broadcast paging messages if any in a superframe corresponding to a paging listening interval.

Figure 1 presents a subframe structure where the PGID information is carried.

Figure 1 A subframe structure where the PGID Info is transmitted

Reference

[1] C80216m-09_1277 Proposal of the Broadcast A-MAP IE for Concurrent transmission of the Broadcast management messages

-------------------------------  Text Start  ---------------------------------------------------

15.2.17.2.3 Operation during paging listening interval

[Change the text from line 57 of page 100 to line 37 of page 101 as follows:]

The ABS shall transmit the PGID InfoPG ID information at a predetermined location in the paging listening interval in order to advertise the paging group(s) that is supported by the ABS. The PGID Infoinformation shall be transmitted by the ABS regardless of whether or not there any notifications for AMSs. The location of PGID(s) is TBD.

The ABS transmits the PGID Info paging information right after SFH and A-MAP of the 1st subframe in the S-SFH SP5 during AMS's paging listening interval as shown in Figure 421—.

The PGID Info shall be presented before any AAI_PAG-ADV message in the superframe. The PGID Info S-SFH SP5 includes the PGID(s)[s]  that ABS belongs to and may also include the whether or not paging indicator is used using the Paging indicator usage flag. When paging indicator usage flag is set to 1, ABS transmits associated [paging indicator flag] in the S-SFH SP5 indicating the presence of full paging messages for the corresponding PGIDs.

Figure 421—Transmission of PGID Info information

The PGID Info format is presented in Table xxx.

Table xxx PGID Info format

| Name | Value |
| --- | --- |
| Num_PGIDs | Number of paging groups in the ABS |
| PGID | Identifier of paging group (0~216) |

The AMS shall determine whether it exists in the same paging group at the preferred ABS as it has most recently belonged using the PGID(s) Info information.

If the AMS determine that its paging group has changed, the AMS shall perform idle mode location update as described in section 15.2.x.4.1.1 15.2.17.4.1.1.

-------------------------------  Text End  ---------------------------------------------------