| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of paging group location update in idle mode (AWD-15.2.17.4.1.1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. Anshuman Nigam Samsung India Software Operation Baowei Ji Samsung Telecommunication America | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | 802.16m AWD: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | The contribution proposes the text for clarifying paging group feature of idle mode section included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of paging group location update in idle mode

Hyunjeong Kang, Jungje Son, Rakesh Taori, Anshuman Nigam, Baowei Ji

Samsung Electronics Co., Ltd.

- Introduction

A paging group location update timer (PG_LU_Timer) is introduced in the idle mode section, which allows for an AMS not to perform paging group based location update even though the AMS leaves its paging group. But the AMS operation applying PG_LU_Timer such as the handling of the expiration of PG_LU_Timer is not clearly described.

This contribution proposes some clarification on paging group based update operation in idle mode.

-------------------------------  Text Start  ---------------------------------------------------

[Replace subclause 15.2.17.4.1.1 at line 22 of page 102:]

15.2.17.4.1.1 Paging group based update

An AMS shall perform Location Update process when an AMS detects a change in the paging groups. The AMS shall detect the change of the paging groups by monitoring the PG IDs, which are transmitted in the PGID info by the preferred ABS during the paging listening interval. If none of the PG ID(s), to which the AMS belongs, is detected, the AMS shall determine that the paging group(s) has changed.

ABSs and Idle Mode AMSs may belong to multiple paging groups. In case an AMS belongs to multiple paging groups and a Paging Group Location Update Timer is given, it starts Paging Group Location Update Timer (PG_LU_TIMER) when it leaves primary paging group. An AMS performs the paging group location update when after PG_LU_TIMER expires and the AMS has not returned to the primary paging group. and may inform its mobility (slow, medium, fast) to ABS. Based on the AMS mobility information, the ABS may assign new paging group(s) of different size(s) to AMS.

If the AMS returns to the primary paging group before the expiration of PG_LU_TIMER, it releases the timer and does not perform location update.

-------------------------------  Text End  ---------------------------------------------------