| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized AWD Text on the Unicast Control Information Elements for Group Resource Allocation |  |
| Date Submitted | 2009-07-13 |  |
| Source(s) | Shweta Shrivastava, Rath Vannithamby Intel Corporation Jason Junsung Lim, Seho Kim Samsung Electronics Kevin Power Fujitsu Wei Xia, Yangfeng Guan, Ping Qiu, Xianming Chen, Ying Liu, Huiying Fang ZTE Corporation Youngsoo Yuk, Jeongki Kim LG Electronics | Shweta.shrivastava@intel.com Junsung.lim@samsung.com Kevin.power@uk.fujitsu.com guan.yanfeng@zte.com.cn sixs@lge.com |
| Re: | Category: AWD comments/Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6.4.2.4 Group Resource Allocation A-MAP IE |  |
| Abstract | Proposed text for Unicast Control Information Elements for Group Resource Allocation |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized AWD Text on the Unicast Control Information Elements

for Group Resource Allocation

Shweta Shrivastava, Rath Vannithamby

Intel Corporation

Jason Junsung Lim, Seho Kim

Samsung Electronics

Kevin Power

Fujitsu

Wei Xia, Yangfeng Guan, Ping Qiu, Xianming Chen, Ying Liu, Huiying Fang

ZTE Corporation

Youngsoo Yuk, Jeongki Kim

LG Electronics

- Introduction

This contribution proposes text to be included in the section 15.3.6.4.2 Unicast Control Information Elements of the 802.16m amendment working document (AWD).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

15.3.6.5.2.4  Group Resource Allocation A-MAP IE

Group control information is used to allocate resources and/or configure resources to one or multiple AMSs within a group. The group resource allocation A-MAP IE is shown in Table 671.

Group scheduling requires two operations

- Assignment of a AMS to a group. In order to add a AMS to a group in the DL or UL, the ABS shall transmit a Group Configuration A-MAP IE

- Allocation of resources to AMSs within a group. In order to assign resources to one or more AMSs in a group, the ABS shall transmit the DL/UL Group Resource Allocation A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE is included in user-specific resource assignment in an A-MAP region. The GRA A-MAP IE contains bitmaps to indicate scheduled AMSs and signal resource assignment, nominal MCS, MIMO mode and resource size.

Table 671–DL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 3 | DL Group Resource Allocation A-MAP IE DL Extended IE |
| Extended IE Type | [3] | DL Group Resource Allocation A-MAP IE |
| Long TTI Length | 1 | Defines number of subframes spanned by the allocated resource. 0: 1 subframe (default) 1: 4 DL/UL subframes for FDD or all DL/UL subframe for TDD |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACK Channel Offset | TBD | Indicates the start of ACK index used for scheduled allocations and de-allocations at this subframe in the group. The ACK channels for scheduled allocations occur first, in the order of user bitmap, followed by the ACK channel for de-allocations, followed by the ACK channels for shifted AMSs. |
| NDA | [2][3] | Indicates the number of deleted AMSs in the group. |
| For(i=0,i++,i<NDA){ |  |  |
| User Bitmap Index | [5] | Indicates the User Bitmap Index of deleted AMSs. |
| } |  |  |
| User Bitmap Size | [2][5] | Indicates the length of User Bitmap |
| User Bitmap | Variable | Bitmap to indicate scheduled AMSs in a group. The size of the bitmap is equal to the User Bitmap Size. 0: AMS not allocated in this subframe 1: AMS allocated in this frame |
| N_deallocated_AMS | 2 | Number of deallocated AMSs |
| For(i=0;i<N_Deallocated_AMS; i++) { |  |  |
| De-allocated AMS index | Variable | Signals the indices of de-allocated AMS among the inactive AMSs in the user bitmap. The length of De-allocated AMS Index field determined as Ceil{log2(Number of Inactive AMSs in User Bitmap)}. |
| } |  |  |
| Re-arrange Indicator | 1 | Signals whether the re-arrange bitmap is present in this GRA IE |
| If (Re-arrange Indicator == 1) { |  |  |
| Re-arrange Bitmap | Variable | This bitmap signals the occupied bits in the user bitmap. The size of the bitmap is equal to the number of 0’s in the user bitmap. |
| } |  |  |
| If( Group MIMO mode set ==0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. |
| } |  |  |
| If( Group MIMO mode set == 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCSburst size/resource size for each scheduled AMS |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC masked by the Group ID |

Group configuration A-MAP IE: The group configuration A-MAP IE is used for initiating and maintaining a group for resource assignment.

Table 672–UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 3 | UL Group Resource Allocation A-MAP IE UL Extended IE |
| Extended IE Type | [3] | UL Group Resource Allocation A-MAP IE |
| Long TTI Length | 1 | Defines number of subframes spanned by the allocated resource. 0: 1 subframe (default) 1: 4 DL/UL subframes for FDD or all DL/UL subframe for TDD |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACK Channel Offset | TBD | Indicates the start of ACK index used for scheduled allocations at this subframe in the group. |
| NDA | [2][3] | Indicates the number of deleted AMSs in the group. |
| For(i=0,i++,i<NDA){ |  |  |
| User Bitmap Index | [5] | Indicates the User Bitmap Index of deleted AMSs. |
| } |  |  |
| User Bitmap Size | [2][5] | Indicates the length of User Bitmap |
| User Bitmap | Variable | Bitmap to indicate scheduled AMSs in a group. The size of the bitmap is equal to the User Bitmap Size. 0: AMS not allocated in this subframe 1: AMS allocated in this frame |
| N_deallocated_AMS | 2 | Number of deallocated AMSs |
| For(i=0;i<N_Deallocated_AMS; i++) { |  |  |
| De-allocated AMS index | Variable | Signals the indices of de-allocated AMS among the inactive AMSs in the user bitmap. The length of De-allocated AMS Index field determined as Ceil{log2(Number of Inactive AMSs in User Bitmap)}. |
| } |  |  |
| Re-arrange Indicator | 1 | Signals whether the re-arrange bitmap is present in this GRA IE |
| If (Re-arrange Indicator == 1) { |  |  |
| Re-arrange Bitmap | Variable | This bitmap signals the occupied bits in the user bitmap. The size of the bitmap is equal to the number of 0’s in the user bitmap. |
| } |  |  |
| If (N_deallocated_AMS != 0b00 or Re-arrange Indicator == 1) { |  |  |
| HF Index | TBD | Starting HF Index for deallocated AMSs and re-arranged AMSs, if present. |
| } |  |  |
| If( Group MIMO mode set == 0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. |
| } |  |  |
| If( Group MIMO mode set == 0b10 or 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCSburst size/resource size for each scheduled AMS |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC masked by the Group ID |

15.3.6.5.2.5  Group Configuration A-MAP IE

The unicast group configuration A-MAP IE is used to assign an AMS to a groupfor initiating and maintaining a group for resource assignment. The broadcast group configuration A-MAP IE is TBD.

Table 673–DL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 3 | DL Group Configuration A-MAP IE DL Extended IE |
| Extended IE Type | [3] | DL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index identifier |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| Lowest MCS in group | 4 | The index of the lowest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Highest MCS in group | 4 | The index of the highest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (Frequency first allocation) 0b1: 4 DL subframe for FDD or all DL frame subframe for TDD (Time first allocation) |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: CL SU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| if (Nsubframe, A-MAP ==2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | TBD[4] | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD[3] | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC masked with the Station ID |

Table 674–UL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 3 | DL Group Configuration A-MAP IE UL Extended IE |
| Extended IE Type | [3] | UL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index identifier. |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| Lowest MCS in group | 4 | The index of the lowest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Highest MCS in group | 4 | The index of the highest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (Frequency first allocation) 0b1: 4 UL subframe for FDD or all UL subframe for TDD (Time first allocation) |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: OL MU-MIMO 0b11: CL SU-MIMO and CL MU-MIMOreserved |
| if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | TBD[4] | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD[3] | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC masked with the Station ID |

-------------------------------  Text End  ---------------------------------------------------