| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD Editorial Comments on MAC HO Procedures (AWD-15.2.6) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Kelvin Chou MediaTek Inc. | Kelvin.Chou@mediatek.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Comment on the 802.16m amendment working document |  |
| Abstract | AWD editorial comment on Section 15.2.6 |  |
| Purpose | To be adopted by TGm for the 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Editorial Comments on MAC HO Procedures (AWD-15.2.6)

Kelvin Chou

MediaTek Inc.

Introduction

In the latest AWD, there are some erroneous implementations of the contributions C802.16m-09/0913r2, C802.16m-09/0914r1 and C802.16m-09/0915r1 from Handover DG approved by TGM in Session #61. Furthermore, some more editorial comments are also included.

Proposed AWD Text Changes

On the 16m AWD 80216m-09_0010r2, page 25, make the following changes to subsection 15.2.6:
--------------------------------------------------  Text Start  -----------------------------------------------------------------

At page 25, line 5

15.2.6 MAC HO procedures

This subclause specifies the HO procedures for the AAI. An AMS/ABS shall perform HO using the procedures defined in 15.2.6

At page 27, line 61

In this case, all included triggers shall have the same Action code (as defined in Table 26).

At page 30, line 15

…the MS finds that the target BS is unreachable as defined in the 'target BS unreachable condition' in section 15.2.6.2.

At page 30, line 27

These error conditions are defined as triggers with action code 0x5, as specified in section 15.2.6.2

At page 31, line 60

The network re-entry procedure is depicted in Figure 392— (HO_Reentry_Mode = 0) and Figure 393 (HO_Reentry_Mode = 1) respectively.

At page 32, figure 392

Figure 392—Network re-entry procedure with HO_Reentry_Mode set to 0. Messages depicted with dotted lines are transmitted only in certain HO scenarios. The dash-line (optional) AAI_RNG-RSP carries time adjustment parameters, etc

At page 33, figure 393

Figure 393—Network re-entry procedure with HO_Reentry_Mode set to 1. Messages depicted with dotted lines are transmitted only in certain HO scenarios. The dash-line (optional) AAI_RNG-RSP carries time adjustment parameters, etc

At page 34, line 1

The AAI_RNG-REQ/RSP transaction for HO is shown in Figure 392.

At page 35, line 17

The detailed procedures for zone switch based handover and direct handover are described in 15.2.6.4.1.2.1 and 15.2.6.4.1.2.2 respectively.

At page 36, figure 394

Figure 394—Handover procedure from YBS to ABS: the T-ABS instructs AMS to switch zone through RNG-RSP with Zone switch TLV

At page 36, figure 394

Figure 395—Handover procedure from YBS to ABS: the T-ABS instructs AMS to switch zone after completion of network reentry at LZone

At Page 38, line 13

AMS context mapping from LZone to MZone is performed by the ABS per section 15.2.6.4.1.3

At page 38, line 43

Zone switch from LZone to MZone follows the procedure per section 15.2.6.4.1.2.

At page 39, line 8

The scanning procedure for an AMS served in MZone of an ABS follows the procedure defined in section 15.2.6.1.2

At page 39, line 24

The AMS or the ABS initiates and executes the handover using AAI_HO-REQ or AAI_HO-CMD per section 15.2.6 (intra-16m HO), if the selected target BS is a YBS.

at page 39, line 29

If the AMS information is required to be transferred to the target BS for handover optimization, the serving ABS shall map the AMS context to the format in WirelessMAN-OFDMA Reference System per section 15.2.6.4.2.3, and provide it to the target YBS over the backbone.

At page 39, line 34

Handover cancellation procedure is performed per section 15.2.6.3.6.

At page 41, figure 396

of

Figure 396—Other RAT information delivery

At page 43, figure 397

16m Access

Target-RAT Access

Information Repository

2. Target RAT information

1.AMS connected to 16m Access

3. Policy Request

4. Policy Response

5. Evaluate Target RAT(s) for HO

6. RAT(s) Information Request

7. RAT (s) Information Response

8. Negotiate scan/sleep interval with 16m ABS to perform Target-RAT(s) measurement

9. Turn on Target-RAT(s) radio, configure measurement reporting

10. Target-RAT Measurements

11. Target-RAT Selection and HO Decision

12. Handover Procedure

AMS

Figure 397—The target RAT discovery and selection procedure

------------------------------------------------------  Text End  ---------------------------------------------------------------