| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose text for the SFH IE (AWD – 15.3.6.5.1) |  |
| Date Submitted | 2009-07-0615 |  |
| Source(s) | Mihyun Lee, Hwasun Yoo, Hyunkyu Yu, Junsung Lim, Jaeweon Cho, Rakesh Taori, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. Kiseon Ryu, Jinsoo Choi, Heejung Cho, Jin Sam Kwak LG Electronics Yih-Shen Chen, Kelvin Chou, I-Kang Fu MediaTek Inc. Ming-Hung Tao, Ying-Chuan Hsiao ITRI Kiran Kuchi, J. Klutto Milleth CeWit Sassan Ahmadi, Shantidev Mohanty Intel | mihyun.mac.lee@samsung.com {ksryu, emptylie, hj77cho, samji}@lge.com Sassan.ahmadi@intel.com Shantidev.mohanty@intel.com |
| Re: | IEEE 802.16m-09/0028r1: “802.16m amendment working document” Target topic: AWD - 15.3.6.5.1 |  |
| Abstract | This contribution proposes HARQ parameters into S-SFH IE |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Propose text for the SFH-IE (AWD – 15.3.6.5.1)

Mihyun Lee, Hwasun Yoo, Hyunkyu Yu, Junsung Lim, Jaeweon Cho,

Rakesh Taori, Hokyu Choi, Heewon Kang
Samsung Electronics Co., Ltd.

Kiseon Ryu, Jinsoo Choi, Heejung Cho, Jin Sam Kwak

LG Electronics

Yih-Shen Chen, Kelvin Chou, I-Kang Fu

MediaTek Inc.

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Kiran Kuchi, J. Klutto Milleth

CeWit

Sassan Ahmadi, Shantidev Mohanty

Intel

- Introduction

This contribution includes the proposed text of S-SFH IE. Refer to AWD[1], SFH carries the essential system information. But, some parameters as the essential system information were missed. For example, HARQ parameters in section 15.2.14.2.1[1] should be specified for 16m system. During last HARQ DG discussion, most of the DG members preferred to configure the value of HARQ parameters by ABS. AMS should know HARQ parameters for HARQ protocol while sending or receiving MAC management message during Network Entry. So, I would like to add or modify the information contents of SFH IE.

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document” (AWD)

- Text proposal

[Suggested Remedy#1 Replace Table 663 as follows]

S-SFH SP1 IE contains information for network re-entry, see Table 656663.

Table 663 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |
| LSB of 48 bit BS MAC ID | 12 | Specifies the 12 least bit of BS ID |
| UL Ack/Nack channel resource size | 2 |  |
| DL Ack/Nack channel resource size | 2 |  |
| Power control channel resource size indicator | 2 |  |
| Non-user specific AMAP location | 1 | Reuse 1 or reuse 3 |
| A-A-MAP MCS selection | 1 |  |
| DL permutation configuration (CRU, DRU partitioning and signaling related to that) | 13 | For 20MHz, DL_CAS_SB0(4), DL_CAS_MB0(6), DL CAS_SBi (4x3) |
| UL permutation configuration (CRU, DRU partitioning and signaling related to that) | 13 | For 20 MHz, UL_CAS_SB0(4), UL_CAS_MB0(6), UL_CAS_SBi (4x3) |
| Unsync ranging allocation interval channel information (ranging region periodicity) | 3 |  |
| Unsync ranging location in the frame (time and frequency) | 2 |  |
| RNG codes information | 7 |  |
| Ranging code subset/partition configuration | 3 |  |
| BS EIRP | 7 |  |
| Cell bar information | 1 |  |
| } |  |  |

[Suggested Remedy#2 Replace Table 664 as follows]

Table 664 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Duplexing mode | 1 | 0b0: TDD 0b1: FDD |
| Sub-frame configuration (DL/UL ratio, duplexing mode) | 7 |  |
| If (Duplexing mode == FDD) { |  |  |
| UL carrier frequency offset | 6 |  |
| UL bandwidth | 3 |  |
| } |  |  |
| MSB bytes of BSID | 36 | Specifies 36 MSB of BSID |
| MAC protocol revision | 4 | Version number of IEEE 802.16m supported on this channel |
| FFR partitioning info for DL region | 12 | For 20MHz, DL_SAC(5), DL_FPSC(3), DL_FPC(4) For 5MHz, DL_SAC(3), DL_FPSC(1), DL_FPC(3) |
| FFR partitioning info for UL region | 12 | For 20MHz, UL_SAC(5), UL_FPSC(3), UL_FPC(4) For 5MHz, UL_SAC(3), UL_FPSC(1), UL_FPC(3) |
| MS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed MS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| (EIRPIRmin) | 5 |  |
| reserved |  |  |
| } |  |  |

[Suggested Remedy#3 Replace Table 665 as follows]

Table 665 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| UL Fast FB Size | 4 | Specifies the size of UL feedback channel per a UL subframe; (Need the decision from UL Ctrl section) |
| # Tx antenna | 2 | 0b00: 2 antennas 0b01: 4 antennas 0b10: 8 antennas 0b11: reserved |
| Default RSSI and CINR averaging parameter | 3 | Default averaging parameter for physical CINR measurements and RSSI measurements, in multiple of 1/16 |
| SP scheduling periodicity information | TBD |  |
| HO Ranging backoff start | 4 |  |
| HO Ranging backoff end | 4 |  |
| Initial ranging backoff start | 4 |  |
| Initial ranging backoff end | 4 |  |
| UL BWREQ Channel information | 3 |  |
| Bandwidth request backoff start | 4 |  |
| Bandwidth request backoff end | 4 |  |
| Uplink subframe bitmap for sounding | 8 |  |
| Sounding Multiplexing Type (SMT) for sounding | 1 |  |
| Decimation value D/Max Cyclic Shift Index P for sounding | 3 |  |
| Reserved | TBD |  |
| } |  |  |

--------------------------------  Text End  ---------------------------------------------------