| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Message Formats for Sleep Mode Operation (AWD-15.2.16.3) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Yih-Shen Chen, Kelvin Chou, I-Kang Fu MediaTek Inc. Ming-Hung Tao, Ying-Chuan Hsiao ITRI Zexian Li Nokia Shantidev Mohanty Intel | yihshen.chen@mediatek.com, kelvin.chou@mediatek.com MHTao@itri.org.tw zexian.li@nokia.com Shantidev.mohanty@intel.com |
| Re: | “802.16m amendment working document”: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content”. |  |
| Abstract | This contribution proposes the message formats for AAI_SLP-REQ/RSP |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Message Formats for Sleep Mode Operation

Yih-Shen Chen, Kelvin Chou, I-Kang Fu

MediaTek Inc.

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Zexian Li

Nokia

Shantidev Mohanty

Intel

Introduction

In Cairo meeting, basic operations of sleep mode are agreed and defined in Section 15.2.16.2.1 of AWD [1]. The sleep-cycle based operation concept is adopted in 802.16m and the associated parameters include: initial sleep cycle, final sleep cycle, default listening window and starting frame offset. And, the concept of variable listening window length (i.e., LWEF) and the optimization of MOB_TRF-IND transmission (i.e., TIMF) are also considered. In this contribution, we propose the message formats of MOB_SLP-REQ/RSP to facilitate the agreed sleep mode operation.

Proposed AWD Text Changes

Remedy 1

On the 16m AWD 80216m-09_0010r2, page94, line 36, make the following changes to subsection 15.2.16.3:
--------------------------------------------------  Text Start  -----------------------------------------------------------------

- 15.2.16.4 message formats of sleep mode operation

- 15.2.16.4.1 AAI_SLP-REQ

Table 1 MOB_SLP-REQ Message Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_SLP-REQ_Message_format() { | — | — |
| Management Message Type = XXX | 8 | — |
| Request_Code | 2 | 0b00 : Exit from Sleep Mode 0b01 : Activate a SCID 0b10 : Change to a SCID 0b11 : Re-activate previously defined sleep cycle pameteters |
| SCID | 6 | The ID of the sleep cycle setting set |
| if(Request_Code == 0b01 ¦¦ Request_Code == 0b10) { | — | — |
| Starting_frame_number | TBD | The number of the frame where the SCID is requested to start to take effect. |
| TIMF | 1 | 0: Traffic Indication Message (AAI_TRF_IND) is never sent 1: Traffic Indication Message is sent during listening window |
| Initial Sleep Cycle | 8 | — |
| Final Sleep Cycle | 10 | — |
| Default Listening Window | 6 | — |
| Listening sub-frame bitmap | 8 | The bitmap indicates the sub-frames where the AMS needs to remain awake |
| NISCF | 1 | New Initial sleep cycle indicator |
| If (NISCF==1) { |  |  |
| New Initial Sleep Cycle | TBD | - |
| } |  |  |
| Padding | variable | Padding bits to ensure byte aligned. |
| TLV encoded Information | variable |  |
| } |  |  |
| } | — | — |

- 15.2.16.4.1 AAI_SLP-RSP

Table 2 AAI_SLP-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_SLP-RSP_Message_format() { | — | — |
| Management Message Type = XXX+1 | 8 | — |
| Response_code |  | 0b00 : Request by BS in Unsolicited Manner 0b01 : Approval of AAI_SLP-REQ 0b10 : Rejection of AAI_SLP-REQ 0b11 : Reserved |
| if(Response_Code == 0b00 ¦¦ 0b01) { |  |  |
| Operation_code | 2 | 0b0 : Exit Sleep Mode 0b1 : Activate/ Change SCID, or re-activate previously defined SCID |
| If (Operation_code == 0b01¦¦ 0b10) { |  |  |
| SCID | 6 | The SCID to be activated. |
| Starting_frame_number | TBD | The number of the frame where the SCID should start to take effect. |
| TIMF | 1 | 0: Traffic Indication Message is never sent 1: Traffic Indication Message is sent during listening window |
| Initial Sleep Cycle | 8 | — |
| Final Sleep Cycle | 10 | — |
| Default Listening Window | 6 | — |
| Listening sub-frame bitmap | 8 | The bitmap indicates the sub-frames where the AMS needs to remain awake |
| NISCF | 1 | New Initial sleep cycle indicator |
| If (NISCF==1) { |  |  |
| New Initial Sleep Cycle | TBD | - |
| } |  |  |
| if(TRF-IND required == 1) { | — | — |
| SLPID | 10 | — |
| } | — | — |
| } |  |  |
| Padding | variable | Padding bits to ensure byte aligned. |
| TLV encoded Information | variable |  |
| } |  |  |
| } | — | — |

------------------------------------------------------  Text End  ---------------------------------------------------------------

Reference

[1] IEEE 802.16m-09/0010r2, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems (working document)”