| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD Text on the MAC Procedure for Group Resource Allocation |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Shweta Shrivastava, Rath Vannithamby Intel Corporation | Shweta.shrivastava@intel.com |
| Re: | Category: AWD comments/Area: Chapter 15.2.8 (Group Resource Allocation) “Comments on AWD 15.2.8 Group Resource Allocation” |  |
| Abstract | Proposed text for Group Resource Allocation MAC Procedure |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Text on the MAC Procedure for Group Resource Allocation

Shweta Shrivastava, Rath Vannithamby

Intel Corporation

- Introduction

This contribution proposes text to be included in the MAC procedure for Group Resource Allocation of the 802.16m amendment working document (AWD).

Support for rate matching was recently adopted in AWD. Hence there is no fixed set of MCS schemes. Thus, configuration of groups for group resource allocation based on MCS schemes is not reasonable. Hence this document proposes to remove the limitation of MCS sets for a group. This not only enables support for rate matching with group resource allocation, but also provides flexibility to the implementation to configure groups. Removing the limitation of MCS sets for grouping will also save overhead by reducing the number of groups and by reducing the number of group changes required for an AMS.

In this document, we propose changes to the MAC procedure to enable rate matching and configure groups for group resource allocation without the limitation of MCS sets for grouping.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

15.2.8	Group Resource Allocation

Group Resource Allocation mechanism allocates resources to multiple users as a group in order to save control overhead. The mechanism takes advantage of common traffic characteristics and grouping is done based on some common parameters such as MCS (modulation and coding scheme)MIMO mode, burst size and resource size, which further saves overhead.

15.2.8.1 Grouping Mechanism

AMSs are assigned to groups based on MIMO mode used and the combination of MCS usedburst size and the resource allocation size (number of LRUs) required. A set of n-bit codes can be used to represent the different combinations of MCSsburst sizes and resource sizes that are used by a group. These codes are included in a bitmap as part of the group’s resource allocation information.

15.2.8.2 Group Configuration

...

ABS configures a MCS Set for each group among the predefined candidate sets listed in Table aaa for both downlink and uplink. When an AMS is added into the group, the configured MCS Set ID is indicated through Group Configuration IE. The assigned MCS to AMS in the group shall be chosen from the configured set.

Table 30 – DL MIMO Mode Set Candidates

| ID | MCS Set |  |  |  |
| --- | --- | --- | --- | --- |
| 000 | 0000 | 0001 | 0010 | 0011 |
|  | 0100 | 0101 | 0110 | 0111 |
|  | 1000 | 1001 | 1010 | 1011 |
|  | 1100 | 1101 | 1110 | 1111 |
| 001 | 0000 | 0001 | 0010 | 0011 |
|  | 0100 | 0101 | 0110 | 0111 |
| 010 | 1000 | 1001 | 1010 | 1011 |
|  | 1100 | 1101 | 1110 | 1111 |
| 011 | 0000 | 0001 | 0010 | 0011 |
| 100 | 0100 | 0101 | 0110 | 0111 |
| 101 | 1000 | 1001 | 1010 | 1011 |
| 110 | 1100 | 1101 | 1110 | 1111 |

ABS configures a HARQ Burst Size Set for each group among the predetermined [4, TBD] HARQ Burst Size Set Candidates. Those candidates are signaled to AMSs via the S-SFH(TBD). When an AMS is added into the group, the configured HARQ Burst Size Set ID is indicated through Group Configuration IE. The assigned HARQ burst size to AMS in the group shall be chosen from the configured set.

As an optional, the combinations of MCS/MIMO mode/Resource size can be signaled explicitly by the ABS to the AMS (TBD).

15.2.8.3 Group Management

…

15.2.8.3.1.2 AMS Operation

Upon receiving Group Configuration A-MAP IE, the AMS receives the required information to interpret the assigned MIMO mode, MCS levelburst size and resource indexsize from the bitmaps in the corresponding Group Resource Allocation A-MAP IE. Once the AMS receives a Group Configuration A-MAP IE, the AMS shall monitor its allocation in the Group Resource Allocation A-MAP IE until it is deleted from the group.

15.2.8.3.2 Deletion of AMS from a Group

The ABS may delete an AMS from a group when one or more of the following conditions applies: (i) the connection is terminated (ii) the MIMO mode/MCS /HARQ burst size suitable for the AMS no longer belongs to the MIMO Mode Set/MCS set /HARQ burst size set corresponding to the group.

...

15.2.8.3.4.1 Bitmaps in Group Resource Allocation

...

The third bitmap is the Resource Allocation bitmap which uses n bits per AMS to signal MCSburst size and Resource Size for the scheduled AMS in the subframe or extended subframe that are scheduled in the frame. The scheduled AMSs may have different number of bits in the third bitmap when they are assigned different MIMO mode and SM parameter.

The bit length used for a scheduled AMS is determined by the total number of effective combinations for MCSburst size and resource size for the assigned MIMO mode with SM parameter. Effective combinations are derived by subtracting useless combinations among all possible combinations. The following steps are needed to find the effective combinations.

- Step 1: List all possible combination set of nominal MCSs and burst sizes for that groupC={ C(0,0), C(0,1), …, C(M,B) }

Table ddd. Combination indexes

| MCS/ HARQ data burst size | 1 | 2 | … | B (Highest burst size) |
| --- | --- | --- | --- | --- |
| 1 | C(1,1) | C(1,2) | … | C(1,B) |
| 2 | C(2,1) | C(2,2) | … | C(2,B) |
| … | … | … | … | … |
| M (Highest MCS) | C(M,1) | C(M,2) | … | C(M,B) |

C(m,b): Combination index for MCS m, HARQ data burst size b

- Step 2: For each HARQ data burst size, useless combination is chosen when it requires same resource size with a lower MCS level comparing to others due to the resource granularity. For b∈IB, m∈IM, n∈IM, and m>n,

{C(m,b)} U1 	if N(m,b) = N(n,b)

Where

U1 : Useless combination set type 1

IM : Group MCS setSet of nominal MCSs

IB : Group HARQ burst size set

N(m,b) : Required number of RUs for MCS m, HARQ data burst size b

- Step 3: For a given MCS, useless combination is chosen when it requires same resource size supporting a smaller HARQ data burst size than others due to the resource granularity. For m∈IM, b∈IB, d∈IB, and b>d,

{C(m,b)} U2 	if N(m,b) = N(m,d)

Where U2 is useless combination set type 2

- Step 43: Derive effective combination set (E) which is

E = C-U1-U2

Step 4: Repeat steps 1 to 3 for other MIMO ranks supported in the group

An index code is identified to each effective combination from lower MCS level and lower HARQ data burst size where n is determined by Ceil{log2(Total number of effective combinations)}.

...

-------------------------------  Text End  ---------------------------------------------------