| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD Text on the Unicast Control Information Elements for Group Resource Allocation |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Shweta Shrivastava, Rath Vannithamby Intel Corporation | Shweta.shrivastava@intel.com |
| Re: | Category: AWD comments/Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6.4.2.4 Group Resource Allocation A-MAP IE |  |
| Abstract | Proposed text for Unicast Control Information Elements for Group Resource Allocation |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Text on the Unicast Control Information Elements

for Group Resource Allocation

Shweta Shrivastava, Rath Vannithamby

Intel Corporation

- Introduction

This contribution proposes changes to the text the section 15.3.6.4.2 Unicast Control Information Elements of the 802.16m amendment working document (AWD).

Support for rate matching was recently adopted in AWD. Hence there is no fixed set of MCS schemes. Thus, configuration of groups for group resource allocation based on MCS schemes is not reasonable. Hence this document proposes to remove the limitation of MCS sets for a group. This not only enables support for rate matching with group resource allocation, but also provides flexibility to the implementation to configure groups. Removing the limitation of MCS sets for grouping will also save overhead by reducing the number of groups and by reducing the number of group changes required for an AMS.

In this document, we propose to remove the specification of MCS Set ID from the DL/UL Group Configuration IE, and also propose corresponding changes to other text in the section.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

15.3.6.5.2.4  Group Resource Allocation A-MAP IE

Group control information is used to allocate resources and/or configure resources to one or multiple AMSs within a group. The group resource allocation A-MAP IE is shown in Table 671.

Group scheduling requires two operations

- Assignment of a AMS to a group. In order to add a AMS to a group in the DL or UL, the ABS shall transmit a Group Configuration A-MAP IE

- Allocation of resources to AMSs within a group. In order to assign resources to one or more AMSs in a group, the ABS shall transmit the DL/UL Group Resource Allocation A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE is included in user-specific resource assignment in an A-MAP region. The GRA A-MAP IE contains bitmaps to indicate scheduled AMSs and signal resource assignment, MCS, MIMO mode, HARQ burst size and resource size.

Table 671–DL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| ... |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/HARQ burst size and resource size for each scheduled AMS |
| ... |  |  |

...

Table 672–UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| ... |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/HARQ burst size and resource size for each scheduled AMS |
| ... |  |  |

15.3.6.5.2.5  Group Configuration A-MAP IE

...

Table 673–DL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| ... |  |  |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| ... |  |  |

Table 674–UL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| ... |  |  |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| ... |  |  |

-------------------------------  Text End  ---------------------------------------------------