| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Extended DL and UL A-MAP Information Element Types in the 802.16m AWD (AWD- 15.3.6.5.2.2) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Yi Hsuan Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | Category: AWD comment / Section: (AWD – 15.3.6.5.2.2 ) |  |
| Abstract | This contribution proposes text related to Extended DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Extended DL and UL A-MAP Information Element Types in the 802.16m AWD (AWD- 15.3.6.5.2.2)

Roshni Srinivasan, Hujun Yin, Yi Hsuan

Intel Corporation

- Introduction

Currently 4 bits are allocated to signal the DL/UL assignment A-MAP IE type in the 802.16m AWD [1] in tables 668 and 670. Contribution C802.16m-09/1327 proposes the allocation of 3 bits to signal the type of assignment A-MAP IE, using which, two extended IE types for the DL and the UL can be signaled.

It is expected that Extended IEs will be required when the size of the IE is either larger or smaller than the basic IE. Extended IEs are further classified into multiple types, which are identified by the first field of size 4 bits in the extended IE. The length of the IE and the corresponding fields are derived from the extended IE type.

- IEs used for control signaling such as the feedback allocation IE, sounding allocation IE, group resource allocation and group configuration IEs are included in the extended IE category.

- Instructions to editor:

- The following changes have been included in the text proposal

- A new Table has been defined to enumerate the different extended IEs currently defined in the AWD. Insert the table following Table 669

Proposed AWD text has been underlined in red, AWD text that has been deleted has been struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

- Amendment text proposal for inclusion in 802.16m-09/0010r2

-------------------------------  Start Text Proposal ---------------------------------------------------

Table 670: Extended A-MAP IE Types

| Extended A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | HARQ Feedback A-MAP-IE |
| 0b0001 | Feedback Allocation A-MAP IE |
| 0b0010 | Power Control A-MAP IE |
| 0b0011 | UL Sounding Command A-MAP IE |
| 0b0100 | DL Group Configuration A-MAP IE |
| 0b0101 | UL Group Configuration A-MAP IE |
| 0b0110 | DL Group Resource Allocation A-MAP IE |
| 0b0111 | UL Group Resource Allocation A-MAP IE |
| 0b1000 | Reserved |
| 0b1001 | Reserved |
| 0b1010 | Reserved |
| 0b1011 | Reserved |
| 0b1100 | Reserved |
| 0b1101 | Reserved |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

-------------------------------  End Text Proposal ---------------------------------------------------