| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD text on Group Resource Allocation |  |
| Date Submitted | 2009-07-07 |  |
| Source(s) | Wei Xia, Yangfeng Guan, Ping Qiu, Xianming Chen, Ying Liu, Huiying Fang ZTE Corporation Ming-Hung Tao ITRI | E-mail: guan.yanfeng@zte.com.cn E-mail: MHTao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.2.8 (Group Resource allocation) and Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.2.8 Group Resource Allocation, 15.3.6.5.2.4 Group resource allocation A-MAP IE and 15.3.6.5.2.5 Group Configuration A-MAP IE” |  |
| Abstract | The contribution proposes text on control information and MAC operation for GRA in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD text on Group Resource Allocation

Wei Xia, Yanfeng Guan, Ping Qiu, Xianming Chen, Ying Liu, Huiying Fang

ZTE Corporation

Ming-Hung Tao

ITRI

# 1 Introduction

In current AWD，the following topics regarding group resource allocation are TBD.

- The broadcast group configuration A-MAP IE.

- The configured MCS set candidates in additional broadcast message.

- Error Handling Procedure.

This contribution proposed text on broadcast GC A-MAP IE, different types of MCS set candidates and error handing procedure for GRA. Please refer to the contribution [1] for detailed analysis.

# 2 References

[1] S80216m-09_1515. Broadcast Group Configuration A-MAP IE and MCS set candidates for Group Resource Allocation: Simulation Support, July, 2009.

[2] IEEE 80216m-09_0010r2. Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems. Advanced Air Interface(working document), June, 2009.

- Proposal Text #1 for inclusion in the 802.16m amendment

-------------------------------  Text # 1 Start  ---------------------------------------------------

15.2.8.2 Group Configuration

…

ABS configures a ISizeOffset Set for each group among the predefined candidate sets listed in Table aaa 30 for both downlink and uplink. When an AMS is added into the group, the configured ISizeOffset Set ID is indicated through Group Configuration IE. The assigned ISizeOffset  to AMS in the group shall be chosen from the configured set. For grouping flexibility and system efficiency, two adjacent ISizeOffset set candidates can have the same ISizeOffset or the ISizeOffset supportable of a group can be discrete.

Table 30 – DL MIMO mode ISizeOffset Set Candidates

| ID | ISizeOffset Set |  |  |  |
| --- | --- | --- | --- | --- |
| 000 | 00000 | 00001 | 00010 | 00011 |
|  | 00100 | 00101 | 00110 | 00111 |
|  | 01000 | 01001 | 01010 | 01011 |
|  | 01100 | 01101 | 01110 | 01111 |
|  | 10000 | 10001 | 10010 | 10011 |
|  | 10100 | 10101 | 10110 | 10111 |
|  | 11000 | 11001 | 11010 | 11011 |
|  | 11100 | 11101 | 11110 | 11111 |
| 001 | 00000 | 00001 | 00010 | 00011 |
|  | 00100 | 00101 | 00110 | 00111 |
|  | 01000 | 01001 |  |  |
| 010 | 01000 | 01001 | 01010 | 01011 |
|  | 01100 | 01101 | 01110 | 01111 |
|  | 10000 | 10001 |  |  |
| 011 | 10000 | 10001 | 10010 | 10011 |
|  | 10100 | 10101 | 10110 | 10111 |
|  | 11000 | 11001 |  |  |
| 100 | 10110 | 10111 | 11000 | 11001 |
|  | 11010 | 11011 | 11100 | 11101 |
|  | 11110 | 11111 |  |  |
| 101-111 | Reserved |  |  |  |

-------------------------------  Text #1 End  ---------------------------------------------------

- Proposal Text #2 for inclusion in the 802.16m amendment

-------------------------------  Text # 3 Start  ---------------------------------------------------

15.2.8.3.1 Addition of AMS to a Group

Addition of an AMS to a group occurs when group resource allocation is initialized for the AMS or when AMS in a group moves to other group. For inclusion, the group information shall be informed to AMS in order to interpret resource assignment information from Group Resource Allocation A-MAP IE. The information is transmitted through Group Configuration A-MAP IE. The Group Configuration A-MAP IE can be either unicast or broadcast. When there are multiple AMSs need to be added to a group simultaneously, the broadcast Group Configuration A-MAP IE can be used for informing multiple AMSs of the group configuration information. The details on broadcasting this IE are TBD.

-------------------------------  Text # 3 End  ---------------------------------------------------

- Proposal Text #3 for inclusion in the 802.16m amendment

-------------------------------  Text # 4 Start  ---------------------------------------------------

15.2.8.3.2 Deletion of AMS from a Group

The ABS may delete an AMS from a group when one or more of the following conditions applies: (i) the connection is terminated (ii) the MIMO mode/MCS/HARQ burst size suitable for the AMS no longer belongs to the MIMO Mode Set/MCS set/HARQ burst size set corresponding to the group.

If deletion information is specified in Group Resource Allocation A-MAP IE, the filed ‘ACK Channel Offset’ indicates the start of ACK index used for deleted AMSs. The start of ACK index used for scheduled allocations in the group is equal to the value of ‘ACK Channel Offset’ plus the number of deleted AMSs.

15.2.8.3.2.1 ABS Operation

ABS may delete multiple AMSs from a group in a subframe. The deletion is informed by listing the User Bitmap Index of deleted AMS in Group Resource Allocation A-MAP IE. The deletion shall apply from the subframe in which the deletion information is sent.

To confirm that the deleted AMS has successfully decoded the deletion information in the corresponding Group Resource Allocation A-MAP IE, the ABS shall assign the HARQ feedback channel to the deleted AMS. If no acknowledgement is detected on the assigned channel, the ABS shall assume that the AMS missed the deletion information.

After the ABS receives the ACK for deletion information from the deleted AMS, it can assign the User Bitmap index of deleted AMS to new AMS added to the group.

15.2.8.3.2.2 AMS Operation

Upon receiving the deletion information in the Group Resource Allocation A-MAP IE, the AMS shall check whether its User Bitmap index is specified. If it is deleted from the group, the AMS shall calculate its ACK channel index according to the strart of ACK index for deleted AMSs and the list of User Bitmap index, and it shall send ACK on the assigned HARQ feedback channel.

-------------------------------  Text # 4 End  ---------------------------------------------------

- Proposal Text #5 for inclusion in the 802.16m amendment

-------------------------------  Text # 5 Start  ---------------------------------------------------

15.2.8.4 Error Handling Procedure

In the case of addition, if the addition information is not received by the new AMS, it may result in resource wastage. The ABS shall assign a HARQ feedback channel for the added AMS to send the acknowledgement. The added AMS specified in Group Configuration IE shall feedback the reception of addition information. Absence of ACK is implicitly considered as an NACK by the ABS. The ABS starts group resource allocation for the new AMS only after receiving ACK from the AMS.

In the case of deletion, if the deletion information is not received by the deleted AMS, it may result in transmission collision. The ABS shall assign a HARQ feedback channel for the deleted AMS to send acknowledgement. The deleted AMS specified in Group Resource Allocation IE shall confirm the reception of deletion information. Absence of ACK is implicitly considered as an NACK by the ABS. The ABS assigns the User Bitmap index of deleted AMS to new AMS only after receiving ACK from the deleted AMS.

-------------------------------  Text # 5 End  ---------------------------------------------------

- Proposal Text #5 for inclusion in the 802.16m amendment

-------------------------------  Text # 5 Start  ---------------------------------------------------

15.3.6.5.2.4  Group Resource Allocation A-MAP IE

…

Table 671- DL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Resource Allocation A-MAP IE |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACK Channel Offset | TBD | For NDA == 0, this field Iindicates the start of ACK index used for scheduled allocations at this subframe in the group. For NDA > 0, this field indicates the start of ACK index used for deleted AMSs. The start of ACK index used for scheduled allocations in the group is equal to the value of ‘ACK Channel Offset’ plus NDA. |
| NDA | [2][3] | Indicates the number of deleted AMSs in the group. |
| For(i=0,i++,i<NDA){ |  |  |
| User Bitmap Index | [5] | Indicates the User Bitmap Index of deleted AMSs. |
| } |  |  |
| User Bitmap Size | [2][5] | Indicates the length of User Bitmap |
| User Bitmap | Variable | Bitmap to indicate scheduled AMSs in a group. The size of the bitmap is equal to the User Bitmap Size |
| If( Group MIMO mode set ==0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. |
| } |  |  |
| If( Group MIMO mode set == 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/resource size for each scheduled user AMS |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC |

Table 672-UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Resource Allocation A-MAP IE |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACK Channel Offset | TBD | For NDA == 0, this field Iindicates the start of ACK index used for scheduled allocations at this subframe in the group. For NDA > 0, this field indicates the start of ACK index used for deleted AMSs. The start of ACK index used for scheduled allocations in the group is equal to the value of ‘ACK Channel Offset’ plus NDA. |
| NDA | [2][3] | Indicates the number of deleted AMS in the group. |
| For(i=0,i++,i<NDA){ |  |  |
| User Bitmap Index | [5] | Indicates the User Bitmap Index of deleted AMSs. |
| } |  |  |
| User Bitmap Size | [2][5] | Size of the user bitmap; may not be needed if user bitmap size is included in configuration message/A-MAP IE |
| User Bitmap | Variable | Bitmap to indicate scheduled users in a group. The size of the bitmap is equal to the User Bitmap Size |
| If( Group MIMO mode set == 0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. |
| } |  |  |
| If( Group MIMO mode set == 0b10 or 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/resource size for each scheduled user |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC |

-------------------------------  Text # 5 End  ---------------------------------------------------

- Proposal Text #6 for inclusion in the 802.16m amendment

-------------------------------  Text # 6 Start  ---------------------------------------------------

15.3.6.5.2.5  Group Configuration A-MAP IE

The unicast group configuration A-MAP IE is used for initiating and maintaining a group for resource assignment. The broadcast group configuration A-MAP IE is TBD used for adding multiple AMSs to a group. The MCRC of unicast IE is masked with Station ID, while the MCRC of broadcast IE is masked with broadcast ID (TBD).

Table 673- DL Unicast Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index. |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (Frequency first allocation) 0b1: 4 DL subframe for FDD or all DL frame subframe for TDD (Time first allocation) |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: CL SU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| if (Nsubframe, A-MAP ==2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | TBD | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD | Indicates the number of ACIDs used for group resource allocation. |
| ACK channel Index | TBD | Indicates the ACK channel used for the AMS to send acknowledgement for the addition information. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC |

Table 674- UL Unicast Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index. |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (Frequency first allocation) 0b1: 4 UL subframe for FDD or all UL subframe for TDD (Time first allocation) |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: OL MU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| if (Nsubframe, A-MAP ==2){ |  |  |
| If(DL:UL !=3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| }else if (DL:UL !=3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | TBD | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD | Indicates the number of ACIDs used for group resource allocation. |
| ACK channel Index | TBD | Indicates the ACK channel used for the AMS to send acknowledgement for the addition information. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC |

Table xxx- DL Broadcast Group Configuration A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index. |
| Number of new AMSs | 2 | Indicates the number of new AMSs added into the group 0b00: 2 AMSs 0b01: 3 AMSs 0b10: 4 AMSs 0b11: 5 AMSs |
| User Bitmap index description indicator | 1 | Indicates whether each AMS’s User Bitmap index is described 0b0: Indicate that the User Bitmap index for the AMS associated with the first STID needs to be described, and the User Bitmap index for remaining AMSs are determined by the order of STIDs described. 0b1: Indicate that the User Bitmap index for every added AMS needs to be described. |
| For (i=0, i++, i< Number of new AMSs ) |  |  |
| { |  |  |
| Station ID | 12 |  |
| Initial ACID | 4 | Indicates the start of ACID used for group resource allocation. |
| N_ACID | 3 | Indicates the number of ACIDs used for group resource allocation. |
| if (User Bitmap index description indicator ==1) && (i !=1){ |  |  |
| User Bitmap Index | 5 | Indicates User Bitmap index of the specified AMS. |
| } |  |  |
| else { |  |  |
| User Bitmap Index for the first new AMS | 5 | Indicates the User Bitmap index of the AMS associated with the first STID described. The User Bitmap index for remaining AMSs are determined by the order of STIDs described. |
| } |  |  |
| } |  |  |
| ISizeOffset Set ID | 3 | Indicates ISizeOffset set supported in the group |
| HARQ Burst Size Set ID | 2 | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | 2 | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (Frequency first allocation) 0b1: 4 DL subframe for FDD or all DL frame subframe for TDD (Time first allocation) |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: CL SU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| if (Nsubframe, A-MAP ==2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| ACK channel Index | TBD | Indicates the start of ACK channel used for the added AMSs to send acknowledgement for the addition information. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | 16 | The CRC is masked with a broadcast ID |

Table xxx - UL Broadcast Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index. |
| Number of new AMSs | 2 | Indicates the number of new AMSs added into the group 00: 2 AMSs 01: 3 AMSs 10: 4 AMSs 11: 5 AMSs |
| User Bitmap index description indicator | 1 | Indicates whether each AMS’s User Bitmap index is described |
| For (i=0, i++, i< Number of new AMSs ) |  |  |
| { |  |  |
| Station ID | 12 |  |
| Initial ACID | 4 | Indicates the start of ACID used for group resource allocation. |
| N_ACID | 3 | Indicates the number of ACIDs used for group resource allocation. |
| if (User Bitmap index description indicator ==1) && (i !=1){ |  |  |
| User Bitmap Index | 5 | Indicates User Bitmap index of the specified AMS. |
| } |  |  |
| else { |  |  |
| User Bitmap Index for the first new AMS | 5 | Indicates the User Bitmap index of the AMS associated with the first STID described. The User Bitmap index for remaining AMSs are determined by the order of STIDs described. |
| } |  |  |
| } |  |  |
| Initial ACID | TBD | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD | Indicates the number of ACIDs used for group resource allocation. |
| ACK channel Index | TBD | Indicates the start of ACK channel used for the added AMSs to send acknowledgement for the addition information. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC |

-------------------------------  Text # 6 End  ---------------------------------------------------