| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to section “Downlink signaling support of DL-MIMO modes” (AWD-15.3.7.2.5.5.) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics. | wbong@lge.com |
| Re: | 802.16m amendment working document |  |
| Abstract | In this contribution, we propose some modification on “Downlink signaling support of DL-MIMO modes” |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications to section “Downlink signaling support of DL-MIMO modes” (AWD-15.3.7.2.5.5.)

Wookbong Lee, Bin-Chul Ihm

LG Electronics.

- Introduction

In comment 280 of C80216m-09_0023r3, editor said “Cannot understand the proposed modifications in -1057r1 -- therefore not implemented.”

In this contribution, I rewrite the proposed modification based on comment 280 (C802.16m-09/1057r1) and according to the current AWD texts.

- Reference

[1] 80216m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- Proposed Remedy

- Delete table 692 and adopt the following texts

15.3.7.2.5.5. Downlink signaling support of DL-MIMO modes

The BS shall send some parameters necessary for DL MIMO operation in a broadcast message. The broadcast information is carried in the SFH or in the additional broadcast information.

The BS shall send some parameters necessary for DL MIMO operation in a unicast message. The unicast information is carried in the A-MAP IE, in the FBCH_Alloc_IE, on in the Sounding_ IE.

Table 692 specifies the DL control parameters required for MIMO operation.

Table 692 – DL MIMO control parameters

| Parameter | Description |  | Value | DL Control channel | Notes |
| --- | --- | --- | --- | --- | --- |
| Broadcast Information |  |  |  |  |  |
| Nt | Number of transmit antennas at the BS |  | 2, 4, 8 | SFH (system information) | Nt must be known before decoding the DL A-MAP IE |
| OL_Region | OL MIMO region, which signaling is used to indicate MS where is the pre¬defined OL MIMO region and number of streams (1 or 2) |  | OL-Region-ON (1 bit): Signal the existence of OL region. OL-Rank1-Config (3 bit): to signal the combination of sub-band and mini-band in OL region type 1. Refer to table TBD. SB-OL-Region-2-Size (4 bit) : signal the number of sub-bands in OL region type 2. | Additional broadcast information | OL region signaling is SFH-SP3 or ABI (TBD) |
| BC_SI | Rank-1 base codebook subset indication for interference mitigation with PMI coordination |  | BitMAP: 8 bits if Nt=2 16 bits if Nt=4 8 bits if Nt=8 | Additional broadcast information | Rank-1 codebook element restriction/recommendation information It shall be ignored if CM = 0b00, 0b01 or 0b10 |
| Unicast Information |  |  |  |  |  |
| MEF | MIMO encoder format |  | SFBC Vertical encoding Horizontal encoding | A-MAP IE | MIMO encoder format. |
| Mt | Number of streams in transmission |  | 1 to 8 | A-MAP IE | Number of streams in the transmission. |
| SI | Index of pilot stream allocation |  | 1 to 4 | A-MAP IE | SI shall be indicated if MEF is HE |
| Feedback Allocation IEs |  |  |  |  |  |
| MFM | MIMO feedback mode | Refer to Table 691 |  |  | To decide the feedback content and related MS processing |
| MaxMt | Maximum number of streams | If MFM indicates a SU feedback mode: 1, 2, 4 or 8. If MFM indicates a MU feedback mode: 1, 2, 3, or 4. |  |  | If MFM indicates a SU feedback mode: the maximum number of streams scheduled for each user If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| CS indication | Codebook subset type for CL MIMO modes 2 and 4 | Base codebook or codebook subset |  |  | Depending on the MFM and CS indication, the MS shall feedback a PMI from the SU or MU base codebook, or from a subset of the SU or MU of the base codebook. |
| CM | Codebook feedback mode for CL MIMO modes 2 and 4 | 0b00: standard mode with codebook coordination disabled 0b01: adaptive mode with codebook coordination disabled 0b10: differential mode with codebook coordination disabled 0b11: standard mode with codebook coordination enabled |  |  | In codebook-based feedback mode: Codebook coordination disabled: MS finds PMI within whole codebook type entry indicated by CS indication Codebook coordination enabled: MS shall find the rate-1 PMI within codebook entries indicated by BC_SI and CS indication |

The indication of the type of MIMO feedback for CL MIMO modes 2 and 4 depends on the IE type sent by the BS. Fast feedback channel allocation IE indicates that the MS shall use codebook-based feedback. The sounding commend IE indicates that the MS shall use uplink sounding.

When CS indication indicates the use of a codebook subset and MFM indicates a SU CL MIMO mode, the MS shall use the SU base codebook subset of Table 709 when Nt=4. When CS indication indicates the use of a codebook subset and MFM indicates a MU CL MIMO mode, the MS shall use the MU base codebook subset of Table 709 when Nt =4.