| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for 8Tx Codebook (AWD - 15.3.7.2.6.6.2.3) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Seunghyun Kang, Sunam Kim, Bin-Chul Ihm and Wookbong Lee LG Electronics. | {sh_kang, snkim816, bcihm, wbong} @ lge.com |
| Re: | 802.16m amendment working document |  |
| Abstract | This contribution provides the proposed text for DL 8Tx codebook. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for 8Tx Codebook (AWD - 15.3.7.2.6.6.2.3)

Seunghyun Kang, Sunam Kim, Bin-Chul Ihm and Wookbong Lee

LG Electronics

- Introduction

The current AWD has 8Tx codebook of 4bit size per rank which provisions enough beamforming gain for highly correlated case. When applying this 8Tx codebook for uniform linear array and cross polarized antenna, we can observe that several entries of the codebook for rank 2 does not play a role as shown in Figure 1. The last 8 entries of codebook for rank 2 are identified as unimportant entries through SLS. This contribution suggests the new entries of codebook for rank 2 to enhance the performance of SU-MIMO.

Figure. 1 Codebook matrix selection ratio when using AWD codebook for rank 2 under ULA and XPOL.

In Figure 1, XPOL type 1 and 2 means the two types for antenna indexing as shown in Figure 2.

Figure. 2 Two types of antenna indexing for XPOL.

- Proposed 8Tx Codebook

We propose to replace the entries 8~15 of codebook for rank 2 with the following matrices:

| Binary index | m | Base matrix | C(8,2,4,m) |
| --- | --- | --- | --- |
| 1000 | 8 |  | 1 2 |
| 1001 | 9 |  | 3 4 |
| 1010 | 10 |  | 5 6 |
| 1011 | 11 |  | 7 8 |
| 1100 | 12 |  | 1 2 |
| 1101 | 13 |  | 3 4 |
| 1110 | 14 |  | 5 6 |
| 1111 | 15 |  | 7 8 |

- Performance comparison

For XPol type 2, the base matrices were modified as below:

Figure 3 compares the selection ratio of the codebook matrices through the SLS based on the following simulation assumptions.

| Bandwidth | 10MHz (TDD) | Antenna Configuration | ULA, XPol |
| --- | --- | --- | --- |
| ISD | 1.5km | Antenna Spacing at BS | 0.5 λ |
| Fading Channel | Ped-B 3km/h | # of Rx at MS | 2 |
| Center frequency | 2GHz | Rank adaptation | No. Only rank 2 used. |

(a) ULA                          (b) XPol type 1                       (c) XPol type 2

Figure. 3 Selection ratio of codebook matrices under several antenna configuration

The last 8 entries of the current codebook for rank 2 do not play important roles in both ULA and XPol antenna configuration as shown in Figure 3 while the proposed 8 entries show great contribution for XPol case. Figure 3 implies the block diagonal matrices or their permuted version are more proper base matrices for XPol configuration. Table 1 proves this implication in terms of sector throughput. The proposed codebook gives the amazing gain in XPol case without increasing feedback overhead whilst keeping the performance in ULA case.

| Antenna configuration | Codebook | Average sector throughput | Cell edge user throughput |
| --- | --- | --- | --- |
| ULA | AWD | 1.95 (100%) | 0.77 |
|  | Proposed | 1.95 (100%) | 0.77 |
| Xpol type 1 | AWD | 1.25 (100%) | 0.42 |
|  | Proposed | 2.94 (235%) | 0.86 |
| Xpol type 2 | AWD | 2.68 (100%) | 0.78 |
|  | Proposed | 2.94 (108%) | 0.86 |

Table 1. Throughput comparison through SLS

- References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- Text proposal for inclusion in the 802.16m amendment

[Remedy 1]

Insert the following two matrices at line 43 page 254 of section 15.3.7.2.6.6.2.3.1 SU-MIMO base codebook

---------------------------------------------------   Text Start  ---------------------------------------------------

----------------------------------------------------   Text End  ----------------------------------------------------

[Remedy 2]

Replace Table 701 with the following table.

---------------------------------------------------   Text Start  ---------------------------------------------------

| Binary index | m | Base Matrix | C(8,2,4,m) | Base Matrix | C(8,3,4,m) | C(8,4,4,m) | C(8,5,4,m) | C(8,6,4,m) | C(8,7,4,m) | C(8,8,4,m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | 0 |  | 1 5 |  | 1 3 5 | 1537 | 12357 | 123567 | 1234567 | 12345678 |
| 0001 | 1 |  | 2 6 |  | 2 4 6 | 2648 | 12468 | 124568 | 1234568 | n/a |
| 0010 | 2 |  | 3 7 |  | 2 3 7 | 3726 | 23467 | 234678 | 1234678 | n/a |
| 0011 | 3 |  | 4 8 |  | 1 4 8 | 4815 | 13458 | 134578 | 1234578 | n/a |
| 0100 | 4 |  | 5 3 |  | 3 5 7 | 5372 | 23567 | 234567 | 2345678 | n/a |
| 0101 | 5 |  | 4 6 |  | 4 6 8 | 6481 | 14568 | 134568 | 1345678 | n/a |
| 0110 | 6 |  | 2 7 |  | 2 6 7 | 7264 | 24678 | 124678 | 1245678 | n/a |
| 0111 | 7 |  | 8 1 |  | 1 5 8 | 8153 | 13578 | 123578 | 1235678 | n/a |
| 1000 | 8 |  | 1 2 |  | 1 2 3 | 1234 | 12345 | 123456 | 1234567 | 12345678 |
| 1001 | 9 |  | 3 4 |  | 1 2 4 | 1246 | 12456 | 124567 | 1245678 | n/a |
| 1010 | 10 |  | 5 6 |  | 2 3 4 | 2437 | 23478 | 123478 | 1234578 | n/a |
| 1011 | 11 |  | 7 8 |  | 1 3 4 | 1348 | 13478 | 134678 | 1234678 | n/a |
| 1100 | 12 |  | 1 2 |  | 5 7 8 | 3578 | 23578 | 235678 | 1235678 | n/a |
| 1101 | 13 |  | 3 4 |  | 6 7 8 | 4678 | 14678 | 145678 | 1345678 | n/a |
| 1110 | 14 |  | 5 6 |  | 5 7 6 | 5678 | 35678 | 345678 | 2345678 | n/a |
| 1111 | 15 |  | 7 8 |  | 5 6 8 | 1568 | 13568 | 123568 | 1234568 | n/a |

----------------------------------------------------   Text End  ----------------------------------------------------