| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Section on OL Region |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi, Kwanhee Roh, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) |  |
| Abstract | This contribution proposes a section on OL region in order to describe the types of OL regions and the operation in OL region. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Section on OL MIMO Region

David Mazzarese et Al.

Samsung Electronics

# Introduction

The support of open-loop region has been accepted in the AWD in Session #61, and a certain number of features have been added to the AWD. But there is no section clearly defining the purpose and formalizing the usage of the OL region. We propose such a section.

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, 2009-05-31.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

# Text proposal for inclusion in the 802.16m amendment

<Insert section “15.3.7.2.5.2 OL Region” on line 58 page 238 as follows>

-------------------------------  Text Start  ---------------------------------------------------

#### 15.3.7.2.5.2 Open-Loop Region

An open-loop region with MaxMt streams is defined as a time-frequency resource using the MaxMt streams pilot pattern and a given open-loop MIMO mode with Mt = MaxMt without rank adaptation. The open-loop region allows base stations to coordinate their open-loop MIMO transmissions, in order to offer a stable interference environment where the precoders and numbers of streams are not time-varying. The resource units used for the open-loop region are indicated in a downlink broadcast message [SFH or ABI is TBD]. These resource units shall be aligned across cells.

Only a limited set of open-loop MIMO modes are allowed for transmission in the open-loop region. There is no limitation to the use of any open-loop MIMO mode outside the open-loop region, as specified in Table 689.

An open-loop region is associated with a specific set of parameters:

- Type (number of streams MaxMt, MIMO mode, MIMO feedback mode, type of permutation)

- Resource unit [TBD]

There are three types of open-loop regions, as specified in Table X.

Table X – Types of open-loop regions

|  | MaxMt | MIMO mode | MIMO feedback mode | Supported permutation |
| --- | --- | --- | --- | --- |
| OL Region Type 0 | 2 streams | MIMO Mode 0 MIMO Mode 1 (Mt = 2 streams) | 0 | DRU |
| OL Region Type 1 | 1 stream | MIMO Mode 5 (Mt = 1 streams) | 1 | Miniband based CRU (diversity allocation) Subband based CRU (localized allocation) |
| OL Region Type 2 | 2 streams | MIMO Mode 2 (Mt = 2 streams) MIMO Mode 3 (Mt = 2 streams) | 5 | Subband based CRU (localized allocation) |

The OL region type 0 is present if OL-Region-ON is indicated in a downlink broadcast message [SFH or ABI is TBD].

All base stations that are coordinated over the same open loop region should use the same number of streams, in order to guarantee low interference fluctuation and thus improve the CQI prediction at the AMS. All pilots are precoded by non-adaptive precoding with MaxMt streams in the open-loop region. CQI measurements should be taken by the AMS on the precoded demodulation pilots rather than on the downlink reference signals.

-------------------------------  Text End  ---------------------------------------------------