| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text on the A-MAP IEs for Group Resource Allocation |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim}@lge.com |
| Re: | Category: AWD comments/Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6.4.2.4 Group Resource Allocation A-MAP IE and AWD 15.3.6.4.2.5 Group Configuration A-MAP IE |  |
| Abstract | Proposed text for Unicast Control Information Elements for Group Resource Allocation |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text on the A-MAP IEs for Group Resource Allocation

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jin Sam Kwak

LG Electronics.

- Introduction

This contribution proposes texts to be included in the section 15.3.6.5.2.4 Group Resource allocation A-MAP IE and the section 15.3.6.5.2.5 Group configuration A-MAP IE of the 802.16m amendment working document (AWD).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

------------------------------- Start of Proposed Text #1 ---------------------------------------------------

[Remedy 1. Modify texts in the Chapter 15.3.6.5.2.4 Group Resource Allocation A-MAP IE as follows.]

15.3.6.5.2.4 Group Resource Allocation A-MAP IE

Group control information is used to allocate resources and/or configure resources to one or multiple AMSs within a group.

Group scheduling requires two operations

- Assignment of a AMS to a group. In order to add a AMS to a group in the DL or UL, the ABS shall transmit a Group Configuration A-MAP IE

- Allocation of resources to AMSs within a group. In order to assign resources to one or more AMSs in a group, the ABS shall transmit the DL/UL Group Resource Allocation A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE is included in user-specific resource assignment in an A-MAP region. The GRA A-MAP IE contains bitmaps to indicate scheduled AMSs and signal resource assignment, MCS, MIMO mode and resource size.

Table 671 DL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Resource Allocation A-MAP IE |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACK Channel Offset | TBD | Indicates the start of ACK index used for scheduled, deleted, or shifting AMSsallocations at this subframe in the group. |
| NDA | [2][3] | Indicates the number of deleted AMS in the group. |
| For(i=0,i++,i<NDA){ |  |  |
| User Bitmap Index | [5] | Indicates the User Bitmap Index of deleted AMSs. |
| } |  |  |
| User Bitmap Size | [2][5] | Indicates the length of User Bitmap |
| Shifting indicator | 1 | Indicates whether User Bitmap should be reconfigured and this A-MAP includes the User Bitmap. - 0b1: User Bitmap should be reconfigured. |
| User Bitmap size | [2][5] | Indicates the length of User Bitmap If Shifting indicator is set to 1, it indicates the size of User Bitmap. Otherwise, it indicates the size of scheduled User Bitmap [0b00: 4, 0b01: 8, 0b10: 12, 0b11: 16] or [0b00: 8, 0b01: 16, 0b10: 24, 0b11: 32] |
| If (Shifting indicator == 1) { |  |  |
| Retransmission flag | 1 | Indicates whether the User Bitmap is retransmitted 0: Transmission of the User Bitmap for shifting is initial transmission. 1: This User Bitmap for shifting is retransmitted. |
| User Bitmap | Variable | Bitmap to indicate scheduled and unscheduled AMSs belonging to this group. If retransmission flag is set to 1, this Bitmap will include the old User Bitmap structure. For each bit, 1 indicates a scheduled AMS or an unscheduled AMS belonging to the group. 0 indicates the resource hole or indicate that the corresponding User Bitmap index is not allocated to an AMS |
| If (retransmission flag == 1) { |  |  |
| ReTx Bitmap | Variable | This field will be included only if retransmission flag is set to 1. Indicates which AMSs among AMSs which should shift their User bitmap indices in User Bitmap should shift their User bitmap indices at this frame. The size of ReTx Bitmap equals the number of AMSs which should shift their User Bitmap indices in User Bitmap at this frame. |
| } |  |  |
| } |  |  |
| Scheduled User Bitmap | Var | Bitmap to indicate scheduled users in a group. If shifting indicator is 0, the size of this Bitmap is equal to the User Bitmap size If shifting indicator is 1, the size of this Bitmap is equal to the number of 1s in User Bitmap. |
| Deletion Indicator | 1 | Indicates whether any users are deleted from the group |
| If (Deletion Indicator == 1) { |  |  |
| Deletion Bitmap | var | Signals deleted users. If shifting indicator is 1 and retransmission flag is 0, Size equals number of 0’s in User Bitmap. Otherwise, [Size equals number of 0’s in scheduled user bitmap] or [Size equals number of 1’s in scheduled user bitmap] |
| } |  |  |
| If( Group MIMO mode set ==0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. |
| } |  |  |
| If( Group MIMO mode set == 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/resource size for each scheduled user AMS |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC |

Table 672 UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Resource Allocation A-MAP IE |
| Resource Offset | [6][8] | Indicates starting LRU for resource assignment to this group |
| ACK Channel Offset | TBD | Indicates the start of ACK index used for scheduled AMSs at this subframe in the group. |
| NDA | [2][3] | Indicates the number of deleted AMS in the group. |
| For(i=0,i++,i<NDA){ |  |  |
| User Bitmap Index | [5] | Indicates the User Bitmap Index of deleted AMSs. |
| } |  |  |
| User Bitmap Size | [2][5] | Indicates the length of User Bitmap |
| Shifting indicator | 1 | Indicates whether User Bitmap should be reconfigured and this A-MAP includes the User Bitmap. - 0b1: User Bitmap should be reconfigured. |
| User Bitmap size | [2][5] | Indicates the length of User Bitmap If Shifting indicator is set to 1, it indicates the size of User Bitmap. Otherwise, it indicates the size of scheduled User Bitmap [0b00: 4, 0b01: 8, 0b10: 12, 0b11: 16] or [0b00: 8, 0b01: 16, 0b10: 24, 0b11: 32] |
| If (Shifting indicator == 1) { |  |  |
| Retransmission flag | 1 | Indicates whether the User Bitmap is retransmitted 0: Transmission of the User Bitmap for shifting is initial transmission. 1: This User Bitmap for shifting is retransmitted. |
| A-MAP HFA index | TBD | Indicates the start of ACK index used for shifting AMSs at this subframe in the group. |
| User Bitmap | Variable | Bitmap to indicate scheduled and unscheduled AMSs belonging to this group. If retransmission flag is set to 1, this Bitmap will include the old User Bitmap structure. For each bit, 1 indicates a scheduled AMS or an unscheduled AMS belonging to the group. 0 indicates the resource hole or indicate that the corresponding User Bitmap index is not allocated to an AMS |
| If (retransmission flag == 1) { |  |  |
| ReTx Bitmap | Variable | This field will be included only if retransmission flag is set to 1. Indicates which AMSs among AMSs which should shift their User bitmap indices in User Bitmap should shift their User bitmap indices at this frame. The size of ReTx Bitmap equals the number of AMSs which should shift their User Bitmap indices in User Bitmap at this frame. |
| } |  |  |
| } |  |  |
| Scheduled User Bitmap | Var | Bitmap to indicate scheduled users in a group. If shifting indicator is 0, the size of this Bitmap is equal to the User Bitmap size If shifting indicator is 1, the size of this Bitmap is equal to the number of 1s in User Bitmap. |
| Deletion Indicator | 1 | Indicates whether any users are deleted from the group |
| If (Deletion Indicator == 1) { |  |  |
| If(Shifting indicator ==0) { |  |  |
| A-MAP HFA index | TBD | Indicates the start of ACK index used for deleted AMSs at this subframe in the group. |
| } |  |  |
| Deletion Bitmap | var | Signals deleted users. If shifting indicator is 1 and retransmission flag is 0, Size equals number of 0’s in User Bitmap. Otherwise, [Size equals number of 0’s in scheduled user bitmap] or [Size equals number of 1’s in scheduled user bitmap] |
| } |  |  |
| If( Group MIMO mode set ==0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. |
| } |  |  |
| If( Group MIMO mode set == 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate MCS/resource size for each scheduled user AMS |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC |

------------------------------- End of Proposed Text #1 ---------------------------------------------------

------------------------------- Start of Proposed Text #2 ---------------------------------------------------

[Remedy 1. Modify texts in the Chapter 15.3.6.5.2.5 Group Configuration A-MAP IE as follows.]

15.3.6.5.2.5 Group Configuration A-MAP IE

The unicast group configuration A-MAP IE is used for initiating and maintaining a group for resource assignment. The broadcast group configuration A-MAP IE is TBD.

Table 673 DL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index. |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| Minimum I_sizeoffset | 5 | Indicates minimum I_sizeoffset supported in the group |
| Maximum I_sizeoffst | 5 | Indicates maximum I_sizeoffset supported in the group |
| SE difference | 2 | Indicates the difference value of Spectral efficiency (modulation * coding rate) e.g.) 0b00: 0, 0b01: 0.1, 0b10: 0.2, 0b11: 0.3 |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Period | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframe for FDD or all DL frame subframe for TDD |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: CL SU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| if (Nsubframe, A-MAP ==2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| A-MAP HFA | [4] | HFA (HARQ feedback allocation) for A-MAP |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | TBD | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC |

Table 674 UL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group index. |
| MCS Set ID | [3] | Indicates MCS set supported in the group that is selected from [the predefined MCS set candidates][the configured MCS set candidates in additional broadcast message]. |
| Minimum I_sizeoffset | 5 | Indicates minimum I_sizeoffset supported in the group |
| Maximum I_sizeoffst | 5 | Indicates maximum I_sizeoffset supported in the group |
| SE difference | 2 | Indicates the difference value of Spectral efficiency (modulation * coding rate) e.g.) 0b00: 0, 0b01: 0.1, 0b10: 0.2, 0b11: 0.3 |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Period | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Long TTI Indicator | 1 | Defines number of subframe spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframe for FDD or all DL frame subframe for TDD |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: OL MU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| if (Nsubframe, A-MAP ==2){ |  |  |
| If(DL:UL !=3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| }else if (DL:UL !=3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| A-MAP HFA | [4] | HFA (HARQ feedback allocation) for A-MAP |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | TBD | Indicates the start of ACID used for group resource allocation. |
| N_ACID | TBD | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |
| MCRC | [16] | 16 bit masked CRC |

------------------------------- End of Proposed Text #2 ---------------------------------------------------