| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text for the IEEE 802.16m Amendment on Femtocell BS Reliability |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Inuk Jung, Jin Lee, Giwon Park, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-1879 [cooper, jins978, gwpark, ksryu, ronnykim, samji] @lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Femtocell (Reliability) |  |
| Abstract | Proposed text for Femtocell in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text for the IEEE 802.16m Amendment on Femtocell Reliability

Inuk Jung, Jin Lee, Giwon Park, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes the proposed texts for Femtocell sections to be in the 802.16m Amendment. The technical texts and ToCs are inherited from the 802.16m SRD [1], 802.16m SDD [2], and the 802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the 802.16m Amendment [4]. In this contribution, amendment text including several illustrations is proposed for the Femtocell section in which 16m members have enough consensus.

- Rationale/Motivation of modifications

The reliability of a femtocell BS during a power down, whether by accident or manual configuration, shall not have great impact on the AMSs that are currently being served by it. Since such is also stated in the WiMAX femtocell requirement document, we propose to further clarify any procedures related to Femtocell Reliability during such shutdown cases. To support such reliability functionalities, the femtocell should be able to indicate its current status and its related parameters to its serving AMSs. Not only disable status but battery/power status or femtocell scanning status can be indicated to AMSs. However, here we mainly focus on disabled cases due to link failure.

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document”

- Proposed Outline for 802.16m Femtocell Operation Amendment Text

---------------------------------------------------------------Text Start ------------------------------------------------------------

15.2.21 Femtocell BS Operation

15.2.21.x Femtocell BS Reliability

Femtocell BS shall disable downlink air interface transmitter as soon as the connection with the service provider network is lost for a configurable pre-defined time. In such a case, Femtocell BS should support the mechanisms to ensure service continuity of the MSs prior to disabling air interface. For example, the BS initiated handover depicted in 15.2. When a Femtocell BS needs disable air interface, it should send out an indication, which contains available out-of-service information such as out of service reasons and expected downtime and/or expected uptime, if it is able to do so, to prevent MS entry or reentry from other cells. Upon reception of the indication, the subordinated MSs should perform handover to neighboring cells.

If a femtocell BS is scheduled to be disabled, it shall broadcast a message AAI_STAT-IND message. The AAI_STAT_IND message conveys information describing the femtocell BSs current status and further action that may be taken. In the where the femtocell BS is scheduled to be disabled, AAI_STAT-IND message conveys information of reason for disablement, duration of disablement and expected downtime/uptime. Also the AAI_STAT-IND message shall include information whether HO may be performed. If HO is to be performed, the AMS may be informed whether

If AMS performed HO to a neighbor BS, the AMS shall scan the previous femtocell BS at the expected uptime, if provided, and decide whether to return to it via HO or not.

The AAI_STAT-IND message includes the following parameters:

- Current BS status indicator (Disable)

- Parameters related to it

    - Reason of link down

    - Expected uptime

    - Expected uptime

    - HO coordination flag

---------------------------------------------------------------Text End ------------------------------------------------------------