| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text for the IEEE 802.16m Amendment on Femto/SON Neighbor Discovery |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Inuk Jung, Jin Lee , Giwon Park, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-1879 [cooper, jins978, gwpark, ksryu, ronnykim, samji] @lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Femtocell |  |
| Abstract | Proposed text for Femtocell in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text for the IEEE 802.16m Amendment on Femto/SON Neighbor Discovery

Inuk Jung, Jin Lee, Giwon Park, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes the proposed texts for Femtocell sections to be in the 802.16m Amendment.

- Rationale/Motivation of modifications

The neighbor list of a femtocell is currently defined to be managed either by reports of AMSs or provided by the core network. Reports from the AMS is requested by the femtocell, which in turns request to used AMSs power for its neighbor list retrieval. Since the environment of femtocell network is apt to change due to newly installed neighbor femtocell BSs or removal of them, the neighbor list of each femtocell BS shall reflect such changes. However, to correctly cover such a diverse environment, the femtocell BS shall periodically request AMSs to report measurements of the environment, which implies high power usage in total of AMS.

Hence, we propose that the femtocell BS shall be able to perform measurement of its environment instead of relying only on reports of AMSs. In this manner, the femtocell BS may detect any newly installed or removed femtocell BSs by monitoring their signals (e.g. preamble) and accordingly add or delete them from its neighbor list respectively.

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access”

[2] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document”

- Proposed Outline for 802.16m Femtocell Operation Amendment Text

---------------------------------------------------------------Text Start ------------------------------------------------------------

15.2.22 Support for Self-Organization

15.2.22.x Neighbor Discovery

The initial neighbor list is obtained from the core network automatically. Any change of the neighbor environment such as BSs installment or removal should automatically trigger the BS to generate an updated neighbor list. The information for updating the neighbor list (e.g. macro BS, Femtocell BS) is collected by means of BS/RS/MS measurement, core network, inter-BS network signaling and BS’s own management. The BS should direct an MS to report measurement and use cached and feedback information to reduce the undesirable transmission from the MS.

Femtocell BS may discover its surrounding network topology by collecting information from the core network, inter-BS network air-signaling and AMS’s scan reports.

Upon installing a femtocell BS, the femtocell BS may receive a location based neighbor list from the core network. Based on the initial neighbor list, the femtocell BS shall schedule a scanning interval to scan neighbor BSs. During the scanning interval, the femtocell BS listens to neighbor BSs preamble. Such scanning interval shall be informed to AMSs that are currently served by the femtocell BS via the AAI_STAT-IND message.

If an initial neighbor femtocell BS’s preamble is not detected over X sequential scanning iterations, the femtocell BS shall discard the femtocell BS from its Femto Neighbor List.

AAI_STAT-IND message shall contain the following parameters when femtocell BS is performing scanning:

- Scan duration

- Scan interleaving interval

- Scan iteration

---------------------------------------------------------------Text End ------------------------------------------------------------