| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text for the IEEE 802.16m Amendment on Femtocell Idle Mode |  |
| Date Submitted | 2026-07-28 |  |
| Source(s) | Giwon Park, Jin Lee, Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-1879 [gwpark, jins978, cooper, ksryu, ronnykim, samji] @lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Femtocell (Idle mode) |  |
| Abstract | Proposed text for Femtocell in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text for the IEEE 802.16m Amendment on Femtocell Idle Mode

Giwon Park, Jin Lee, Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

One or more paging groups may be assigned to a Femtocell BS as Macro BS. The idle mode operation of Femtocell may be same as Macro idle mode operation. Thus, we propose Femtocell idle mode operation shall be the same as the Macro idle mode operation (paging and location update process) defined in section 15.2.7 of IEEE 802.16m AWD [1].

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- Proposed Outline for 802.16m Femtocell Operation Amendment Text

---------------------------------------------------------------Text Start ------------------------------------------------------------

15.2.17.8 Idle Mode Support for SON/Femto

The OSG Femtocell BSs operate like macro BSs when paging an MS.

Femtocell BS shall support idle mode. The CSG Femtocell BSs may broadcast the paging messages that are related to only the MSs of this CSG.

Dependent on topology design to support both Femtocell BS and Macro BS, one or more PGs may be assigned to a Femtocell or a Macro BS. The overlay Macro BS and the CSG Femtocell BS may share the same paging group ID or have the different paging ID.

The Femtocell idle mode operation (paging and location update procedure) shall be the same as the Macro idle mode operation.

---------------------------------------------------------------Text End ------------------------------------------------------------