| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Mitigation DG Harmonized Text Proposal |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Clark Chen Intel Corporation Jeongho Park Samsung Electronics Co. Ltd. | clark.chen@intel.com Jeongho.jh.park@samsung.com |
| Re: | Interference Mitigation DG |  |
| Abstract |  |  |
| Purpose | IM DG Harmonized Text proposal for AWD, for TGm approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Mitigation DG Text Proposal

- Proposed Text Input to P802.16m Amendment Working Document

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.2.X  Interference Mitigation Mechanism

The Advanced Air Interface use interference mitigation (IM) schemes that shall reduce unnecessary measurement feedback and be flexible enough for adoption of any available IM method.

15.2.X.1 DL FFR

Fractional frequency reuse (FFR) techniques allow different frequency reuse factors to be applied over different frequency partitions. The maximum number of frequency partition is four. Note that the frequency partition is defined in 15.3.5.2.3.

The FFR partition boundary is aligned with PRU units. The FFR partitions are indexed from lower Logic Resource Unit (LRU) index to highest LRU index. It always starts from reuse-1 partition and then followed by the three reuse-3 partitions. They are numbered as FFR partition(0), partition(1), partition(2), and partition(3) respectively. The Partition configuration should be semi-static and change with very long interim. The minimum interim is TBD.

Figure xxx

Each partition may have different power level per sector.  The transmission power level on different FFR partitions is decided by BS. The power loading level should be semi-static and change with very long interim. The minimum interim is TBD.

The ABS transmits necessary information for DL FFR. The DL frequency partition configuration including the number of frequency partitions and size of each frequency partition is broadcasted through S-SFH IE as described in 15.3.6.5.1.2.

For DL FFR, the AMSs shall be capable of reporting the interference information to the serving ABS. The serving ABS can instruct AMS to perform interference measurement over the designated radio resource region in solicited/unsolicited manner. The measurement results can then be reported by message and/or feedback channel.

### 15.2.X.1.1 DL/UL Siganling

When supporting FFR operation, the AMS shall be capable to measure the interference statistics over specific frequency partitions for evaluating the preferred frequency partition. AMS shall be capable to report the preferred frequency partition through the fast feedback channel.

AMS shall also be capable to report the interference statistics of specific frequency partition to ABS by AAI_FFR-REP message. The AAI_FFR-REP message defined in 15.2.5.y is sent in response to the measurement request from AAI_FFR-CMD defined in 15.2.5.x message by ABS. If ABS cannot serve the AMS by the recommended frequency partition, ABS can base on the interference statistics reported by AMS to schedule DL data transmission.

The ABS transmits necessary information for DL FFR. DL FFR configuration including the number of frequency partitions and size of each frequency partition is broadcasted through S-SFH. The S-SPH SP2 IE related to DL FFR is given in 15.3.6.5.1 and shown in Table x1. The DL frequency partition configuration (DFPC) information is given in 15.3.5.2.3 and shown in Table y1 and y2. [DL power boosting level for each frequency partition is broadcasted].

{Editor’s Note: Replace DL FFR-related contents in Table 664 in 15.3.6.5.1 as contents in Table x1}

Table x1 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| . . . . | . . . . | . . . . |
| Frequency Partition Configuration | [3~4] | 4 bits for 20MHz 3 bits for 10MHz and 5MHz |
| . . . . | . . . . | . . . . |
| } |  |  |

Table 652—Mapping between DFPC and frequency partitioning for 20MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 1 : 1 : 1 | 4 | NPRU * 9/12 | NPRU * 1/12 |
| 6 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 7-15 | Reserved |  |  |  |

Table 653—Mapping between DFPC and frequency partitioning for 10MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 5 : 1 : 1 : 1 | 4 | NPRU * 5/8 | NPRU * 1/8 |
| 5 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 6-7 | Reserved |  |  |  |

Table 654—Mapping between DFPC and frequency partitioning for 5MHz

| DFPC | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 1 : 0 : 0 : 0 | 1 | NPRU | 0 |
| 1 | 0 : 1 : 1 : 1 | 3 | 0 | NPRU * 1/3 |
| 2 | 1 : 1 : 1 : 1 | 4 | NPRU * 1/4 | NPRU * 1/4 |
| 3 | 3 : 1 : 1 : 1 | 4 | NPRU * 1/2 | NPRU * 1/6 |
| 4 | 9 : 5 : 5 : 5 | 4 | NPRU * 3/8 | NPRU * 5/24 |
| 5-7 | Reserved |  |  |  |

{Editor’s Note: Insert the following subsections 15.2.5.x and 15.2.5.y under the 15.2.5 section.}

15.2.5.x. AAI_FFR-CMD (FFR Command) Message

An AAI_FFR-CMD message shall be transmitted by the ABS to instruct AMS perform measurement over specific frequency partition. The following are the parameters shall be included into the AAI_FFR-CMD message:

Table xxx AAI_FFR-CMD message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Frequency Partition Index | 2 | - |
| Measurement Duration | 8 | AMS shall reply AAI_FFR-REP message to report the measurement after this indicated time |

15.2.5.y. AAI_FFR-REP (FFR Report) Message

An AAI_FFR-REP message is sent by an AMS to report the interference statistics of specific frequency partition. The following are the parameters shall be included into the AAI_FFR-REP message:

Table xxx AAI_FFR-REP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Frequency Partition Index | 2 | - |
| Reporting Type | 1 | 0: Interference Measurement Report 1: SINR Measurement Report |
| If (Reporting Type == 0) { |  |  |
| Interference Measurement Report - Mean | 8 |  |
| Interference Measurement Report - Variance | 8 |  |
| } else { |  |  |
| SINR Measurement Report - Mean | 8 |  |
| SINR Measurement Report - Variance | 8 |  |
| } |  |  |

15.2.X.2 UL FFR

UL FFR allows the system to designate a different UL power control target such as target IoT (exact power control target (TBD) is related to UL power control section.) per frequency partition. Note that the UL frequency partition is defined in 15.3.8.2.3.

For UL FFR, the ABSs shall be capable to measure the interference statistics over each frequency partition. In order to support UL FFR, the ABS transmits necessary information. UL FFR configuration including the number of frequency partitions and size of each frequency partition is broadcasted through S-SFH. The S-SPH SP2 IE related to UL FFR is given in 15.3.6.5.1 and shown in Table xx1. The UL frequency partition configuration (UFPC) information is given in 15.3.8.2.3 and shown in Table yy1 and yy2. Also,UL power control target for each frequency partition is broadcasted as shown in Table zz1 [through ABI or S-SFH is TBD].

{Editor’s Note: Replace UL FFR-related contents in Table 664 in 15.3.6.5.1 as contents in Table xx1}

Table xx1 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| . . . . | . . . . | . . . . |
| Uplink Frequency Partition Configuration | [3~4] | 4 bits for 20MHz 3 bits for 10MHz and 5MHz |
| . . . . | . . . . | . . . . |
| } |  |  |

Table zz1 – Broadcast information of UL power control target for each frequency partition

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| IE format () { |  |  |
| . . . . | . . . . | . . . . |
| UL power control target for FP0 (and FP1 when UFPCT=4) | 4 | UL power control target is γ or delta_IoT [Note: depending on PC decision] |
| If (UFPCT >1) { |  |  |
| for (i=UFPCT-2; i<UFPCT; i++) { |  |  |
| Relative adjustment of PC target value for FPi (i>1) | [2 or 3] | For FPi (i>=UFPCT-2) |
| } |  |  |
| } |  |  |
| . . . . | . . . . | . . . . |
| } |  |  |

{Editor’s Note: Replace Table 716 and 717 in 15.3.8.2.3 as Table 652, 653, 654 in section 15.2.x.1.1}

15.2.X.3 DL Multi-BS MIMO

Multi-BS MIMO techniques improve sector throughput and cell-edge throughput through multi-BS cooperative signaling. These include single-BS precoding with multi-BS coordination and multi-BS joint processing.

### 15.2.X.3.1 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques based on the MIMO schemes defined in Section 15.3.7 with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

### 15.2.X.3.1.2 DL/UL Signaling

### 15.2.X.3.1.2 Operation procedure

Two types of single BS precoding techniques with Multi-BS coordination may be supported in AAI. One is PMI coordination, supported by codebook-based feedback, and the other is interference nulling, supported by codebook-based feedback or by uplink sounding.

Single BS precoding with Multi-BS Coordination may be enabled by the ABS for one or several AMSs when CL MIMO precoding is applied in the serving and neighboring cells. The inter-cell interference can be mitigated by coordinating the precoders applied in neighboring cells via higher layer signaling, based on feedback from AMSs to their respective serving ABSs.

With codebook-based feedback, PMI coordination can be applied as either PMI recommendation or PMI restriction, as instructed by the ABS in Feedback_Polling_IE.

If ICT (interference coordination type) is set to 0b00 in Feedback_Polling_IE, then the MS finds the PMI which acts as the strongest interference for the neighboring cell in the frequency resource unit indicated by TRU (target resource unit) indicated in Feedback_Polling_IE.

If ICT (interference coordination type) is set to 0b01 in Feedback_Polling_IE, then the MS finds the PMI which acts as the weakest interference for the neighboring cell in the frequency resource unit indicated by TRU (target resource unit) indicated in Feedback_Polling_IE.

[Additional information on channel quality improvement may be reported in the form of a CQI associated with the reported PMI for the neighboring cell.]

Restricting or recommending the usage of rank-1 codebook elements as a response to the neighboring cell’s request is done by the BS transmission of BC_SI in additional broadcast information. Details are in 15.3.7.2.6.6.2.4. {Editor’s Note: This subclause15.3.7.2.6.6.2.4 should have the tile of “Codebook subset selection”}

The operation procedure of PMI coordination follows the steps below:

1.	The ABS may send a Feedback_Polling_IE to an AMS. The ABS indicates the feedback mode in the Feedback_Polling_IE.

2.	Once an AMS receives a Feedback_Polling_IE, it shall send a periodic feedback header with the requested information. This information may include a PMI or a set of PMIs, [subband index], Temp_BSID (diversity member ID) and CQI.

[

- For each PMI () in the rank-1 DL base codebook, AMS estimates the interference power of the dominant interference links. AMS also calculates the cross correlation of each PMI to the recommended (or restricted) PMI wk. The cross-correlation between PMIs i and k is given in equation (1), with superscript H indicating conjugate transpose.

i=1,…, N		(1)

Assume that the N correlation values,, are sorted in descending order, such that:

(2)

- The AMS determines the size of the subset of PMIs to be jointly recommended (or restricted), based on two fixed correlation levels determined by n1 and n2 in equation (2). The AMS indicates the selection of n1 or n2 via PMI_coordination_subset in a feedback header. The value of PMI_coordination_subset is specified in Table X3.

Table X3 - PMI_coordination_subset

| PMI_coordination_subset | Value |
| --- | --- |
| 0b0 | n1 |
| 0b1 | n2 |

]

3.	Upon receiving feedback from multiple AMSs, an ABS should communicate with neighboring ABSs to coordinate their usage of PMIs via higher layer signaling. The ABS should then broadcast codebook subset information in BC_SI in Additional Broadcast Information (ABI) message to all AMSs in its cell. BC_SI is indicated by a bitmap [, or by reusing the same format as feedback by one AMS in the feedback header].

4.	The ABS may send a Feedback Allocation A-MAP IE with CM set to 0b11 to selected AMS. Consequently, these AMSs should feedback their desired PMI in the codebook subset broadcasted in BC_SI.

[With Nt = 4 transmit antennas at the ABS, the recommended (or restricted) PMIs in the base codebook C(4,1,6) are derived from the restricted or recommended PMIs in the base codebook subset C(4,1,4) according to the hierarchical relationship specified in Table 678 [TBD]. The level 2 codebook elements shall be restricted or recommended when the level 1 codebook element of the base codebook subset is in the restricted or recommended set, respectively.]

Inter-cell interference nulling can be done using PMI which acts as a strongest interference for the neighboring cell or overhearing neighboring cell’s sounding signal.

### 15.2.X.3.2 DL Multi-BS Joint MIMO Processing

This subclause introduces interference mitigation techniques based on joint MIMO transmission across multiple ABSs. The ABS and AMS may optionally support both adaptive precoding based multi-BS joint processing e.g. Closed-loop Marco Diversity (CL-MD) and Collaborative MIMO (Co-MIMO) transmission and non-adaptive precoding based multi-BS joint processing. Multi-BS joint MIMO processing may be enabled by the ABS for one or several AMSs when adaptive or non-adaptive precoding is applied in the serving and neighboring cells and user data is shared among multiple cells.

### 15.2.X.3.2.1 DL/UL Siganling

Table x1 – Control parameters for DL Multi-BS MIMO supported by codebook based feedback

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - ICT | - Interference coordination type | - 0b000: PMI restriction - 0b001: PMI recommendation - [0b010 : Non-adaptive precoding based mutli-BS joint prcessing] - 0b011: CL-MD - 0b100: Co-MIMO | - Feedback_Polling_IE | - Indicates which Multi-BS MIMO mode |
| - TRU | - Target resource unit | - TBD | - Feedback_Polling_IE | - Indicates resource units for measurement |
| - [SIS | - Stream index set | - TBD | - Feedback_Polling_IE | - Indicates preferred streams for both serving cell and neighboring cells] |
| - [PCF | Phase Correction Feedback | 0b0: feedback - 0b1: not feedback | - Feedback_Polling_IE | - Only used in CL-MD or Co-MIMO] |
| - MaxUser | Maximum number of users supported in Co-MIMO | Maximum supported user number = MaxUser+2, e.g., 0b000 means 2 AMSs are supported in maximum. | - Feedback_Polling_IE | - Only used in Co-MIMO. In CL-MD, the maximum supported user number is 1. |

Co-MIMO and CL-MD can be supported as instructed by the BS in the Sounding_command_IE.

Table x2 – Feedback information for DL Multi-BS MIMO supported by codebook based feedback

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is diversity set member ID assigned to this ABS. |
|  | PMI report for serving and neighboring cell | For PMI coordination among multiple ABSs |
|  | [PMI_coordination_subset indication] | [Indicating one correlation level or two correlation levels associated with the PMI report for neighboring cell] |
|  | [Subband index] | Subband index associated with the PMI report for neighboring cell |
|  | [SIS report for serving and neighboring cells | Preferred streams among multiple ABSs] |
|  | [PCF for neighboring cell | Phase correction factor for neighboring cell] |
|  | [CQI, differential CQI, or normalized interference power] | For resolving conflict or link adaptation |
| Event-driven feedback | Requesting Multi-BS MIMO | For AMS reporting of its preference on Multi-BS MIMO operation using a MAC management message |

Uplink sounding can be used to support Co-MIMO and CL-MD.

### 15.2.X.3.2.2 Operation procedure

With non-adaptive precoding, the precoding matrix is chosen from the codebook described in section 15.3.7.1.2.1.

[Each AMS shall report its preferred stream index for every coordinating ABS to its serving ABS. All coordinating ABSs shall exchange preferred stream index accompanied with corresponding CQI and the data traffic for their associating AMSs through backhaul network.]

With adaptive precoding, the precoder matrix Wk is derived from the feedback of the MS, e.g. codebook-based feedback and sounding-based feedback. Two types of adaptive precoding based multi-BS joint processing are supported, CL-MD and Co-MIMO. When CL-MD is enabled, a single AMS is served jointly by multiple coordinating ABSs. When Co-MIMO is enabled, several AMSs are served jointly by the multiple coordinating ABSs through MU-MIMO scheduling and precoding.

For codebook-based feedback, the AMS(s) choose the PMIs for the serving cell and the neighboring cells based on the respective estimated channel state information. [Optionally, the serving ABS can also instruct the AMS(s) to feedback a phase correction for the neighboring cells to further improve the system performance.]

When DL Multi-BS joint processing is enabled, radio resource allocation, data mapping and pilot pattern allocation should be aligned among coordinating ABSs. The same data packet is transmitted by the coordinating ABSs on the same time and frequency resources. The same pilot patterns without interlacing shall apply to the coordinating BSs.

The operation procedure of DL Multi-BS Joint Processing follows the steps below:

1.	DL Multi-BS Joint Processing may be triggered by an AMS in an unsolicited manner. Based on the channel measurements of the desired and interference links reference signals, an AMS may request operation with DL Multi-BS Joint Processing in an unsolicited manner via an event-driven feedback report to its serving BS using one codeword in the P-FBCH.

2.	The ABS may accept or reject the request from the AMS. The ABS shall send a Feedback_Polling_IE to the AMS to accept the request, or it may do nothing to reject the request. The ABS may also send a Feedback_Polling_IE to an AMS in an unsolicited manner. The ABS shall indicate operation with non-adaptive precoding based multi-BS joint processing, CL-MD or Co-MIMO in the Feedback_Polling_IE.

3.	Once an AMS receives a Feedback_Polling_IE, it shall send a periodic feedback header with the requested information in case of [stream index feedback and] codebook feedback, or send UL sounding as instructed by Sounding_Allocation_IE() in case of sounding feedback.

4.	Upon receiving feedback from multiple AMSs [, in case of stream index feedback, the ABS shall forward the preferred stream index and CQI to neighboring ABSs to coordinate the usage of streams.] In case of codebook feedback, the ABS shall forward the PMIs [and PCI (in case of PCI feedback enabled)] to neighboring ABSs to coordinate the usage of PMIs. In case of sounding feedback, each involved ABS can perform precoding based on the received sounding signal(s) from single AMS for CL-MD or from multiple AMSs for Co-MIMO.

### 15.2.X.3.3 Collaborative MIMO (Co-MIMO) Transmission

The default number of neighboring ABSs coordinated to support Collaborative MIMO (Co-MIMO) transmission is three.

The Collaborative MIMO Zone (Co-MIMO Zone) is defined to facilitate inter-ABS coordination for supporting Co-MIMO transmission. Co-MIMO Zone is a radio resource region composed by LRUs and sub-frames, where the Co-MIMO Zone utilized by neighboring ABSs for Co-MIMO transmission will associate to the same LRU and sub-frames. The permutation of the Co-MIMO Zone for these ABSs shall be the same.

Each Co-MIMO Zone will be used for serving one AMS, and each ABS can have multiple Co-MIMO Zones to support Co-MIMO transmission for multiple AMSs. The associated LRUs, permutation and sub-frames for each Co-MIMO Zone will be negotiated by the ABSs involved in Co-MIMO transmission through the backhaul network before the allocation.

15.2.X.4 UL Multi-BS MIMO

### 15.2.X.4.1 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques based on the MIMO schemes defined in Section 15.3.10 with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

### 15.2.X.4.1.1 DL Signaling

Table x1- Control parameters for UL multi-BS MIMO

| - Parameter | - Description | - Value | - Control channel (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - UCCE | - Uplink codebook coordination enable | - 0b0: disable - 0b1: enable | - UL basic assignment A_MAP_IE | - Indicates operation of PMI combination |
| - UTRU | - Uplink target resource unit | - TBD | - UL basic assignment A_MAP_IE | - Indicates resource units for measurement |
| - PMImin | - PMI minimizing interference |  | - UL basic assignment A_MAP_IE | - Enable when UCCE=0b1 - [Preferred PMI minimizing inter-cell interference for the neighboring ABS] |

### 15.2.X.4.1.2 UL Signaling

Table x2-Feedback information for UL multi-BS MIMO

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. |

### 15.2.X.4.1.3 Operation procedure

Single BS precoding with Multi-BS Coordination proceeding by combining two egoistic and altruistic PMIs can be used to mitigate inter-cell interference (ICI) without much increase of feedback overhead for CL SU MIMO.

The operation procedure of PMI combination follows the steps below:

- PMI combination may be triggered by an ABS in an unsolicited manner. Based on channel measurements, ABSs may send an UL basic assignment A_MAP_IE to an AMS indicating operation with PMI combination.

- Once an AMS receives an UL basic assignment A_MAP_IE, it shall send a periodic feedback header with the requested information. This information may include a Temp_BSID.

-The BS_ID of the neighboring BS which suffers the most interference caused by the AMS is selected through the following criteria:

,                                                           (1)

where i is the index of AMS who reported the RSSI information, q is the index of serving ABS and p is the index of neighboring ABS. RSSIp,iis the RSSI measurement result from ABS p measured by AMS i.

- By using UL sounding signals, the serving and neighboring ABSs shall measure channels and determine PMImax maximizing transmission power and PMImin minimizing interference, respectively. The neighboring ABS shall inform the serving ABS of PMImin through the backhaul network.

-Two PMIs of the predefined codebook, one of which maximizes the transmission power for the serving ABS and another of which minimizes the interference power for the neighboring ABS from (1), should be selected through the following equations:

,                                                                (2)

where  is the PMI.  and  denote information of channels from the AMS to its serving ABS and the neighboring ABS, respectively.

- The serving ABS shall feedback PMImin and PMImax to the AMS.

-The AMS determines a PCR  based on two fixed RSSI_difference criterion  (TBD).

(3)

-The transmitted precoder  combines two precoders (,) corresponding to PMImin and PMImax through, i.e.,

(4)

### 15.2.X.4.2 UL Multi-BS Joint MIMO Processing

The Advanced Air Interface shall support uplink multi-BS MIMO to allow joint reception of multiple ABSs, e.g. macro-diversity combining and cooperative beamforming. The uplink multi-BS MIMO capability can be enabled or disabled in the REG-REQ/RSP (TBD) message exchange. With uplink multi-BS MIMO enabled, the MS shall perform the following stages:

- Decision: Uplink multi-BS MIMO operation begins with a decision for an MS to receive from multiple BSs at the same time. This decision can be made at MS and notify its serving BS (anchor BS) through MAC request message from MS, or made at anchor BS and notify MS through MAC request message from BS.

- Report: An MS may measure interference information of neighboring BSs, select BSs that contribute to majority of inter-cell interference, and then report the interference information e.g. RSSI or the selected BSs to anchor BS.

- Collaborative zone initialization: A common radio resource allocation shall be allocated as a collaborative zone among those BSs involved in uplink multi-BS MIMO operation. Uplink sounding signals shall be allocated orthogonally among MSs in the collaborative zone.

- Inter-BS information exchanging and joint processing: With macro-diversity combining enabled, soft decision information in the form of log-likelihood ratios are generated at neighboring BSs, transmitted to anchor BS accompanied with scheduling information through M-SAP/C-SAP primitives (TBD) over backhaul network, and combined at anchor BS. With cooperative beamforming enabled, quantized versions of received signals are generated at neighboring BSs, transmitted to anchor BS accompanied with channel state information and scheduling information through M-SAP/C-SAP primitives (TBD) over backhaul network, and jointly processed at anchor BS.

{Editor’s Note : Insert the following section 15.2.3.1.x in line 14, page 12 of AWD10r2.}

15.2.3.1.x. MIMO channel feedback header

Table Y. Description of MIMO channel feedback header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| if (ICT == 0b000, 0b001, 0b010, 0b011 or 0b100) { |  | ICT is defined in Feedback Polling A-MAP IE |
| for (i = 1; i <= N_multiBS_reports; i++) { |  |  |
| Temp_BSID | [TBD] | Diversity set member ID |
| If (ICT==0b000 or 0b001) { |  |  |
| PMI | [3 to 6] | PMI from the rank-1 base codebook [or base codebook subset] |
| [PMI_coordination_subset] | [1] | Set of PMIs from the rank-1 base codebook [or base codebook subset] |
| [Subband index] | [TBD] | This field may be omitted depending on TRU |
| } |  |  |
| If (ICT==0b010) { |  |  |
| [SIS | [TBD] | Preferred stream index set] (Notes: this is relevant to principle of OL multi-BS MIMO, and needs further discussions) |
| [CQI] | [TBD] | Indicator of channel quality associated with multi-BS MIMO interference mitigation technique |
| } |  |  |
| If (ICT==0b011 or 0b100 and PCF == 0b0 and Temp_BSID isn’t the serving BS) { |  | ICT and PCF are defined in Feedback Polling A-MAP IE |
| PMI | [3 or 6] | PMI from the rank-1 base codebook |
| [PCI | [TBD] | Phase correction indicator for neighboring cells] (Notes: this is relevant to phase correction, and needs further discussions) |
| [CQI] | [TBD] | Indicator of channel quality associated with multi-BS MIMO interference mitigation technique |
| } |  |  |
| } |  |  |
| } |  |  |

{Editor’s Note : Insert the following section 15.3.6.4.2.x in line 60, page 67 of AWD10r2.}

15.3.6.4.2.x. Feedback Polling A-MAP IE

This IE is used by BS to schedule feedback header transmission by the MS.

Table X. Feedback Polling A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | Feedback_Polling_IE |
| Feedback Type | 2 | [0b00 : stream feedback] 0b01 : codebook feedback 0b10 : sounding feedback |
| If (Feedback Type = 0b00 or 0b01) |  | Note: for sounding feedback, MS shall detect the Sounding_Allocation_IE for detailed sounding allocation information. |
| { |  |  |
| MCS | [2 or 4] | MCS level for transmitting feedback header |
| Resource Allocation | [TBD] | Indicating the first Distributed LRU index for transmission |
| Allocation Duration (d) | [3] | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| Period (p) | [2] | Transmit feedback header every [2p superframe] |
| TRU | [3] | - Target RU indicating which RUs or which type of RU to work on for feedback |
| ICT | [3] | 0b000: PMI recommendation for single-BS precoding; 0b001: PMI rejection for single-BS precoding; 0b010: Non-adaptive precoding based mutli-BS joint processing 0b011: CL-MD for multi-BS precoding; 0b100: Co-MIMO for multi-BS precoding; 0b101~0b111: [TBD] |
| N_multiBS_reports | [3] | If Feedback type == 0b000 to 0b011, then N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b011 or 0b100) { |  |  |
| 4. [PCF | 1 | Phase Correction Feedback; 0b0: feedback phase correction factors for N_multiBS_reports-1 diversity set members; 0b1: not feedback phase correction factors] |
| If (ICT = 0b100) { |  |  |
| MaxUser | 3 | Maximum number of users supported in Co-MIMO, 0b000 corresponds to 2 users in maximum are supported in the same resource. This will impact the CQI estimation at the MS side. |
| } |  |  |
| } |  |  |
| Measured Logical Carrier Index | [3] | - TBD |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

--------------------------------------------------------End of the Text-------------------------------------------------------