| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Mitigation DG Text Proposal Draft |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Clark Chen, Talwar, Yi Hsuan, Yang-seok Choi, Jong-Kae Fwu, Hujun Yin Intel Corporation Zhaohua Lu, Kun Liu, Yanfeng Guan, Ying Liu; ZTE Corporation Kiran Kuchi, Klutto Milleth CWiT I-Kang, Paul Cheng MTK Zheng Yan-Xiu, Richard Li ITRI HanGyu Cho LGE Jeongho Park Samsung | clark.chen@intel.com |
| Re: | Interference Mitigation AWD Text |  |
| Abstract | FFR UL/UL Signaling and Operation Procedure |  |
| Purpose | To be adopted in AWD text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Mitigation DG Text Proposal

Clark Chen, Shilpa Talwar, Yi Hsuan, Yang-seok Choi, Jong-Kae Fwu, Hujun Yin

Intel Corporation

Zhaohua Lu, Kun Liu, Yanfeng Guan, Ying Liu

ZTE Corporation

Kiran Kuchi, Klutto Milleth

CWiT

I-Kang, Paul Cheng

MTK

Zheng Yan-Xiu, Richard Li

ITRI

HanGyu Cho

LGE

- Proposed Text Input to P802.16m Amendment Working Document

The contributors would like to add/modify the following text in addition to IM DG consensus text.

-------------------------------------------------------Start of the Text-------------------------------------------------------

### 15.2.X.1.1.1 DL/UL Siganling

Insert the following bracketed text after the second paragraph:

[The following text should be consistent with section 15.3.5.2.]

Essential system parameters and system configuration information is carried in the S-SFH, and is categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted with different timing and periodicity.

A

In addition to FFR configuration information, S-SFH SPx IE or ABI (TBD) includes resource metric information needed for AMS to select DL frequency partition in case of distributed permutation, or to select subchannel in case of localized Permutation, as indicated in Table y. The period to transmit S-SFH SPx IE or ABI is N superframes, where value of N is TBD.

Table y S-SFH SPx IE or ABI

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Contents format () { |  |  |
| … |  |  |
| If (FPCT >= 3){ |  |  |
| for (i=0; i < 2; i++) { |  |  |
| Resource_Metric | [8] (TBD) | Resource metric for two power de-boosted frequency partitions in reuse 3 frequency region. Resource metric of the partition with lower partition index is sent first, followed by the resource metric of the other partition. Example: Partition(0) is reuse-1 partition, partition(1) is reuse-3 with power boosting, partitions(2) and partition(3) reuse-3 with power de-boosting; the resource metric of partition(2) will be sent first, and partition(3) will be sent after that. The resource metric is defined as a fractional number between 0 and 1. It is encoded as unsigned digital as following with resolution 1/ 256. Resource metric of FFR partition(0) with reuse-1 is fixed as ‘1’; Resource metric of FFR partition(i) with power boosting is calculated as following: resource metric(i) = 3 – sum(resource metric of partition with power de-boosting); |
| } |  |  |
| … |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

[This section should be consistent with section 15.3.6.5.2.9.]

To allow CQI report for FFR partition other than active partition, wideband CQI feedback of the alternate partition is multiplexed with CQI feedback of the AMS’ current active partition. The period of this alternate CQI is sent every N frames, where N is TBD.

ABS use the CQICH allocation A-MAP IE to control AMS CQI reporting. ABS may send a new CQICH allocation A-MAP IE to reconfigure the CQI reporting after receiving Frequency Partition Switch Indicator (FPSI).

[This section should be consistent with CQICH allocation A-MAP IE, table 682]

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { | - | - |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE = 0b0010 |
| … | … | … |
| FPI | [2] | Frequency Partition Indication: ABS indicate AMS to send CQI of the Frequency Partition and Reuse Factor in the future: - 0b00: Frequency Partition index 0 - 0b01: Frequency Partition index 1 - 0b10: Frequency Partition index 2 - 0b11: Frequency Partition index 3 - FFR Frequency Partition index is specifieed in section x.x.x.x.x. |
| … | … | … |

[This section should be consistent with section 15.3.9.3.1.]

A Frequency Partition Selection Indicator (FPSI) is required for MS using Distributed Permutation to feedback preferred partition. The FPSI is sent in P-CQICH channel, using 4 codewords in the 6-bit CQICH code space. The FPSI is defined as following:

| 58 | 0b111010 | Select FFR partition 1 (reuse-1) | PFBCH | The feedback CQI afterward is for this FFR partition. |
| --- | --- | --- | --- | --- |
| 59 | 0b111011 | Select Frequency partition 2 (reuse-3) | PFBCH | Same as above |
| 60 | 0b111100 | Select Frequency partition 3 (reuse-3) | PFBCH | Same as above |
| 61 | 0b111101 | Select Frequency partition 4 (reuse-3) | PFBCH | Same as above |

For AMS using localized permutation, it reports the preferred sub-channels and their corresponding CQI in CQICH channel according to the signaling defined in section 15.3.9.3.x.  The selection of preferred partition in case of distributed permutation or of preferred subchannels in case of contiguous permutation is described below.

]

### 15.2.X.1.1.2 Operation procedure

Insert the following bracketed text as follow:

# 15.2.X.1.1.2.1 ABS Broadcast and Update of DL FFR information

FFR partition information is broadcasted in SFH every supper frame, and the resource metric of is broadcasted every N super frames, where N is TBD.

Figure. yyy

# 15.2.X.1.1.2.2 AMS feedback recommend FFR partition

In case of Distributed permutation, AMS send Frequency Partition Selection Indicator (FPSI) to ABS to indicate whether it need to switch to a different FFR partition or it will stay in the current FFR partition.

ABS may send a new CQICH allocation IE to response to AMS’ FPSI. After AMS receive a CQICH allocation IE, AMS should send CQI report according to the new configuration.

Two examples of above operation procedure are illustrated in the following figures:

Figure xxx: AMS send FPSI and ABS agrees to change FFR partition Example

Figure yyy: AMS send FPSI and ABS refuses to change FFR partition

# 15.2.X.1.1.2.3 Initial FFR Partition for AMS at system entry

When FFR is applied in system, AMS use FFR partition indicated by ABS. Only after AMS has received one superframe which has the resource metric information, it will start to use the resource metric to recommend its preferred partition to ABS in case of DRU, or select the preferred sub-channels in case of LRU, as defined in previous section.

# 15.2.X.1.1.2.3 AMS FFR Partition Selection in case of Distributed Permutation

AMS select the preferred FFR partition as following:

AMS estimates the average SINR on each FFR partition, and then estimates the MCS Rate according to Link Adaptation procedure. The Estimated MCS Rate and PER, is used to calculate the expected SE on the next frame as follow:

Expected_SE = MCS_Rate * (1-PER);

AMS then calculates the normalized SE of each partition as follows:

Normalized_SE(i) = Expected_SE(i) / Resource_Metric(i);

AMS compares the Normalized_SE(i) of each partition, and selects the Partition with maximum Normalized_SE(i). The Normalized SE(i) can be smoothed (filtered) to avoid rapid changes in partition selection.  If this Normalized_SE(i) is of a different FFR partition than current partition choice, and it is x% (FFS) higher than current partition’s normalized SE, AMS should report a switch of FFR partition and report CQI in CQICH channel according to the signaling defined in section 15.3.9.3.1.

# 15.2.X.1.1.2.4 AMS FFR Partition selection in case of Localized Permutation

[Note: this section will depend on whether CRU is allowed in FFR reuse-3 partitions.]

When AMS is using LRU permutation, and Best-M feedback is enabled, it indicates to BS its preferred sub-channels for downlink transmission. BS can allocate DL resources in this preferred sub-channel, and also can allocate DL transmission in other sub-channel as needed.

AMS selects the preferred sub-channels as following:

AMS estimates the average SINR on each sub-channel of all FFR partitions, and then estimates the MCS Rate according to Link Adaptation procedure. The Estimated MCS Rate and PER of each sub-channel, is used to calculate the expected SE on the next frame as follow:

Expected_SE = MCS_Rate * (1-PER);

AMS then calculates the normalized SE of each sub-channel as follow:

Normalized_SE(m) = Expected_SE(m) / Resource_Metric(i);

Here, ‘i’ is the FFR partition index of the corresponding sub-channel ‘m’.  AMS then selects the sub-channel with maximum Normalized_SE(m).

]

--------------------------------------------------------End of the Text-------------------------------------------------------