| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for the physical structure of ranging channel |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Hwasun Yoo, Si-Hyun Park, Jaehee Cho, Heewon Kang Samsung Electronics | Voice: +82-31-279-4983 E-mail: hwasun.yoo@samsung.com |
| Re: | IEEE 80216m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | In this contribution we propose the text for the ranging channel’s physical structure in IEEE 802.16m amendment working document IEEE 80216m-09/0010r2. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for the Physical structure of Ranging Channel

Hwasun Yoo, Si-Hyun Park, Jaehee Cho, Heewon Kang

Samsung Electronics

# Introduction

This contribution provides the proposed text regarding the physical structure of ranging channel for IEEE 802.16m amendment working document [1].

# Reference

[1] IEEE 802.16m-08/0010r2, “IEEE 802.16m Amendment Working Document,” June 2009

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements,” January 2009

# Text Proposal

------------------------------------------------Start of suggested text #1---------------------------------------------------------

15.3.9.1.4 Ranging channel

15.3.9.1.4.1 Ranging channel structure for non-synchronized AMSs

Figure 500 illustrates the ranging channel structures in the time domain.

(a) Structure 1

(b) Structure 2

(c) Structure 3

(d) Structure 4

Table 721 contains ranging channel formats and parameters.

Table 721-Ranging Channel Formats and Parameters.

| Format No. | Ranging Channel Format | TRCP | TRP | ∆fRP |
| --- | --- | --- | --- | --- |
| 0 | Structure 1 | Tg + k x Tb(a) | 2 x Tb | Δf/2 |
| 1 | Structure 3 |  |  |  |
| 2 | Structure 2 | m x Tg + n x Tb(b) | 2 x 2 x Tb(c) |  |
| 3 | Structure 3 | 7 x Tg + Tb | 8 x Tb | Δf/8 |
| 4 | Structure 4 | Tg | Tb | Δf |

------------------------------------------------End of suggested text #1----------------------------------------------------------

------------------------------------------------Start of suggested text #2---------------------------------------------------------

15.3.9.1.4 Ranging channel

15.3.9.1.4.1 Ranging channel structure for non-synchronized AMSs

In the ranging channel Format 0, the repeated RCPs and RPs are used as a single time ranging opportunity within a subframe in Figure 500 (a). Format 2 consists of a single RCP and repeated RPs within a subframe. Format 1 consists of a single RCP and RP which is a part of the Format 0. When Format 1 is used, there are two time opportunities within a subframe. Format 3 has the same structure with Format 1 but its length is different. RP1, RP2, RP3, and RP4 in Format 4 are constructed by the first portion, second portion, third portion, and the last portion of the ranging sequences [the detail is TBD depending on the ranging sequences], respectively.

When the ranging channel format is configured as Format 0, 2, 3, 4, or Format 1 using the first time-opportunity in the time domain, the transmission start time of the ranging channel is aligned with the UL subframe start time at the AMS. For the Format 1 using the second time-opportunity, the transmission of the ranging channel starts at TRCP+TRP in Format 1 after the start time of first time-opportunity.

------------------------------------------------End of suggested text #2----------------------------------------------------------