| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Mitigation DG Text Proposal Draft – Multi-BS Bracketed Text : Part3 |  |
| Date Submitted | 2009-07-02 |  |
| Source(s) | Clark Chen Intel Corporation Jeongho Park Samsung Electronics Co. Ltd. | clark.chen@intel.com Jeongho.jh.park@samsung.com |
| Re: | Interference Mitigation DG |  |
| Abstract | Multi-BS MIMO DL/UL Signaling (Bracketed Text) |  |
| Purpose | To be adopted in AWD text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Mitigation DG Text Proposal

- Proposed Text Input to P802.16m Amendment Working Document

Chairman note: The following bracketed text has gained more than 50% support in IM DG company based straw-poll but less than 75% support. According to IM DG voting rule, the text is bracketed and submitted to TGm to be voted for accept in AWD.

-------------------------------------------------------Start of the Text-------------------------------------------------------

{Editor’s Note : Replace section 15.2.x.3 of IMDG baseline document by the following text}

15.2.X.3 DL Multi-BS MIMO

Multi-BS MIMO techniques improve sector throughput and cell-edge throughput through multi-BS cooperative signaling. These include single-BS precoding with multi-BS coordination and multi-BS joint processing.

Multi-BS MIMO techniques refer to a set of precoding techniques that involves more than one ABSs. In these techniques, a high-speed backbone network is used to deliver the necessary information between the ABSs. Depending on whether the multi-BS channel information is used in the design (or selection) of the precoding matrices, the precoding techniques are classified into two classes: i. e., single BS precoding techniques with multi-BS coordination, and multi-BS joint precoding techniques. Here, the multi-BS channel information refers to the aggregate channel information of the system comprised of multiple ABSs and their corresponding AMSs. These multi-BS techniques can be used to increase the diversity gain of the wireless links for the AMSs and/or to coordinate the interference between different cells.

{Editor’s Note : Insert the following paragraph and Table X in section 15.2.x.3.1 in addition to current  section 15.2.x.3.1 at the last part of the section }

15.2.X.3.1 DL/UL Signaling

In a multi-BS joint processing mode(i.e., CL-MD mode or Co-MIMO), an ABS shall inform the corresponding AMS of the set of the ABSs with which the AMS is recommended to set up wireless links. [This allows the system to use a BS selection algorithm that can improve the spectral efficiency of the multi-BS joint processing system with a limited transmit power by selecting two or three ABSs taking into account the given channel condition.] The IDs of the joint processing ABSs should be transmitted from the serving ABS to the AMS by using the following message format.

Table X. Parameters to be transferred from serving ABSs to AMSs.

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| N_ABSs | 3 | N_ABSs denotes the number of cooperating ABSs excluding the serving ABS, which is expressed as a 3-bit binary number. For example, N__ABSs=0b001 means that the serving ABS is going to cooperate with one neighbor ABS for the AMS. |
| BS ID(s) | [TBD] | Note: IDs of the neighboring ABSs |

--------------------------------------------------------End of the Text-------------------------------------------------------