| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification on Ranging Channel Power Control Section (AWD-15.3.9.4.3) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Dong-cheol Kim, Wookbong Lee, Hyunwoo Lee, HanGyu Cho, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7919 E-mail: dongcheol.kim@lge.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” Category: AWD comments / Area: Chapter 15.3.9.4.3 (Ranging Channel Power Control) “Comments on AWD 15.3.9.4.3 Ranging Channel Power Control” 802.16m amendment working document |  |
| Abstract | The contribution proposes the text modification on Power Control to be included in the 802.16m amendment |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification on Ranging Channel Power Control Section (AWD-15.3.9.4.3)

Dong-cheol Kim, Wookbong Lee, Hyunwoo Lee

HanGyu Cho, Jin Sam Kwak

LG Electronics

# Introduction

It seems that there are lots of unnecessary parameters for ranging channel power control in the current AWD, which increases signaling overhead. This contribution suggests to delete some parameters regarding ranging power control to reduce signaling overhead.

# Text proposal for the 802.16m amendment

Proposed Remedy

Modify 15.3.9.4.3 Uplink Ranging Channel Power Control section as follows;

============================== Start of Proposed Text =================================

…

15.3.9.4.3 Ranging Channel Power Control

For initial ranging, AMS sends initial ranging code at a random selected ranging channel. The initial transmission power is decided according to measured RSS. If AMS does not receive a response, AMS may send a new initial ranging code and increase its Tx tone power level by . AMS may increase its transmit tone power by PIR,Step and may send a new initial ranging code where PIR,Step is the step size to ramp up which is 2dB (TBD). AMS could further increase the power until maximum transmit power reached.

The initial transmission power of MS is calculated as :

PTX_IR_MIN = EIRxPIR,min + BS_EIRP – RSS, where EIRxPIR,min is the minimum targeting receiving power and BS_EIRP is the transmission power of the BS, which are obtained from S-SFH SP2. And RSS is the measured receiving signaling strength by AMS.

The maximum transmit power for initial ranging is calculated as:

PTX_IR_MAX = EIRxPIR,max + BS_EIRP – RSS, where EIRxPIR,max and BS_EIRP are obtained from ABS through decoding of the S-SFH IE SP3, see <<table 1>>.

EIRxPIR,max is the maximal targeting receiving power for initial ranging code.

BS_EIRP is the transmission power of the BS.

In the case that the Rx and Tx gain of the AMS antenna are different, the AMS shall use Equation (248):

PTX_IR_MAX = EIRxPIR,max + BS_EIRP – RSS + (GRx_MS – GTx_MS)          (248)

Where

GRx_MS is the antenna gain of AMS RX.

GTx_MS is the antenna gain of AMS TX.

RSS is the measured receiving signaling strength by AMS.

For periodic ranging, once an AMS sends a periodic ranging code and fails to receive RNG-RSP, the AMS may adjust its transmit power for the subsequent periodic ranging codes transmission, step-by-step, up to the maximum transmit power PTx_IR_MAX.

============================== End of Proposed Text =================================