| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed PHY structure of Bandwidth Request Channel for IEEE 802.16m Amendment Text |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Hwasun Yoo, Kaushik Josiam, Min-Ho Jang, Heewon Kang Samsung Electronics Co., Ltd. | +82-31-279-4983 hwasun.yoo@samsung.com |
| Re: | 802.16m amendment working document |  |
| Abstract | The contribution proposes the text of BW REQ channel to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed PHY structure of Bandwidth Request Channel for IEEE 802.16m Amendment Text

Hwasun Yoo, Kaushik Josiam, Min-Ho Jang, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text on the physical layer structure of the Bandwidth Request Channel to be included in the 802.16m amendment ([1]). The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([2]), it is compliant to the 802.16m SRD ([3]) and the 802.16m SDD ([4]), and it follows the style and format guidelines in ([5]).

- References

[1] IEEE 802.16m-09/0010r2, “802.16m Amendment Working Document (AWD)”

[2] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[3] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[4] IEEE 802.16m-08/003r6, “IEEE 802.16m System Description Document”

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

---------------------------------------- Text Proposal #1 Start ---------------------------------------------------

Replace section 15.3.9.1.5, page 286-288 with the following text:

15.3.9.1 Physical Uplink Control Channel

15.3.9.1.5 Bandwidth Request Channel

In the L-Zone with PUSC, a BW REQ tile is defined as gour contiguous subcarriers by six OFDM symbols.  The number of BW REQ tiles per BW REQ channel is three or six. Each BW REQ tile carries a BW REQ access sequence preamble only as shown in Figure 502.

In the M-Zone, a BW REQ tile is defined as six contiguous subcarriers by six OFDM symbols. Each BW REQ channel consists of three distributed BW-REQ tiles. Each BW REQ tile carries a BW REQ accesss sequence preamble and part of a BW REQ message as shown in Figure 501.. The AMS may transmit the access sequence preamble only and leave the resources for the quick access BW REQ message unused.

Figure 501- 6X6 BW REQ Tile Structure in the Advanced Air Interface

Figure 501 6X6 BW REQ Tile Structure in the Advanced Air Interface

A BW REQ tile in the Advanced Air Interface specifications is defined as 6 contiguous subcarriers by 6 OFDM symbols as Figure 501, where  and  indicate BW REQ preamble sequence and BW REQ message, respectively.

As shown in Figure 501, the BW REQ preamble shall be transmitted on a resource that spans 4 subcarriers by 6 OFDM symbols. Additionally a quick access BW REQ message shall may be carried in the data portion of the tile that spans 2 contiguous subcarriers by 6 OFDM symbols.  Each BW REQ channel shall comprise of 3 distributed BW REQ tiles for frequency diversity.  The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

Let denote a total of 16 bits of information to be carried as the quick access message.  4 bits of the 16 information bits shall be carried in the BW REQ preamble using the preamble index.  The combined resource in the data portion of the three tiles that form the BW REQ channel shall be used to transmit the remaining 12 bits of information.  The construction of the BW REQ preamble for a 6x6 tile structure is TBD.

The 12 bits of information in the quick access message transmitted in the BW REQ channel shall be encoded into 72 bits using the 1/6 TBCC code described in <<< section 15.3.9.2.1.2 (Channel coding for secondary fast feedback channel)>> with paramters , and .  The 72 coded bits shall then be QPSK modulated as described in Section TBD and scrambled to generate 36 data symbols, .  The combined data portions of the three distributed BW REQ tiles that form the BW REQ channel shall be used to transmit these data symbols.

In order to support operation in the legacy mode, a BW REQ tile shall be defined as 4 contiguous subcarriers by 6 OFDM symbols.  As shown in Figure 502, only the BW REQ access sequence or BW REQ preamble shall be transmittedin all 24 subcarriers that form the BW REQ tile.  In this case, the shall be randomly selected from all available logical preamble sequences.

---------------------------------------- Text Proposal #1 End ---------------------------------------------------

---------------------------------------- Text Proposal #2 Start ---------------------------------------------------

Replace section 15.3.9.2.5, page 295 with the following text:

15.3.9.2 Uplink Control Channels Physical Resource Mapping

15.3.9.2.5 Bandwidth request channel

Contention based random access is used to transmit bandwidth request BW REQ information on this control channel.  The bandwidth request (BW REQ) channel contains resources for the AMS to send a BW REQ access sequence and an optional quick access message.  Prioritized bandwidth requests are supported on this channel.  The mechanism for such prioritization is TBD.  Both 3-step and 5-step BW REQ procedures described in <<Section 15.2.2 >> are supported on this channel.

When the type of bandwidth requested by an AMS is not one of urgent bandwidth request types defined in <<Section 15.x>>, the AMS randomly chooses a preamble index k with a range from 0 to 23. The selected preamble index is used for generating BW REQ preamble sequence according to Table XXX.

| Index k | BW REQ preamble sequence | Index k | BW REQ preamble sequence |
| --- | --- | --- | --- |
| 0 | 111111111111111111111111 | 12 | 111111111111000000000000 |
| 1 | 101011100010101011100010 | 13 | 101011100010010100011101 |
| 2 | 100101110001100101110001 | 14 | 100101110001011010001110 |
| 3 | 110010111000110010111000 | 15 | 110010111000001101000111 |
| 4 | 101001011100101001011100 | 16 | 101001011100010110100011 |
| 5 | 100100101110100100101110 | 17 | 100100101110011011010001 |
| 6 | 100010010111100010010111 | 18 | 100010010111011101101000 |
| 7 | 110001001011110001001011 | 19 | 110001001011001110110100 |
| 8 | 111000100101111000100101 | 20 | 111000100101000111011010 |
| 9 | 111100010010111100010010 | 21 | 111100010010000011101101 |
| 10 | 101110001001101110001001 | 22 | 101110001001010001110110 |
| 11 | 110111000100110111000100 | 23 | 110111000100001000111011 |

For 3step BW REQ procedure, 15bits of BW request information is constructed from 12bits of STID and 3bits of pre-defined BW size information, which is decribed in Section (15.X). Let  and  denot e the STID and pre-defined BW size information, respectively. By reordering the bits of STID and pre-defined BW size information, 15bits of BW request information is formed as

---------------------------------------- Text Proposal #2 End ---------------------------------------------------