| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Multicarrier PHY for the IEEE 802.16m Amendment [MCDG] |  |
| Date Submitted | 2009-07-11 |  |
| Source(s) | Jaehee Cho, Samsung Electronics Co., Ltd. 416 Maetan-3, Suwon, 442-600, Korea | Voice: +82-31-279-5596 E-mail: jaehee1.cho@samsung.com |
| Re | 80216m-09_0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content |  |
| Abstract | The contribution proposes the text of multi-carrier operation section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Multicarrier guard subcarriers for the IEEE 802.16m Amendment

Jaehee Cho

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes the text of multicarrier to be included in the IEEE 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D7 [1], it is compliant to the 802.16m SRD [2], 802.16m SDD [4] and it follows the style and format guidelines in [3].

- Proposal in Multicarrier

This contribution propose permutation of mini-band for multi-carrier guard subcarriers.

- References

[1] IEEE P802.16 Rev2 / D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”

[3] IEEE C802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[4] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- 15. Advanced Air Interface

- 15.3Physical layer

[Insert the new section as follows]

15.3.5.5 Downlink physical structure for multicarrier support

For multicarrier operation, guard subcarriers between carriers can form additional PRUs. The definition of PRU is the same as defined in 15.3.5.1 and 15.3.5.4. The additional PRUs only can be used as miniband CURs for partition FP0.

15.3.x.6 Downlink Control Structure for Multicarrier Support

All DL controls channel needed for single carrier operation are needed for the fully configured carrier. For partially configured carrier, DL control channels necessary for UL transmission are not present. AMS needs to listen to control channels of all activated carriers.

The allocation information for the burst carried in a active carrier shall be carried through the corresponding carrier using the normal single carrier AMAP message.

15.3.8.5 Uplink physical structure for multicarrier support

For multicarrier operation, guard subcarriers between carriers can form additional PRUs. The definition of PRU is the same as defined in 15.3.8.1 and 15.3.8.4. The additional PRUs only can be used as miniband CURs for partition FP0.

15.3.9.5 Uplink Control Structure for Multicarrier Support

All UL controls channel needed for single carrier operation are supported for the fully configured carrier. A partially configured carrier does not have any uplink capability.

Secondary carrier can assign UL control channels except bandwidth request for AMSs that take the carrier as their active secondary carriers

-------------------------------  Text End  ---------------------------------------------------