| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femtocell BS Network Entry/De-Registration Procedures (AWD-Femto) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Kanchei (Ken) Loa, Chun-Yen Hsu, Chiu-Wen Chen, Youn-Tai Lee Institute for Information Industry (III) Whai-En Chen, Shih-Yuan Cheng National Ilan University (NIU) Shiann-Tsong Sheu, Chih-Cheng Yang National Central University (NCU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw hcy@iii.org.tw |
| Re: | IEEE 802.16m-09/0028r1: Call for Comments and Contributions on Project 802.16m Amendment Content |  |
| Abstract | This contribution proposes text for femtocell BS network entry |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Femtocell BS Network Entry/De-Registration Procedures (AWD-Femto)

Kanchei (Ken) Loa, Chun-Yen Hsu, Chiu-Wen Chen, Youn-Tai Lee, Whai-En Chen, Shih-Yuan Cheng, Shiann-Tsong Sheu, Chih-Cheng Yang
III, NIU, NCU

Introduction

In current SDD [1], a Femtocell BS supports Relay Link transmission may establish the air interface connection with the overlapped Macrocell BS for exchanging control messages. This contribution proposes network entry/de-registration procedures for Femtocell BSs to enter/exit the access network provided by the overlaid Macrocell BS. The proposed Femtocell BS network entry/de-registration procedures are based on the relay station network entry/de-registration procedures stated in [2].

Reference

[1] IEEE 802.16m-08/003r9a, IEEE 802.16m System Description Document.

[2] IEEE 802.16j-2009, Part 16: Air Interface for Broadband Wireless Access Systems Amendment 1: Multiple Relay Specification.

Proposed Changes:

[Insert subclause 17.x as indicated:]

17.x Femtocell BS network entry

For a Femtocell BS that uses air interface connection with the overlapped Macrocell BS for exchanging control messages, the Femtocell BS shall perform Femtocell BS network entry procedure to register itself to the overlaid Macrocell BS and obtain relay link configuration information.

The procedure can be divided into the following phases:

a) Scan for DL channel and establish synchronization with the ABS

a1) Perform the overlaid Macrocell ABS selection (May be omitted if the overlaid Macrocell ABS was determined)

b) Obtain UL parameters

c) Perform ranging

d) Negotiate basic capabilities

e) Authorize Femtocell BS and perform key exchange

f) Perform registration

g) Establish IP connectivity

j) Set up connections (Basic and management service flows only)

k) Configure operation parameters

[Insert subclause 17.x as indicated:]

17.y Femtocell BS De-Registration

A Macrocell ABS may decide to remove the direct link to a Femtocell BS. In this case, the ABS sends DREG-CMD to the Femtocell BS to initiate the Femtocell BS de-registration procedure.

A Femtocell BS may disconnect its air link with the overlaid Macrocell ABS. In this case, the Femtocell BS transmits DREG-REQ to the overlaid Macrocell ABS so that it initiates the de-registration procedure. Upon receiving DREG-REQ, the overlaid Macrocell ABS decides whether it allows the Femtocell BS de-registration and transmits DREG-CMD to inform the decision. If the de-registration is rejected, upon receiving DREG-CMD with rejection information, the Femtocell BS continues normal operation.