| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Emergency Service Notification in Femtocells (AWD-Femto) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Kanchei(Ken) Loa, Chiu-Wen Chen, Chun-Yen Hsu, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) Whai-En Chen, Shih-Yuan Cheng National Ilan University (NIU) Chih-Cheng Yang National Central University (NCU) | Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | IEEE 802.16m-09/0028r1: Call for Comments and Contributions on Project 802.16m Amendment Content; AWD-Femto |  |
| Abstract | This contribution proposes text for emergency notification in Femtocells |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Emergency Service Notification in Femtocells (AWD-Femto)

Kanchei(Ken) Loa, Chiu-Wen Chen, Chun-Yen Hsu, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai
Institute for Information Industry

Whai-En Chen, Shih-Yuan Cheng
National Ilan University (NIU)

Chih-Cheng Yang
National Central University (NCU)

Introduction

Support for emergency service is a critical requirement in the 802.16m SRD [1]. For handling Emergency Telecommunications Service and E-911, emergency service flow will be given priority in admission control over the regular service flow. Service flow parameters for emergency service may be pre-defined in the system. Emergency service notification may be transmitted by the pre-defined emergency preamble or by the emergency service indication either in the AAI_RNG-REQ message or in the bandwidth request message to initiate the emergency service flow setup [2]. In this contribution, we propose to initiate emergency service flow setup using quick access message.

In Femtocells, all types of ranging purposes are accommodated in a unified BR ranging channel. The PHY structure of BR ranging channel is identical as Macro BS. Each AMS may transmit the pre-defined emergency preamble with the emergency indicator inside the quick access message for emergency service flow setup. The design of quick ranging structure in Femtocells could reference by the relative contributions (e.g., C80216m-09_1561). The AMS can request the emergency service flow setup during initial ranging process or connected state. During initial ranging process, the AMS may initiate emergency service flow setup by sending the quick access message with emergency service indication as shown in Figure 1.

Figure 1  Emergency service notification during initial ranging process

In connected state, to initiate emergency service flow, the AMS may send the pre-defined ranging preamble for emergency service flow or a quick access message with the emergency service notification being set to ‘1’ as shown in Figure 2.

Figure 2  Emergency service notification in connected state

Proposed text for the 802.16m AWD:

----------------------------------------------------Start of the Text---------------------------------------------------------

[Insert the following new subcluase into Section 15.x in the 802.16m-09/0010r2]

#### 15.x  Support for Femtocell

##### 15.x.y  Emergency service support

The emergency service flow setup procedure is identical with that in Macro BS unless stated otherwise in this section.

##### 15.x.y.1  Emergency service notification

Emergency service notification is used for AMSs to initiate emergency service flow setup. During initial ranging, the AMS may send the quick access message with emergency service indication being set to ‘1’ to initiate emergency service flow. In connected state, to initiate emergency service flow, the AMS may send the pre-defined ranging preamble for emergency service flow or a quick access message with the emergency service notification being set to ‘1’.

----------------------------------------------------End of the Text---------------------------------------------------------

[1] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements”

[2] IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document”