| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized AWD Text for Control Plane Signaling Protection |  |
| Date Submitted | 2009-07-11 |  |
| Source(s) | GeneBeck Hahn, KiSeon Ryu and Ronny YongHo Kim LG Electronic Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea Youngkyo Baek Samsung Electronics | Voice: +82-31-450-7188 E-mail: gbhahn@lge.com, ksryu@lge.com and ronnykim@lge.com E-mail: youngkyo.baek@samsung.com |
| Re: | IEEE 802.16m-09/0028r1. ”AWD call for contribution after session #61” Category: AWD/Area: Chapter 15.2.4 (AWD-Security DG) |  |
| Abstract | This contribution proposes the text of control plane signaling protection section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment working document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for Control Plane Signaling Protection

Gene Beck Hahn, Ki Seon Ryu and Ronny Yong Ho Kim

LG Electronics

# 1. Introduction

This contribution proposes the amendment text for control plane signaling protection of 16m. The proposed text is developed so that it can be combined with 16 Rev2/D9 [1], and it is compliant to 16m SRD [2]. The text proposal is based on the current 16m SDD [3].

# 2. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-09/0010R2, “IEEE 802.16m Amendment Working Document”

# 3. Text Proposal for Control Plane Signaling Protection Section of IEEE 802.16m Amendment

============================= Start of Proposed Text =============================

# 15.2.4.4.2 Control Plane Signaling Protection

802.16m supports the confidentiality protection over MAC management messages. Specifically, encryption is selectively applied to the management messages whenever required by the selected ciphersuite. The encrypted unicast management messages shall be mapped to the primary SA. The selective confidentiality protection over management messages is the mandatory feature of 16m and the negotiated keying materials/ciphersuite are used to encrypt the management messages.

The indication required for selective confidentiality protection over management messages is contained in the security extended header. Contrary to the transport flows where the established SA is applied to all data, the SA is selectively applied to the management flows. Security extended header is used only for management flows to indicate whether PDU contains the management message that is encrypted based on management message type and its usage. In particular, whether management message is encrypted or not is decided on the security level with which the message is associated.

The selective protection over MAC management messages is made possible after the successful completion of local TEK derivation.

Figure 1 shows the three levels of protection over management messages.

- No protection: If AMS and ABS have no shared security contexts or protection is not required, then the management messages are neither encrypted nor authenticated. The management messages before the authorization phase also fall into this category.

- CMAC based integrity protection; CMAC Tuple is included as the last attribute of MAC management message. CMAC protects the integrity of entire management messages. Actual management message is plaintext

- AES-CCM based authenticated encryption; ICV part of the encrypted MPDU is used for the integrity protection about the payload of management messages as well as GMH.

Figure 1: Flow of 802.16m Selective Management Message Protection

============================= End of Proposed Text =============================