| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text for the DL/UL PA A-MAP IEs |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim}@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes updates to the text related to persistent allocation in the 802.16m AWD |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for the DL/UL PA A-MAP IEs

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jin Sam Kwak

LG Electronics.

Introduction

This contribution proposes texts to be included in the section 15.3.6.5.2.6 DL PA A-MAP IE and the section 15.3.6.5.2.7 DL PA A-MAP IE of the 802.16m amendment working document. Remove the persistent flag and the related fields in DL/UL PA A-MAP IE, since DL/UL PA A-MAP IEs are used only for initial resource allocation of persistent DL/UL traffics. In Composite persistent A-MAP IE, 2 bits-Resource Allocation Indicator should be used to reduce the MAP overhead.

References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, March 2009.

[2] C802.16m-09/1136, “Proposed Text for the MAC Procedure for DL and UL Persistent Allocation in the IEEE 802.16m Amendment”, May 2009.

[3] C802.16m-09/926, “Proposed Text for the MAC Procedure for Persistent Allocation in the IEEE 802.16m Amendment”, April 2009.

Text proposal for inclusion in 802.16m amendment

------------------------------------------- Start of Proposed Text -----------------------------------------------

### 15.3.6.4.2.6 DL PA A-MAP IE

The DL persistent A-MAP IE is specified in Table 675.

Table 675: DL Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Persistent A-MAP IE |
| if MCRC is masked with Station ID { |  |  |
| DL Individual Persistent A-MAP_IE() |  | Refer to Table 665 |
| } else if MCRC is masked with Composite ID { |  |  |
| DL Composite Persistent A-MAP_IE() |  | Refer to Table 666 |
| } |  |  |
| MCRC | [16] | 16 bit CRC masked by Station ID for individual PA and masked with composite ID (well-known ID specified in the system, TBD) for multi-user PA. |
| } |  |  |

### 15.3.6.4.2.6.1 DL Individual PA A-MAP IE

The DL individual persistent A-MAP IE is specified in Table 676.

Table 676: DL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| If (Allocation Period==0b00){ |  |  |
| A-MAP HFA | [4] | TBD HARQ Feedback Allocation for this A-MAP |
| } else if (Allocation Period != 0b00){ |  |  |
| MCS | 4 | Depends on supported modes, 16 modes assumed as baseline |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| } else{ |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| } |  |  |
| } |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier 0b00: 2 0b01: 3 0b10: 4 0b11: 5 |
| } |  |  |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

### 15.3.6.4.2.6.2 DL Composite PA A-MAP IE

The DL composite persistent A-MAP IE is specified in Table 677.

Table 677: DL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits – depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) Allocation size |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| A-MAP HFA | [4] | HARQ Feedback Allocation for this A-MAP |
| } else if (Allocation Period != 0b00){ |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| MCS | 4 | Depends on supported modes, 16 modes assumed as baseline |
| } |  |  |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| } else{ |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| } |  |  |
| } |  |  |
| Resource Allocation Indicator | 1 | If Resource Allocation Indicator is 1, it indicates that Duration is explicitly assigned for this subburst (allocation or deallocation). Otherwise, this subburst (allocation or deallocation) will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| if (Resource Allocation Indicator ==1) { |  |  |
| RAI | 2 | Resource Allocation Indicator (RAI) 0b00: It indicates that resource allocation information is explicitly assigned for this subburst. 0b01: It indicates that resource offset is explicitly assigned for this subburst and this subburst will use the same duration as the previous subburst. 0b10: It indicates that this subburst will use the same duration as the previous subburst and follow the previous subburst. 0b11: Rsvd If j is 1 then this indicator shall be 0b00. |
| if (RAI ==0b00) { |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| } else if (RAI == 0b01) { |  |  |
| Resource offset | TBD | Variable number of bits - depends on system bandwidth. It indicates the start position of resource region for this subburst |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits – depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) Allocation size |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period ACID as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity | 2 | Periodicity of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier 0b00: 2 0b01: 3 0b10: 4 0b11: 5 |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } //ACID indicator |  |  |
| } |  |  |
| }// allocation period !=0b00 |  |  |
| } // for |  |  |
| } |  |  |

### 15.3.6.4.2.7 UL PA A-MAP IE

The UL persistent A-MAP IE is specified in Table 678.

Table 678: UL Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Persistent A-MAP IE |
| if MCRC is masked with Station ID { |  |  |
| UL Individual Persistent A-MAP_IE() |  | Refer to Table 668 |
| } else if MCRC is masked with Composite ID { |  |  |
| UL Composite Persistent A-MAP_IE() |  | Refer to Table 669 |
| } |  |  |
| MCRC | [16] | 16 bit CRC masked by Station ID for individual PA and masked with composite ID (well-known ID specified in the system, TBD) for multi-user PA. |
| } |  |  |

### 15.3.6.4.2.7.1 UL Individual PA A-MAP IE

The UL individual persistent A-MAP IE is specified in Table 679.

Table 679: UL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of persistent resource. 0b00: 0 frame 0b01: 2 frames 0b10: 4 frames 0b11: 6 frames |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| } |  |  |
| else if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| If (Allocation Period==0b00){ |  |  |
| A-MAP HFA | [4] | TBD HARQ Feedback Allocation for this A-MAP |
| } else if (Allocation Period != 0b00){ |  |  |
| MCS | 4 | MCS used for burst |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| CSM | 1 | - 0b0: CSM enabled 0b1: No CSM enabled |
| If(MEF == 0b1){ |  | Paraneters for vertical encoding |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ) 0b00: 1 stream 0b11: 2 streams 0b00: 3 stream 0b11: 4 streams |
| if(CSM == 0b1){ |  |  |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 0b01){ |  |  |
| SI | - 1 | - First pilot index for CSM with TNS = 2 |
| } else if( TNS == 0b10 or TNS==0b11){ |  |  |
| SI | - 2 | - First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| } |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation for subburst |
| ACID | 3 | HARQ channel identifier |
| N_ACID | 2 | Number of ACID for implicit cycling of HARQ channel identifier 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| } |  |  |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| } | - | - |

### 15.3.6.4.2.7.2 UL Composite PA A-MAP IE

The UL composite persistent A-MAP IE is specified in Table 680.

Table 680: UL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits – depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) Allocation size |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| A-MAP HFA | [4] | HARQ Feedback Allocation for this A-MAP |
| } else if (Allocation Period != 0b00){ |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| MCS | 4 | Depends on supported modes, 16 modes assumed as baseline |
| } |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC 0b1: Vertical encoding |
| CSM | 1 | - 0b0: CSM enabled 0b1: No CSM enabled |
| if(MEF == 0b1){ |  | Paraneters for vertical encoding |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ) 0b00: 1 stream 0b11: 2 streams 0b00: 3 stream 0b11: 4 streams |
| if(CSM == 0b1){ |  |  |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 0b01){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else if( TNS == 0b10 or TNS==0b11){ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| Resource Allocation Indicator | 1 | If Resource Allocation Indicator is 1, it indicates that Duration is explicitly assigned for this subburst (allocation or deallocation). Otherwise, this subburst (allocation or deallocation) will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| if (Resource Allocation Indicator ==1) { |  |  |
| RAI | 2 | Resource Allocation Indicator (RAI) 0b00: It indicates that resource allocation information is explicitly assigned for this subburst. 0b01: It indicates that resource offset is explicitly assigned for this subburst and this subburst will use the same duration as the previous subburst. 0b10: It indicates that this subburst will use the same duration as the previous subburst and follow the previous subburst. 0b11: Rsvd If j is 1 then this indicator shall be 0b00. |
| if (RAI ==0b00) { |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| }else if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| } else if (RAI == 0b01) { |  |  |
| Resource offset | TBD | Variable number of bits - depends on system bandwidth. It indicates the start position of resource region for this subburst |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| }else if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits – depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) Allocation size |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period ACID as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity | 2 | Periodicity of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| ACID | 4 | HARQ channel identifier. The ACID field shall be set to the initial value of HARQ channel identifier for implicit cycling of HARQ channel identifiers. |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier 0b00: 2 0b01: 3 0b10: 4 0b11: 5 |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } //ACID indicator |  |  |
| } |  |  |
| }// allocation period !=0b00 |  |  |
| } |  |  |

--------------------------------------------- End of Proposed Text ---------------------------------------------