| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text for the MAC Procedure for Persistent Allocation |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | E-mail: {chegal, sixs, ksryu, ronnykim}@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.2.7 (Persistent Scheduling) “Comments on AWD 15.2.7 Persistent Scheduling” |  |
| Abstract | The contribution proposes text for the MAC procedure for persistent allocation to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for the MAC Procedure for Persistent Allocation

Jeongki Kim, Youngsoo Yuk, Kiseon Ryu, Yongho Ronny Kim, Jin Sam Kwak

LG Electronics.

Introduction

This proposal is to update the Section 15.2.7 Persistent Scheduling in Amendment working document (AWD) [1]. In the case of the uplink persistent scheduling, ABS can know that the UL Persistent A-MAP IE is successfully received by AMS if the ABS detects the UL data burst indentified by UL Persistent A-MAP IE.

References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, May 2009.

[2] C802.16m-09/1135, Proposed Text for the DL and UL Persistent Allocation A-MAP IEs in the IEEE 802.16m AWD, May 2009.

[3] C802.16m-09/925, Proposed Updates to the Persistent Allocation A-MAP IE in the IEEE 802.16m AWD, April 2009.

Text proposal for inclusion in the 802.16m amendment

------------------------------------------- Start of Proposed Text -----------------------------------------------

### 15.2.7.2 Deallocation Mechanism

### 15.2.7.2.1 Deallocation Mechanism for an Individual Connection

For deallocation of individual persistent allocations in the DL/UL, the ABS shall transmit the DL/UL Individual Persistent A-MAP IE. When the Allocation Period is set to 0b00 in the DL/UL Individual Persistent A-MAP IE, the assigned persistent resource in DL/UL Individual Persistent A-MAP IE is deallocated in referenced DL/UL subframe and the ABS and AMS terminates the persistent allocation.

### 15.2.7.2.2 Deallocation Mechanism for Multiple Connections

For deallocation of multiple persistent allocations in the DL/UL, the ABS shall transmit the DL/UL Composite Persistent A-MAP IE. When the Allocation Period is set to 0b00 for each connection that is being deallocated in the DL/UL Composite Persistent A-MAP IE, the assigned persistent resource in DL/UL Composite Persistent A-MAP IE is deallocated in referenced DL/UL subframe and the ABS and AMS terminates the persistent allocation.

### 15.2.7.3 HARQ Retransmissions

Asynchronous HARQ retransmission is used for downlink individual and composite persistent allocations. The DL Basic Assignment A-MAP IE shall be is transmitted to signal control information for HARQ retransmission. Synchronous HARQ retransmission is used for uplink individual and composite persistent allocations. The UL Basic Assignment A-MAP IE may be is transmitted to signal control information for HARQ retransmission.

### 15.2.7.4 Error Handling Procedure

### 15.2.7.4.1 Error Handling Procedure for an Individual Connection

For transmissions with HARQ enabled, an ACK is transmitted to acknowledge the successful decoding of a data burst, or a NACK is transmitted to notify failure in decoding a burst transmitted on the DL/UL. If an ACK or a NACK for the initial data burst identified by the DL/UL Individual Persistent A-MAP IE is detected in the assigned HARQ Feedback channel, the ABS shall assume that the DL/UL Individual Persistent A-MAP IE is correctly received. If an ACK or a NACK for the data burst identified by the DL Individual Persistent A-MAP IE is detected in the assigned HARQ Feedback channel, the ABS shall assume that the DL Individual Persistent A-MAP IE is correctly received by AMS. If the UL data burst identified by the UL Individual Persistent A-MAP IE is detected, the ABS shall assume that the UL Individual Persistent A-MAP IE is correctly received by AMS.

In the absence (NULL detection) of an ACK or a NACK in the HARQ feedback channel assigned in the DL/UL Individual Persistent A-MAP IE, the ABS shall assume that the AMS has not received the DL/UL Individual Persistent A-MAP IE and the same DL/UL Persistent A-MAP IE can be transmitted again.

In the absence (NULL detection) of an ACK or a NACK in the HARQ feedback channel assigned in the DL Individual Persistent A-MAP IE, the ABS shall assume that the AMS has not received the DL Individual Persistent A-MAP IE and the ABS may transmit the same DL Persistent A-MAP IE again. In the absence (NULL detection) of the UL data busrt assigned in the UL Individual Persistent A-MAP IE, the ABS shall assume that the AMS has not received the UL Individual Persistent A-MAP IE and the ABS may transmit the same UL Persistent A-MAP IE again.

In the case of deallocation of individual persistent allocations in the DL/UL, the ABS shall transmit a HARQ Feedback Allocation in the DL/UL Individual Persistent A-MAP IE. This allocation is used to identify the HARQ channel in which the ACK for the DL/UL Individual Persistent A-MAP IE signaling the deallocation is transmitted. In the absence (NULL detection) of an ACK, the ABS shall assume that the AMS has not received  the DL/UL Individual Persistent A-MAP IE, and the same DL/UL Persistent A-MAP IE that signaled the deallocation can be transmitted again.

### 15.2.7.4.2 Error Handling Procedure for Multiple Connections

For transmissions with HARQ enabled, an ACK is transmitted to acknowledge the successful decoding of a data burst, or a NACK is transmitted to notify failure in decoding a burst transmitted on the DL/UL. If an ACK or a NACK for all the initial data bursts identified by the DL/UL Compoosite Persistent A-MAP IE is detected in the assigned HARQ Feedback channel, the ABS shall assume that the DL/UL Composite Persistent A-MAP IE is correctly received by all the connections. If an ACK or a NACK for the data burst identified by the DL Composite Persistent A-MAP IE is detected in the assigned HARQ Feedback channel, the ABS shall assume that the DL Composite Persistent A-MAP IE is correctly received by AMS. If the UL data burst identified by the UL Composite Persistent A-MAP IE is detected, the ABS shall assume that the UL Composite Persistent A-MAP IE is correctly received by AMS.

In the absence (NULL detection) of an ACK or a NACK in the HARQ feedback channel assigned to one or more connections in the DL/UL Composite Persistent A-MAP IE, the ABS shall assume that the corresponding AMSs have not received the DL/UL Composite Persistent A-MAP IE and the DL/UL Composite A-MAP IE can be transmitted again for these connections. If the persistent allocation needs to be transmitted to only one connection the DL/UL Individual A-MAP IE can be transmitted again for this connection. In the absence (NULL detection) of an ACK or a NACK in the HARQ feedback channel assigned in the DL Composite Persistent A-MAP IE, the ABS shall assume that the AMS has not received the DL Composite Persistent A-MAP IE and the ABS may transmit the same DL Persistent A-MAP IE again. In the absence (NULL detection) of the UL data busrt assigned in the UL Composite Persistent A-MAP IE, the ABS shall assume that the AMS has not received the UL Composite Persistent A-MAP IE and the ABS may transmit the same UL Persistent A-MAP IE again.

In the case of deallocation of multiple persistent allocations in the DL/UL, the ABS shall transmit HARQ Feedback Allocations in the DL/UL Composite Persistent A-MAP IE. These allocations are used to identify the HARQ channels in which the ACK for the DL/UL Composite Persistent A-MAP IE signaling the deallocations are transmitted. In the absence (NULL detection) of an ACK from one or more connections, the ABS shall assume that the corresponding AMSs have not received the DL/UL Composite Persistent A-MAP IE, and the same DL/UL Composite A-MAP IE that signaled the deallocation canmay be transmitted again. If the deallocation needs to be transmitted to only one connection the DL/UL Individual A-MAP IE can be transmitted again for this connection.

--------------------------------------------- End of Proposed Text ----------------------------------------------