| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Load Balance between Overlay Macrocell and Femtocell (AWD-Femto) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Keiichi Nakatsugawa, Masato Okuda, Wei-Peng Chen Fujitsu | E-mail: nakatsugawa@jp.fujitsu.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content”. Target call: Femtocell |  |
| Abstract | Propose to support the load balance operation between overlay macrocell and femtocell. |  |
| Purpose | To be discussed and adopted in 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Load Balance between Overlay Macrocell and Femtocell (AWD-Femto)

Keiichi Nakatsugawa, Masato Okuda, Wei-Peng Chen

Fujitsu

# Introduction

Traffic monitoring and the low-duty operation mode for Femtocell ABS are supported in the current SDD [1]. In this contribution, we propose the text to support the load balance operation between overlay macrocell and femtocell in AWD.

# Proposed Text

[Insert a new subclause 15.x.x as follows]

============================== Start of the Text=================================

# 15.x.x Load Balance between Overlay Macrocell and Femtocell

To alleviate the overloaded traffic in the macrocells, an overlay macro ABS may redirect part of the traffic load to the Femtocell ABSs under its overlay area. A Femtocell ABS that has the light traffic load may enter low-duty operation mode and monitor DL and UL signal associated with an AMS which is served by overlay macro ABS during the unavailable interval. The monitoring may be initiated by overlay macro ABS or by the Femtocell ABS itself to detect the existence of the AMS in the coverage of the Femtocell ABS. Then the Femtocell ABS can inform the macro ABS over the backhaul that the AMS is in its coverage and subsequently handover to Femtocell ABS can be accomplished.

============================== End of Proposed Text =================================

# References

[1] IEEE 802.16m-09/003r9a, “IEEE 802.16m System Description Document (SDD)”