| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ feedback IE for ARQ sub-block (AWD-section 15.2.13.1.1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics | E-mail: yb.chang@samsung.com Phone: +82-31-279-5519 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m AWD Comment” IEEE 802.16m-09/0028r1, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. |  |
| Abstract | This contribution mainly proposes ARQ feedback IE for ARQ sub-block and clean up the ARQ feedback IE for ARQ block in AWD document |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ feedback IE for ARQ sub-block

Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

This contribution includes two parts.

1. ARQ feedback IE for ARQ sub-block should be defined to support current ARQ section in AWD.

2. ARQ feedback IE and text clarification for ARQ feedback IE for ARQ sub-block.

# Text proposal

============================= Start of Proposed Text ========================

[Modify the text from line 29 to 34 on page 73 as follows:]

15.2.13.1.1 ARQ feedback IE

Table 40— defines the ARQ Feedback IE for ARQ block. Table xxx defines the ARQ feedback IE for ARQ sub-blocks. It is they are used by the receiver to signal positive or negative acknowledgments. A set of ARQ feedback IEs of this format in the table 40 and xxx may be transported either as a part of extended header packed payload ("piggybacked") within a packed MAC PDU or a standalone MAC management message as a payload of a standalone MAC PDU.

Table 40- ARQ feedback IE format for ARQ block

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST){ | variable |  |
| TYPE | 1 | 0 = ARQ feedback IE for ARQ block 1 = ARQ feedback IE for ARQ sub-block This bit is always set to ‘0’ |
| Flow ID LAST | 41 | The ID of the flow being referenced 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| LAST Flow ID | 14 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list The ID of the flow being referenced |
| FLAG | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SN | 10 | FLAG = 0, ARQ block up to and including SN has been received successfully. FLAG = 1, ARQ block less than SN has been received successfully. |
| if(FLAG == 1) { |  |  |
| Selective ACK MAP | 7 | Each bit represents ACK or NAK of corresponding ARQ block. ‘0’ is NAK and ‘1’ is ACK. First MSB of first ACK MAP represents ACK or NAK information of SN. Contiguous bits after first MSB of first ACK MAP is corresponding to contiguous SN. |
| FLAG | 1 | 0 = No more selective ACK MAP 1 = Another set of selective ACK MAP and FLAG follows |
| } |  |  |
| Reserved | variable |  |
| } |  |  |

[Insert the table at line 14 on page 74 as follows:]

Table xxx – ARQ feedback IE format for ARQ sub-block

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE2(LAST){ | variable |  |
| TYPE | 1 | 0 = ARQ feedback IE for ARQ block 1 = ARQ feedback IE for ARQ sub-block This bit is always set to ‘1’ |
| LAST | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| Flow ID | 4 | The ID of the flow being referenced |
| Num_SN | 4 | Number of ARQ blocks having error an ARQ sub-block |
| for(i=0;i<Num_SN;i++){ |  |  |
| SN | 10 | ARQ block sequence number |
| FLAG | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SUB_SN | TBD | ARQ sub-block sequence number |
| while(FLAG == 1) { |  |  |
| Selective ACK MAP | 7 | Each bit represents ACK or NAK of corresponding ARQ block. ‘0’ is NAK and ‘1’ is ACK. First MSB of first ACK MAP represents ACK or NAK information of SN. Contiguous bits after first MSB of first ACK MAP is corresponding to contiguous SN. |
| FLAG | 1 | 0 = No more selective ACK MAP 1 = Another set of selective ACK MAP and FLAG follows |
| } |  |  |
| } |  |  |
| Reserved | variable |  |
| } |  |  |

============================== End of Proposed Text ========================