| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on CSG ID (AWD-Femto) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Kanchei (Ken) Loa, Yung-Lan Tseng Chun-Yen Hsu, Youn-Tai Lee Institute for Information Industry (III) Shiann-Tsong Sheu, Chih-Cheng Yang National Central University (NCU) Whai-En Chen, Shih-Yuan Cheng National Ilan University (NIU) | Voice: +886-2-66000100 E-mail: loa@iii.org.tw |
| Re: | IEEE 802.16m-09/0028r1. “Call for Comments and Contributions on Project 802.16m Amendment Content” Target topic : Support for Femtocell BS. |  |
| Abstract | This contribution proposes the CSG ID as an identifier of CSG |  |
| Purpose | To be discussed and adopted in IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on CSG ID (AWD-Femto)

Kanchei (Ken) Loa, Yung-Lan Tseng, Chun-Yen Hsu, Youn-Tai Lee

Institute for Information Industry

Shiann-Tsong Sheu, Chih-Cheng Yang

National Central University

Whai-En Chen, Shih-Yuan Cheng

National Ilan University

# Introduction

CSG ID is used as an MAC level identifier for CSG Femtocell BS. In order to distinguish different CSGs in the system, each CSG should be assigned with a unique CSG ID (within the network of an operator) and AMS can find out the available CSG through the reception of CSG ID. CSG ID is shared by a group of Femtocell BSs in the same CSG. CSG Femtocell ABS should broadcast its CSG ID in the S-SFH SP1. The length of CSG ID is 24 bits, which is the same length with the LSB of BS ID.

# Reference

[1] IEEE 802.16m-08/003r9a,” IEEE 802.16m System Description Document [Draft]” May 31, 2009.

--------------------------------------------------------------------------------------------------------------------------------------

# Proposed text

[Modify Table 663 as indicated]

Table 663—S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSB of superframe number | [8] | Remaining bit of SFN except LSB of SFN in P-SFH |
| LSB of BS ID | 24 | Specifies the 24 least bit of BS ID |
| CSG ID | 24 | Specifies the CSG ID |
| Periodicity of A-MAP | 1 | 0b0: every subframe 0b1: every 2 subframes |
| DL permutation configuration (CRU, DRU partitioning and signaling related to that) | [22] | DL_CAS_SB0(4), DL_CAS_MB0(6), DL CAS_SBi (4x3) (Up to 22 bits, Need the decision from DL physical structure section) |
| UL permutation configuration (CRU, DRU partitioning and signaling related to that) | [22] | UL_CAS_SB0(4), UL_CAS_MB0(6), UL_CAS_SBi (4x3) (Up to 22 bits, Need the decision from UL physical structure section) |
| Initial ranging channel information (initial ranging region location) | TBD | (Need the decision from UL Ctrl section) |

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| RNG codes information | 12 | Parameters for determining the root sequences and their cyclic shifts in the preamble set for the cell (Up to 12 bits, Need the decision from UL Ctrl section) |
| HO ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| HO Ranging backoff start | 4 | Initial backoff window size for HO rang- ing contention, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| HO Ranging backoff end | 4 | Final backoff window size for HO rang- ing contention, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| BS EIRP | 16 | Signed in units of 1 dBm |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for any new initial entry |
| Reserved | TBD |  |
| } |  |  |