| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification for Feedback Allocation A-MAP Information Elements in the 802.16m AWD |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Sangheon Kim, David Mazzarese, Hokyu Choi, and Heewon Kang Samsung Electronics Huaning Niu, Hongmei Sun, Guangjie Li, Yuan Zhu, Qinghua Li, Ayelet Doron, Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi Intel Corporation Jinyoung Chun, Wookbong Lee, Bin-Chul Ihm LG Electronics Rajni Agarwal, Kevin Power Fujitsu | E-mail: sangheon.kim@samsung.com huaning.niu@intel.com jychun03@lge.com Rajni.agarwal@uk.fujitsu.com Kevin.power@uk.fujitsu.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” Category: AWD comment / Area: DL-CTRL (Section 15.3.6) |  |
| Abstract | The contribution proposes text for the DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification for Feedback Allocation A-MAP Information Elements in the 802.16m AWD

Sangheon Kim, David Mazzarese, Hokyu Choi, and Heewon Kang

Samsung Electronics Co., Ltd.

Huaning Niu, Hongmei Sun, Guangjie Li, Yuan Zhu, Qinghua Li, Ayelet Doron, Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi

Intel Corporation

Jinyoung Chun, Wookbong Lee, Bin-Chul Ihm

LG Electronics

Rajni Agarwal, Kevin Power

Fujitsu

- Introduction

This contribution provides amendment text modification for feedback allocation A-MAP IEs to be included in Section 15.3.6.5 in the 802.16m Amendment Working Document [1]. This contribution clarifies the name of the field and provides description for the fields.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

<Modify section 15.3.6.5.2.9 on page 224 line 61 as shown below>

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.6.5.2.9 Feedback Allocation A-MAP IE

Table 682 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating UL fast feedback control channels (including both PFBCH and SFBCH) to an AMS. If an AMS has an existing fast feedback control channel and receives a new feedback channel allocation, the original fast feedback channel is de-allocated automatically.

Definitions of the fields in the Feedback Allocation A-MAP IE are listed below in Table 682.

Note: 1. Need to differentiate it is in OL region or not. In OL region, MaxMt is not required

Table 682. Feedback Allocation A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { | - | - |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE = 0b0010 |
| Channel Index | Variable | - Feedback channel index within the UL fast feedback control resource region - (Dependent on LFB,FPi defined in section 15.3.8.3.3.2) |
| Allocation Short-term Feedback Period (p) | [3] | A feedback is transmitted on the FBCH every 2pframes |
| Long-term Allocation Feedback Period (q)(TBD) | [2] | TBD A long-term feedback is transmitted on the FBCH every 2q short-term feedback opportunity If q = 0b00, long-term feedback is not used. |
| Frame offset (TBD) | [3] | Depends on the decision of A-MAP allocation IE The AMS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the AMS should start reporting in eight frames |
| Allocation Duration(d) | [3] | A FBCH is transmitted on the FBCH channels indexed by Channel Index for 82d frames. If d = 0b000, the FBCH is deallocated. If d=0b111, the AMS should report until the ABS command for the AMS to stop. |
| ACK Allocation Flag | [1] | Indicate if one ACK channel is allocated |
| If (ACK Allocation Flag == 0b1){ |  |  |
| HFA | [3] | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } |  |  |
| MaxMt | Variable [1-2] | Variable number of bits - depends on number of transmit antenna Nt - If Nt=2 (SU-MIMO and MU-MIMO): - 0b0: 1 - 0b1: 2 - If Nt=4 (SU-MIMO and MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt=8 (SU-MIMO): - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - If Nt=8: (MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| MFM | [3] | - MIMO Feedback Mode as defined in Table 691 |
| If (MFM = 2, 3, 5, 6) { |  |  |
| Feedback Format | Variable [2-3] | Variable number of bits –depends on MIMO feedback mode |
| } |  |  |
| If(MFM=0,1,4,7){ |  | (TBD depending on IM group decision on FFR) |
| Number of Reports (n) | [1] | - 0: Same Feedback reported on each FFB opportunity - 1: Different Feedback reported in a time-interleaved fashion over the PFBC opportunities |
| If(Number of Reports>0) { |  | (TBD depending on IM group decision on FFR) |
| Long Allocation Period_multiple_reports (q) | [2] | Long Period Fast Feedback reports are transmitted on the FBCH channel every 2p+q+1frames |
| } |  |  |
| For (i==0 to n){ |  | - For specified Number of Reports (n) - (TBD depending on IM group decision on FFR) |
| FPS | [2] | - Frequency Partition and Reuse Factor as follows: - (Refer to section 20.1 for the frequency partition indexing definition.) - 0b00: Frequency Partition 0 (Reuse 1) - 0b01: Deboosted reuse 3 - 0b10: Boosted Reuse 3 - 0b11: Reserved. |
| } |  |  |
| } |  |  |
| If(MFM=0,1,4,7){ |  |  |
| FPI | [2] | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate of the frequency partition and reuse factor in the future: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| If (MFM = 0,1 & Long-term Feedback Period != 0b00) |  |  |
| { |  |  |
| Long term FPI | [2] | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate for the second frequency partition using long term feedback: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | Variable [0, 2] | Codebook Feedback Mode and Codebook Coordination Enable 0b00: standard with CCE disabled 0b01: adaptive with CCE disabled 0b10: differential with CCE disabled 0b11: standard with CCE enabled If MFM = 2,5 CM is empty (0 bit) |
| CS | [1] | Codebook subset |
| } |  |  |
| If(MFM==2,3,5,6) (TBD) { |  | Localized feedback mode |
| If (feedback_Format == 0) { |  | Specified CQI (used to report specified DLRU/subband CQI) |
| DLRU/subband index | Variable [3-4] | - Variable number of bits – depends on system bandwidth. - 0b000 - 0b110 (3bits): to index 6 subband when system bandwidth is 5MHz - 0b0000-0b1100(4bits): to index 12 subband when system bandwidth is 10MHz |
| } |  |  |
| } |  |  |
| If(MFM=0, 1, 5){ |  |  |
| Measurement Method Indication | [1] | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

Channel Index: Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between Channel Index and the AMS. In case of multiple SFBCH are allocated for an AMS, it is assumed that multiple SFBCH are allocated contiguously and this field specifies the index of the first one.

ACK Allocation Flag: BS may set ACK Allocation Flag to 0b1 if Allocation Duration equals 0b00. BS may set ACK Allocation Flag to 0b1 if Allocation Duration doesn’t equal 0b00 and the channel index of the newly allocated FBCH is the same as the channel index of the deallocated FBCH.

Short-term Feedback Period (p): A short-term feedback is transmitted on the FBCH every 2pframes

Long-term Feedback Period (q): A long-term feedback is transmitted on the FBCH every 2q short-term feedback opportunity. If q = 0b00, long-term feedback is not used.

Allocation duration (d): A FBCH is transmitted on the FBCH channels indexed by Channel Index for 82d frames. If d = 0b000, the FBCH is deallocated. If d=0b111, the AMS shall report until the ABS command for the AMS to stop.

MFM: MIMO feedback mode, defined in Table 691.

Feedback Format: This field specifies the feedback format index when reporting fast feedback information in FBCH. Feedback format definitions for different MIMO feedback modes are described in Error: Reference source not found15.3.9.3.1.4.

FPI: The frequency partition over which the short term period report shall be measured by the AMS.

Long term FPI: The frequency partition over which the long term period report shall be measured by the AMS.

Long-term Allocation Period: Long term period report punctures the normal report.

Number of Reports (n): Indicates whether the FFB channel allocated to the AMS carries 1 or 2 PFBCH Reports.

Allocation Period (p):

Long Allocation Period multiple reports (q) : Indicates the allocation period for Long-Period report and these puncture the Short-Period reports when MFM=0, 1, 4, 7.

FPS: Frequency Partition Selection & Reuse Factor Information. (Refer to section 20.1 for the frequency partition indexing definition)

MaxMt: This field specifies the maximum rank to be fed back by the AMS if MFM=0,1,2,3,4 (which indicates a SU MIMO feedback mode for SM), or it specifies the maximum number of users scheduled on each RU at the BS if MFM=5,6,7 (which indicates a MU MIMO feedback mode)

CCE: Codebook Coordination Enable. When CCE is enabled, MS finds PMI within whole broadcasted codebook type entry; when this fields is set to 0b1, it means MS finds rate-1 PMI within broadcasted codebook entries indicated by BC_SI and codebook subset indication (CS in DL A-MAP IE).

DLRU: This field specifies the index of a certain RU which needs AMS to estimate and report CQI/PMI feedback. When a certain RU is specified, AMS only report one CQI (may plus RI)/PMI for it.

CM: this field specifies codebook feedback mode, which is needed only for MFM 3 and 6.

Measurement Method Indication: This field indicates the use of midamble or pilots for CQI measurement.

----------------------------------------- End of the Proposed Amendment Text --------------------------------------------